3GPP TSG-RAN WG2 Meeting #130                                        R2-2503783

St Julian’s, Malta, May 19th - 23rd, 2025

Agenda Item:	8.3.5

Source:	Mediatek Inc.

Title:	Discussion on Performance Monitoring for AI Mobility

Document for:	Discussion, Decision

Introduction

The performance monitoring procedure is responsible for controlling and determining whether an AI model is suitable for the current inference task. This procedure is closely related to whether inference, monitoring, and decision take place on the UE side or the NW side. For NW-sided model, it is natural to consider all inference, monitoring, and decision are performed on NW side. For UE-sided model, in RAN2 #129bis meeting, RAN2 agreed to consider both NW-sided monitoring and UE-sided monitoring as given below.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

In this contribution, we discuss the performance monitoring procedure for both NW-sided model and UE-sided model, respectively. The discussion of monitoring configuration and inference configuration is provided to support the performance monitoring procedure for both cases.

Discussion

Performance Monitoring Procedure

NW-sided Model

For NW-sided model, RAN2 has the following agreement in AI/ML for NR air interface as follows.

RAN#125bis

Agreements

1    RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2    RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3    RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

According to the agreement, the UE will only be involved in providing the required data if configured by the network. Both the monitoring and decision operations can be fully implemented by the network and do not need to be specified in the standard. The network first configures the measurement configuration to the UE. Based on this configuration, the UE performs measurements and sends the measurement report to the network. The network can then compare the inference output with the monitoring data according to a certain monitoring metric.

Proposal 1: For NW-sided model, the performance monitoring and management decisions are made on the NW side and can be fully NW-implemented. UE is only involved in providing the measurement for monitoring based on the configuration provided by NW.

UE-sided Model

For the UE-side model, RAN2 agrees to consider both NW-sided performance monitoring and UE-sided performance monitoring, as mentioned in the introduction section. We will discuss the corresponding procedures and configurations for both cases in the following sections.

For NW-sided monitoring, NW configures both monitoring configuration and inference configuration to UE. UE then reports the monitoring data (i.e., ground-truth) and the inference output to NW based on the monitoring configuration and inference configuration, respectively. The reporting of monitoring data and inference output can be independent, e.g., the timing (and/or report interval) can be configured with different values. Determining which monitoring data and inference output to compare is NW-implemented. In this case, NW performs monitoring and makes management decisions.

Proposal 2: For UE-sided model, NW-sided monitoring, the performance monitor procedure contains

- NW sends monitoring configuration and inference configuration to UE.

- UE reports the monitoring data and inference output based on the corresponding configurations to NW.

- NW performs monitoring and makes decisions, which can be fully NW-implemented.

For UE-sided monitoring, NW configures monitoring configuration to UE. UE measures the monitoring data (i.e., ground-truth) based on the monitoring configuration. UE then compares its inference output with monitoring data (which inference output should be compared is FFS). The information about monitor criteria, e.g., L3 RSRP difference, should be configured in the monitoring configuration. Once monitoring results are generated by UE, there are two options.

- (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision.

- (UE-decision): UE makes the management decision and sends the decision to NW.

The NW-decision approach allows the network to fully control the AI performance. However, it has a higher latency for UE to react to the model management decision. For example, if the monitoring result generated by UE shows that the AI model is not suitable for current tasks, in the NW-decision approach, UE still needs to send the report and wait for corresponding NW signaling. In the UE-decision approach, UE can quickly fallback to the non-AI approaches to maintain the connection quality.

Proposal 3: For UE-sided model, UE-sided monitoring, the performance monitor procedure contains

- NW sends monitoring configuration to UE

- UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data.

- (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision.

(UE-decision): UE makes the management decision and sends the decision to NW.

Although both NW-sided monitoring and UE-sided monitoring are supported for AI mobility, in our view, the UE-sided monitoring is more reasonable for UE-sided model. The key reason is that for UE-sided model, all the necessary information to perform model monitoring is measured or generated on the UE side, i.e., the monitoring data and inference output. If we want to consider NW-sided performance monitoring, UE must report those information to the NW side. A simple illustration of the additional overhead is given as follows.

Figure 1: The report overhead for (a) NW-sided monitoring (b) UE-sided monitoring

As shown in the figure, for NW-sided monitoring, UE needs to report monitoring data and inference report for each monitoring data period (assuming the inference report period is configured the same as the monitoring report). However, for UE-sided monitoring, UE only needs to report the monitoring result when a predefined condition is met, e.g., when the RSRP difference between monitoring data and inference output is higher than a given threshold. Clearly, UE-sided monitoring can significantly reduce the overhead associated with monitoring and inference reports.

Observation 1: For UE-sided model, it is more efficient to perform monitoring on the UE side, as it does not require sending the inference output and monitoring data to the NW side.

Inference and Monitoring Configuration

To support the performance monitoring for AI mobility, NW must configure both the inference configuration and monitoring configuration to UE. RAN2 has agreed to reuse the legacy RRM measurement configuration framework as the baseline for AI mobility inference configuration. In our other contribution [1], we propose an overall structure of inference configuration as shown in the following figure.

Figure 2: Overall structure of inference configuration.

The full inference configuration should contain the following components:

- Measurement object: Describes the information of measurement/observed target.

- Report configuration: Describes the triggering condition and report quantity for the measurement report. The measurement report can be generated based on both real measurement and inference results.

- Predicted object: Describes the information of inference/predicted target.

- Inference identity: Provides the mapping of a measurement object, a predicted object, and a report configuration.

When UE receives the inference configuration, it can realize the inference task, e.g., AI mobility sub-case, via the measurement object, predicted object, and report configuration without any additional description. UE can identify the observation target via the measurement object and the inference target via the predicted object. The report configuration provides the information for UE to report the results (of the AI inference task, e.g., A3 event).

In addition to the inference configuration, to perform the performance monitoring, NW also needs to provide the monitoring configuration. By reusing the legacy RRM measurement framework as the baseline, we propose the overall structure of the monitoring configuration as shown in the following figure.

Figure 3: Overall structure of monitoring configuration.

The legacy measurement object can be reused to carry the information about the measurement target (to measure ground-truth data). Several new items are added to provide information about monitoring criteria, monitoring reports, and the corresponding inference task. In our view, a monitoring configuration should contain the following components:

- Measurement object: Describes the information about the measurement target (to measure ground-truth/monitoring data).

- Monitoring report configuration: Describes the report triggering condition, report quantity, and monitoring criteria.

- Monitoring identity: Provides the mapping of a measurement object, a monitoring report configuration, and an inference identity.

With monitoring configuration, UE can measure the ground-truth/monitoring data according to the measurement object. For NW-sided monitoring, UE directly reports the raw monitoring data to the NW. The necessary information for reporting the monitoring data is configured in the monitoring reporting configuration, including the report type (e.g., event-triggered, periodical), and the report quantity (e.g., L1/L3 RSRP, SINR, RSSI). For UE-sided monitoring, UE reports the monitoring results (the comparison result) instead of raw monitoring data (e.g., ground-truth). The monitoring criteria (e.g., average L3 RSRP difference, F1 score, predefined threshold) and the report triggering condition are all provided in the monitoring report configuration. It should be noted that, for the UE-sided monitoring, RAN4 needs to be informed and involved to define the requirement to determine whether the AI prediction is accurate or not.

Proposal 4: The monitoring configuration contains

- Measurement object: Describes the information about the measurement target

- Monitor report configuration: Describes the report triggering condition, report quantity, and monitoring criteria

- Monitoring identity: Provides the mapping of a measurement object, a monitoring report configuration, and an inference identity

Proposal 5: For UE-sided monitoring, UE compares the monitoring data and inference output based on the monitoring criteria in the monitoring configuration. RAN4 needs to be informed and involved to define the corresponding requirement to determine whether the AI prediction is accurate or not.

To perform performance monitoring, the device must know which monitoring result and inference output should be compared. The information about the corresponding inference identity is therefore provided in the monitoring identity. It should be noted that the timing (and periodicity) of generating inference outputs may not align with the timing (and periodicity) of measuring monitoring data as shown in the following figures.

For NW-sided monitoring, which monitoring data and inference output are used for comparison is up to NW implementation. For UE-sided monitoring, the monitoring result should be generated by comparing the closest inference output with the monitoring data.

Figure 4: For UE-sided monitoring, monitoring result is generated by comparing the closest inference output and monitoring data.

Proposal 6: If the timing and/or period of inference output and monitoring data are not the same,

- For NW-sided monitoring, which inference output and monitoring data are used for comparison is up to NW implementation.

- For UE-sided monitoring, the monitor result is generated by comparing the closest inference output with the monitoring data.

Conclusion

Performance Monitoring Procedure:

NW-sided Model

Proposal 1: For NW-sided model, the performance monitoring and management decisions are made on the NW side and can be fully NW-implemented. UE is only involved in providing the measurement for monitoring based on the configuration provided by NW.

UE-sided Model

Observation 1: For UE-sided model, it is more efficient to perform monitoring on the UE side, as it does not require sending the inference output and monitoring data to the NW side.

Proposal 2: For UE-sided model, NW-sided monitoring, the performance monitor procedure contains

- NW sends monitoring configuration and inference configuration to UE.

- UE reports the monitoring data and inference output based on the corresponding configurations to NW.

- NW performs monitoring and makes decisions, which can be fully NW-implemented.

Proposal 3: For UE-sided model, UE-sided monitoring, the performance monitor procedure contains

- NW sends monitoring configuration to UE

- UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data.

- (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision.

(UE-decision): UE makes the management decision and sends the decision to NW.

Inference and Monitoring Configuration:

Proposal 4: The monitoring configuration contains

- Measurement object: Describes the information about the measurement target

- Monitor report configuration: Describes the report triggering condition, report quantity, and monitoring criteria

- Monitoring identity: Provides the mapping of a measurement object, a monitoring report configuration, and an inference identity

Proposal 5: For UE-sided monitoring, UE compares the monitoring data and inference output based on the monitoring criteria in the monitoring configuration. RAN4 needs to be informed and involved to define the corresponding requirement to determine whether the AI prediction is accurate or not.

Proposal 6: If the timing and/or period of inference output and monitoring data are not the same,

- For NW-sided monitoring, which inference output and monitoring data are used for comparison is up to NW implementation.

- For UE-sided monitoring, the monitor result is generated by comparing the closest inference output with the monitoring data.

Reference

[1]   R2-2503782 Discussion on Functionality Management for AI mobility