3GPP TSG-RAN WG2 Meeting #129                                                                             R2-2503789

Malta, May 19th – 23rd , 2025

Agenda item:	8.2.2

Source:	China Telecom

Title:	Discussion on Paging Procedure for Ambient IoT

Document for:	Discussion

- Introduction

In the RAN#106 meeting, a new work item [1] was agreed for Ambient IoT (Internet of Things) in NR. The scope includes specifying the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission. RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. A-IoT paging includes subsequent paging for the same service.

In the last RAN2#129bis meeting, the following agreements were made.

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 A-IoT R2D messages extensible to accommodate devices and features of future release.

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to tak this into account for their design.

However, a few open issues remain on the topic. Firstly, how to handle parallel service requests requires further discussion. Some open issues on paging identity still require handling. For instance, transaction ID discussions are still ongoing.  Finally, efficient design of the paging message to handle subsequent paging case should be considered.

Based on the above progress, this paper mainly analyses the open issues for A-IoT paging procedure.

- Discussion

2.1 Behavior for Parallel Service Requests

Recent discussions on A-IoT paging have focused on whether parallel service requests should be supported in A-IoT scenarios, particularly when initiated by different readers. Supporting parallel requests from the same reader was ruled out to maintain device design simplicity. However, in cases involving multiple readers, it is possible that more than one reader may send paging messages—either for the same or different CN service requests—simultaneously. This is benefit to enhance coverage or assist with device localization.

A key question is whether such operations should be executed in parallel or in a time-divided manner. The general consensus is that, with proper coordination—especially when readers are managed by the same network node—overlaps can be avoided by assigning different start times. Even in more distributed configurations, such as when readers are managed by different A-IoT NG-RAN node, the coordination is still feasible.

Another scenario involves a reader stopping and restarting an operation using different resources—for example, in response to collisions or when moving between cells. In these cases, the device must be able to recognize the updated paging message and treat it as a new operation, even if a previous one is still in progress.

Two points were highlighted:

- Overlapping operations from different readers can be managed through coordination.

- Devices should be capable of handling service restarts triggered by updated paging messages

Regarding device behaviour upon receiving new service requests during an ongoing procedure:

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

As discussed in the previous meeting, two options are currently under consideration:

- Option A: Ignore all new service requests and R2D messages addressed to the device unless they are associated with the ongoing procedure.

- Option B: Terminate the ongoing procedure and respond to the most recent service request.

To address the two key considerations outlined earlier, we believe that Option B provides a more straightforward and efficient solution. This approach assumes that the CN will prevent simultaneous service requests from multiple readers, thereby minimizing the risk of conflicts or unintended interruptions.

- Proposal 1: To define the device behavior for getting a new service request, as terminating the ongoing procedure and responding to the latest request.

2.2 Paging Round

The A-IoT-devices may have the following operation state and may switch the state depending on its energy harvesting status.

- ON state: transmission, reception, and a clock running

- OFF state: energy harvesting only

- SLEEP sate: energy harvesting, monitoring, maintaining memory, and a rough clock running

whether device 1 can support sleep state in addition to the ON and OFF state is discussed in RAN1. It is clear that the device 2b can support ON; OFF and SLEEP state.

In RAN1 meeting #118, RAN1 had reached the following agreements:

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

In Direction 2, the reader includes information in the R2D message that enables the device to determine whether it should become available or unavailable for energy harvesting. For instance, if the A-IoT paging message indicates that the device is not intended to participate in any access round during the current paging round, the device can make itself unavailable for communication—e.g., by transitioning to the OFF or SLEEP state—to harvest energy until the paging round concludes. Additionally, the reader may include further information in the A-IoT paging message to help unintended devices estimate the duration of the current paging round. This could include, for example, the total number of access rounds scheduled or an estimated time for the completion of the paging round.

- Proposal 2:  	If the A-IoT paging message indicates that a device is not an intended device to participate in any access round in the current paging round, the device can make itself unavailable for communications with the reader to harvest energy until the end of the current paging round.

- Proposal 3:  	The reader may further include information in the A-IoT paging message to indicate, or for unintended devices to estimate, how long the current paging round may be, e.g., the number of total access rounds to be scheduled in the current paging round or an estimated duration for the current paging round to be completed.

If the A-IoT paging message indicates that a device is intended to participate in access round #N of the current paging round, the device will make itself available for communication with the reader (e.g., by entering the ON state) during access round #N. During other access rounds for which the device is not eligible to contend, it will remain unavailable for communication (e.g., by transitioning to the OFF or SLEEP state) to harvest energy, until the end of the current paging round. To support this behaviour, the reader may include additional information in the A-IoT paging message to indicate—or enable the intended device to estimate—the duration of each access round within the current paging round. This allows the device to determine its availability and unavailability periods in a way that maximizes energy harvesting. If the device successfully contends for an access occasion during access round #N, it will include an energy status indication in its Msg3 and/or D2R message(s) sent to the reader in response to commands. This indication reflects the device’s predicted energy status at the end of the current paging round, taking into account the energy expected to be harvested during the remaining unavailability period (i.e., from the end of access round #N to the end of the paging round). Details on how the energy status indication is generated and used—as well as how it interacts with the reader’s control of the device’s availability and unavailability periods—are provided in one of our companion papers submitted to this meeting.

2.3 Multi-Devices CFRA Paging Scenario

When multiple A-IoT devices are activated for the CFRA procedure using group or mask IDs, each target device can determine its access timing index based on its position relative to the group or mask. However, if the target devices belong to different groups, their individual device IDs must be explicitly included in the A-IoT paging message. A third option involves using a PF/PO-like (Paging Frame/Paging Occasion) method to assign access timing based on the device ID.  This method limits collisions to devices with the same modulo result (i.e., devices sharing the same access index). Each device then determines its rank among others sharing the same modulo result and adjusts its timing accordingly to derive its unique access occasion. When using this PF/PO-like method, the A-IoT paging message only needs to include:

- The list of device IDs that share the same modulo result, and

- The number of access occasions in one cycle (the divisor N).

To trigger the CFRA procedure for multiple devices, the A-IoT paging message must provide a mapping between each device ID and its corresponding access occasion index. However, explicitly signalling a one-to-one mapping creates significant overhead. Alternatively, when using group or mask IDs, the access occasion index for each device can be inferred from its relative position in the group/mask. But if the devices are not part of a single group, individual device IDs must be included, again requiring one-to-one mapping, which increases message size. In contrast, the PF/PO-like solution provides a scalable and efficient way to allocate access occasions. By using a consistent modulo-based computation and minimal signalling (i.e., only the modulo result group and total number of access occasions), devices can independently derive their assigned timing, thus reducing paging overhead and managing contention more effectively.

- Proposal 4: To trigger A-IOT devices to find their correct access occasions in A-IOT CFRA procedure, three options can be discussed to be included in the A-IOT paging message:

- One-to-one mapping of the device ID to the access occasion

- PF/PO-like access index allocation approach

- Mask ID

2.4 Multiple Device IDs in Paging Message

RAN2 has made the agreement that the following cases for A-IoT paging message will be considered:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message.

- a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion.

RAN2 did not reach consensus on the necessity of supporting multiple device IDs and suggested that the need for this use case be confirmed through SA2 discussions. However, no relevant discussions or agreements were reached in the recent SA2 meetings. In practice, the use of multiple device IDs arises from both the CN/AF and A-IoT air interface perspectives. From the CN/AF viewpoint, it may be necessary to inventory or read/write a small set of specific devices that are not part of any preconfigured group. On the A-IoT air interface side, after several paging rounds, the reader may need to identify a few remaining unresponsive devices with non-contiguous device IDs. For example, although each paging message targets the same batch of devices, not all devices may have the opportunity to respond in a given round. When multiple device IDs with discontinuous values are included in a service request and subsequently need to be signaled over the A-IoT air interface, repeatedly sending the same list of device IDs in every paging message is inefficient. Especially if some devices have already responded successfully in previous rounds, including them again in future messages is redundant and wasteful. A more logical approach would be to remove devices that have already responded from subsequent paging messages. While the size of the transport block (TB) may limit the number of device IDs that can be included in a single message, the above use case highlights a valid need. Detailed solutions can be further discussed during the Work Item (WI) stage.

- Proposal 5: To support paging message containing multiple IDs of A-IoT devices. The multiple IDs can be used to indicate multiple devices with discontinuous device IDs.

- Conclusions

Based on the discussion in the previous sections, we propose the following:

- Proposal 1: To define the device behavior for getting a new service request, as terminating the ongoing procedure and responding to the latest request.

- Proposal 2:  	If the A-IoT paging message indicates that a device is not an intended device to participate in any access round in the current paging round, the device can make itself unavailable for communications with the reader to harvest energy until the end of the current paging round.

- Proposal 3:  	The reader may further include information in the A-IoT paging message to indicate, or for unintended devices to estimate, how long the current paging round may be, e.g., the number of total access rounds to be scheduled in the current paging round or an estimated duration for the current paging round to be completed.

- Proposal 4: To trigger A-IOT devices to find their correct access occasions in A-IOT CFRA procedure, three options can be discussed to be included in the A-IOT paging message:

- One-to-one mapping of the device ID to the access occasion

- PF/PO-like access index allocation approach

- Mask ID

- Proposal 5: To support paging message containing multiple IDs of A-IoT devices. The multiple IDs can be used to indicate multiple devices with discontinuous device IDs.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- R2-25xxxxx, Draft Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things