3GPP TSG-RAN WG2 Meeting #130	R2-2503795

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.7.5

Source:	China Telecom

Title:	Discussion on Remaining Issues of RLC AM Enhancements

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129bis meeting, the following agreements were made for RLC AM enhancements.

When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

The paper aims to discuss the remaining issues on autonomous retransmission and polling enhancements.

2 Discussion

It has agreed in the last meeting that the discardTimer at PDCP is reused to determine the remaining time that triggers autonomous retransmission and/or polling. In this case, the PDCP layer can provide the necessary indication to the RLC layer at the precise timing based on the remaining time and configured threshold. Given that the PDCP layer already supports the discard indication, delay-critical indication and the newly introduced delay-reporting indication, there is no need to introduce additional indications​​ to support ​​autonomous retransmission/polling enhancement​s. In other words, if the discardTimer at PDCP is reused to determine the remaining time that triggers autonomous retransmission and/or polling, there is no concern regarding the too early or unnecessary retransmission/polling problem.

Proposal 1: If the discardTimer at PDCP is reused to determine the remaining time, there is no concern regarding the too early and/or unnecessary retransmission.

In Rel-18 XR study, a PDU set containing one or more PDUs is introduced to carry the payload of one unit of information generated at the application level. Per TS38.322, the UE polls the gNB after a configured number of PDUs or bytes have been transmitted. It is likely that the UE may trigger a poll in the middle of a PDU set. As a result, the status report may only contain the reception status of a part of PDUs in a PDU set. If the remaining part of the PDUs has not been retransmitted promptly, the entire PDU set including previously successfully transmitted PDUs may be discarded, resulting in resource wastage.

Therefore, for XR traffic, the polling mechanism may need to be enhanced to allow the UE to trigger polling based on the PDU set. For instance, the polling bit could be set based on the PDU set traffic pattern. In this way, the UE can receive the status report of one or more PDU sets.

Proposal 2: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

- Conclusions

Based on the above analysis, we have the following proposals:

Proposal 1: If the discardTimer at PDCP is reused to determine the remaining time, there is no concern regarding the too early and/or unnecessary retransmission.

Proposal 2: Polling enhancement is needed to enable the UE to poll the RLC status based on the PDU set.

- References

- R2-25xxxxx, Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece