3GPP TSG-RAN WG2 Meeting#130                               	R2-2503797

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.3

Source: 	Google

Title: 	Open issues in the running CRs for OD-SIB1

Document for:	Discussion and Decision

# Introduction

The second phase (i.e., Release-19) of the NES WI was kicked off in February 2024, and the revised Work Item Description (WID) RP-242354 [1] was updated after the overall progress was checked at the checkpoint in the RAN#105 meeting. The WID indicates that 3GPP will specify the procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, with the focus on the scenario where the UE obtains UL WUS configuration from Cell A, transmits UL WUS on NES Cell, and receives on-demand SIB1 from NES Cell (i.e., Case 2).

The running CR for TS 38.304 [2] has listed two open issues that the rapporteur has identified while implementing the accumulated RAN2 agreements on OD-SIB1. This paper discusses the 2nd issue (i.e., Issue 2 shown below) and also an FFS item pending from the RAN2#129bis meeting [4], on the provision of the cell access related information in the OD-SIB1 configuration.

- Issue 2: FFS whether to explicitly capture the failure case of OD-SIB1 window expiry in 38.304. [Issue description] Some companies proposed to explicitly capture the failure case of OD-SIB1 window expiry in 38.304 according to RAN2#129b agreement. And they proposed to add the failure condition in 38.331. - If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred. [Rapporteur view] After coordination with RRC CR rapporteur and MAC CR rapporteur, current conclusion is to make it FFS for further discussion due to below concern: - It seems to be obvious that the failure is after the window. - If we specify the window, then we may need to capture separately all the other cases why UE doesn’t receive SIB1.

# Discussion

## (38.304 Issue 2) SIB1 acquisition failure upon the expiry of SIB1 monitoring window

RAN2 has explicitly agreed three cases where the UE can bar a SIB1-less cell, and these cases are:

- UE had no corresponding UL WUS configuration,

- MAC indicates max number of preamble transmission for the OD-SIB1 request, and

- UE fails to acquire SIB1 upon the expiry of the SIB1 monitoring window.

Agreement in RAN2#128 - The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request. - A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell. Agreement in RAN2#129bis - If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

Currently, only the first two cases were implemented in the running CR for TS 38.304 [2], while the implementation of the last case was still pending for decision. One concern on implementing the last case is the worry about the necessity of also implementing all other cases where the UE does not receive SIB1. However, as RAN2 only agreed these three cases for barring a SIB1-less cell, there is no need to consider/specify other cases. Besides, if the UE has implemented the last case, the UE is either able to acquire SIB1 before the window expires, or unable to acquire SIB1 upon the expiry of the window (and hence has to bar the cell). There is no other cases that could possibly occur.

RAN2 agreed that UE can bar a SIB1-less cell ONLY when: 1) UE had no corresponding UL WUS configuration, 2) MAC indicates max number of preamble transmission for the OD-SIB1 request, and 3) UE fails to acquire SIB1 upon the expiry of the SIB1 monitoring window. Hence, no other cases need to be considered and specified in TS 38.304 or TS 38.331.

On the other hand, if we do not specify the UE behavior for case 3) above, the consequence is that the UE may trigger another round of the OD-SIB1 requesting procedure after failing to acquire OD-SIB1 in the SIB1 monitoring window. Such reattempt may be repeated endlessly by the UE, even if the OD-SIB1 acquisition is doomed to failure due to factors like a poor downlink channel or the network not transmitting OD-SIB1.

If we don’t specify the UE behavior for the case 3) above, UEs may keep requesting the OD-SIB1 transmission from a SIB1-less cell even if the OD-SIB1 acquisition is doomed to failure.

To conserve radio resources and UE power that would otherwise be wasted on repeated unsuccessful attempts, defining UE behavior for case 3) is important. Moreover, to simplify the UE implementation, it is desired to have a unified UE handling for all the cases when the UE is unable to acquire OD-SIB1.

(38.304 Issue 2) RAN2 to specify the UE behavior on the expiry of the SIB1 monitoring window in TS 38.304 (i.e., to remove the brackets in the sub-clause 5.3.1 of the 38.304 running CR [2]).

If Proposal 1 can be agreed, to align the UE behavior in both TS 38.304 and TS 38.331, a change to the procedure text in the current running CR for TS 38.331 [3] is also needed. The TP appended in the Annex A at the end of this paper can be used to update the current 38.331 running CR [3], in case Proposal 1 is agreed.

(38.304 Issue 2) If P1 is agreed, RAN2 to adopt the TP in Annex A into the 38.331 running CR [3].

## The provision of the cell access related information

Since the idle/inactive UE may need to check the PLMN ID and certain feature-specific barring indication (e.g., cellbarredNTN-r17, cellBarredRedCap1Rx-r17, etc.) in SIB1 to determine whether the UE can camp on the cell or not, the entire cell reselection process may take much longer time if the cell the UE attempts to reselect to is an OD-SIB1 cell. The worst case is that the UE may find an OD-SIB1 cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

The UE may find an OD-SIB1 cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

In the RAN2#129bis meeting [4], it was discussed whether to include the cell access related information in the UL WUS configuration, to avoid unnecessary SIB1 requests on a cell where UE does not have access. However, as there was no clear majority supporting the proposal, the decision was pending until the next meeting after the online discussion.

R2-2502916 On demand SIB1 handling Nokia, Nokia Shanghai Bell discussion Rel-19 Netw_Energy_NR_enh-Core Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access. [Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information. Offline among interested companies. Comeback in CB session (Nokia). [Comeback session]: [Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia. Revisit it next meeting.

Some companies think that the cell access related information might be very big, as it may need to include the access information for specific features such as the slicing, NPN, NTN, etc. However, we think each feature-specific access information should be an optional field and hence the network can choose which feature-specific access information to be included, based on the feature(s) supported by the network. As it is unlikely a network will support multiple features simultaneously, the actual cell access related information included in the UL-WUS configuration won’t be too large in a live network.

Therefore, we propose to include cell access related information (i.e., the CellAccessRelatedInfo IE) and feature-specific barring indications per cell in the UL-WUS configuration, as optional fields. A TP reflecting the necessary changes on top of the current 38.331 running CR [3] is appended in the Annex B, and can be considered as the baseline if P3 can be agreed.

UL-WUS configuration can include the cell access related information (i.e., CellAccessRelatedInfo IE and feature-specific barring indications) per neighboring OD-SIB1 cell.

If P3 is agreed, RAN2 to adopt the TP in Annex B into the 38.331 running CR [3].

# Conclusion

In this paper, we discuss several open/FFS issues for supporting the on-demand SIB1 mechanism based on the current running CRs for TS 38.304 and TS 38.331 [2][3]. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

(38.304 Issue 2) SIB1 acquisition failure upon the expiry of OD-SIB1 monitoring window:

Observation 1	RAN2 agreed that UE can bar a SIB1-less cell ONLY when: 1) UE had no corresponding UL WUS configuration, 2) MAC indicates max number of preamble transmission for the OD-SIB1 request, and 3) UE fails to acquire SIB1 upon the expiry of the SIB1 monitoring window. Hence, no other cases need to be considered and specified in TS 38.304 or TS 38.331.

Observation 2	If we don’t specify the UE behavior for the case 3) above, UEs may keep requesting the OD-SIB1 transmission from a SIB1-less cell even if the OD-SIB1 acquisition is doomed to failure.

Proposal 1	(38.304 Issue 2) RAN2 to specify the UE behavior on the expiry of the SIB1 monitoring window in TS 38.304 (i.e., to remove the brackets in the sub-clause 5.3.1 of the 38.304 running CR [2]).

Proposal 2	(38.304 Issue 2) If P1 is agreed, RAN2 to adopt the TP in Annex A into the 38.331 running CR [3].

The provision of the cell access related information:

Observation 3	The UE may find an OD-SIB1 cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Proposal 3	UL-WUS configuration can include the cell access related information (i.e., CellAccessRelatedInfo IE and feature-specific barring indications) per neighboring OD-SIB1 cell.

Proposal 4	If P3 is agreed, RAN2 to adopt the TP in Annex B into the 38.331 running CR [3].

# Reference

RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

R2-2503708, Running 38.304 CR for network energy saving.

R2-2503607, Running RRC CR for enhancements for network energy efficiency.

Draft Report of 3GPP TSG RAN WG2 meeting #129bis, ETSI MCC.

# Annex A: TP for the UE handling on the expiry of SIB1 monitoring window

------------------------------------------- TP on the running CR for TS 38.331[3] ------------------------------------------------------

5.2.2.3.3x Request for on demand SIB1

The UE shall, while SDT procedure is not ongoing:

1>	trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig included in stored valid version of od-SIB1-Config for this  cell;

2>	if indication that maximum number of PRACH attempts is reached is received from lower layers as defined in TS 38.321 [3]:

3>	perform the actions as specified in clause 5.2.2.5.

2>	if acknowledgement for on demand SIB1 request is received from lower layers:

3>	acquire the requested SIB1 message as defined in FFS, immediately;

4> if the UE is unable to acquire the SIB1:

5> perform the actions as specified in clause 5.2.2.5.

3>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2;

1>	if cell reselection occurs while waiting for the acknowledgment for SIB1 request from lower layers:

2>	reset MAC;

Editor’s note:

FFS: if there is need to emphasize it is normal uplink

FFS reference for where are the details on how UE is obtaining SIB1, possibly RAN1 specification

FFS Co-existence of SDT and OD-SIB1

# Annex B: TP for the inclusion of the cell access related information in the UL-WUS configuration

------------------------------------------- TP on the running CR for TS 38.331[3] ------------------------------------------------------

- 	SIBXX

SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

cellAccessRelatedInfo               CellAccessRelatedInfo 								 OPTIONAL,        -- Need R

redCap-ConfigCommon-r17             RedCap-ConfigCommonSIB-r17							 OPTIONAL,        -- Need R

cellBarredNTN-r17            		  ENUMERATED {barred, notBarred}						 OPTIONAL,        -- Need R

ncr-Support-r18                     ENUMERATED {true}										 OPTIONAL,        -- Need R

cellBarredATG-r18                   ENUMERATED {barred, notBarred}						 OPTIONAL,        -- Need R

cellBarredNES-r18                   ENUMERATED {notBarred}								 OPTIONAL,        -- Need R

mobileIAB-Cell-r18                  ENUMERATED {true}										 OPTIONAL,        -- Need R

eRedCap-ConfigCommon-r18            ERedCap-ConfigCommonSIB-r18							 OPTIONAL,        -- Need R

cellBarredFixedVSAT-r18             ENUMERATED {barred, notBarred}						 OPTIONAL,        -- Need R

cellBarredMobileVSAT-r18            ENUMERATED {barred, notBarred}						 OPTIONAL,        -- Need R

cellBarred2RxXR-r18                 ENUMERATED {barred}									 OPTIONAL,        -- Need R

barringExemptEmergencyCall-r18   	  ENUMERATED {true}                                      OPTIONAL         -- Need R

...

}

OD-SIB1-Config-r19  ::= {

sib1-rsrp-ThresholdSSB-r19                       RSRP-Range                                                              OPTIONAL,   -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                                   OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}  OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL, -- Need R

offsetToCarrier-r19                          INTEGER (0..2199)                                                          OPTIONAL, -- Need R

absoluteFrequencyPointA-r19                  ARFCN-ValueNR                                                              OPTIONAL, -- Need R

p-Max-r19                                    P-Max                                                                      OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                       INTEGER (-60..50)                                                          OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19                                                     OPTIONAL  -- Need R

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                    SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SEQUENCE (SIZE (1..maxSIB1-Message)) OF SIB1-RequestResources-r19,

preambleTransMax-r19                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-ra-PreambleStartIndex-r19               INTEGER (0..63),

sib1-ra-AssociationPeriodIndex-r19           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

sib1-ra-ssb-OccasionMaskIndex-r19            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

SIBxx field descriptions

od-SIB1-CellConfigList Provides a configuration to request SIB1 for serving cell and neighbor cells.

OD-SIB1-Config field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2).

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2) used for OD-SIB1 request transmission.

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1[57] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) to be used for OD-SIB1 request transmission.

sib1-rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt to transmit OD-SIB1 request, see TS 38.321 [3].

sib1-tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

sib1-restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1.

cellBarredNTN Value barred means that the cell is barred for connectivity to NTN, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to NTN. If not present, the UE considers the cell is not allowed for connectivity to NTN, as defined in TS 38.304 [20]. This field is only applicable to NTN-capable UEs.

ncr-Support This field combines both the support of NCR and the cell status for NCR. If the field is present, the cell supports NCR and the cell is also considered as a candidate for cell (re)selection for NCR-node; if the field is absent, the cell does not support NCR and/or the cell is barred for NCR-node.

cellBarredATG Value barred means that the cell is barred for connectivity to ATG, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to ATG. If not present, the UE considers the cell is not allowed for connectivity to ATG, as defined in TS 38.304 [20]. This field is only applicable to ATG-capable UEs.

cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2.

mobileIAB-Cell The presence of this field indicates that this is a mobile IAB cell.

cellBarredFixedVSAT Value barred means that the cell is barred for fixed VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for fixed VSAT UEs. This field is ignored by non-VSAT UEs.

cellBarredMobileVSAT Value barred means that the cell is barred for mobile VSAT UEs, as defined in TS 38.304 [20]. If not present, the cell is not allowed for mobile VSAT UEs. This field is ignored by non-VSAT UEs.

cellBarred2RxXR Value barred means that the cell is barred for a 2Rx XR UE indicating supportOf2RxXR for the selected frequency band as specified in clause 5.2.2.4.2. This field is ignored by all UEs not indicating supportOf2RxXR for the selected frequency band. This field may be configured only if the cell operates in a frequency band where 4Rx antenna ports are mandated as specified in TS 38.101-1 [15]. If this field is absent on a cell operating in a frequency band where 4RX antenna ports are mandated, a 2RX XR UE shall treat the cell as not barred, as specified in TS 38.304 [20].

barringExemptEmergencyCall Indicates whether the cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [20].

SIB1-RequestConfig field descriptions

preambleTransMax Max number of RA preamble transmission performed for OD-SIB1 request (see TS 38.321 [3]).

ssb-perRACH-Occasion Number of SSBs per RACH occasion for OD-SIB1

sib1-RequestPeriod Periodicity of the SIB1-Request configuration in number of association periods.

sib1-RequestResources Resources for SIB1 request

SIB1-RequestResources field descriptions

sib1-ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SIB1 request for SIB corresponding to this si-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

sib1-ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SIB1 request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SIB1 request.

sib1-ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3].

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources configured in the corresponding OD-SIB1-Config, excluding preambles used for other purposes (e.g. for SI request, or SIB1 request).

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

Editor’s note: Only parameters in R1-2501645 that are in own rows are implemented and not all listed e.g. in cell 17P or 21P.

FFS to group some parameters under subIEs like frequencyInfoUL

FFS to separate IE OD-SIB1 as own IE, for review purposes it is here now.

FFS: value for maxCells, maxSIB1-Message, maxPCI

FFS: optionality of the parameters as there was no input on this

FFS: if list of cells is ARFCN&PCI or only PCI