3GPP TSG-RAN WG2 Meeting#130                               	R2-2503798

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.9.2

Source: 	Google

Title: 	Open issues on the S&F mode transition time

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the revised WID of Rel-19 IoT-NTN [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Significant progress was made in RAN#128, RAN2#129, and RAN2#129bis, on the S&F indication, the S&F mode transition time, and the list of satellite IDs provided by MME [2][3][4]. There are still some leftover issues on the S&F mode transition time, such as how to signal the transition time from the normal mode to the S&F mode, and the usage of the transition time in the AS layer. In this paper, we discuss these leftover issues and provide TPs at the end of the paper with the intention to resolve these issues.

# Discussion

## (RRC-1) Transition time from the normal mode to the S&F mode

It was agreed in RAN2#128 [2] that the time information can be broadcasted to indicate when the current satellite operation mode will transit from S&F mode to normal mode (i.e., non-S&F mode), and the usage of such time information is to allow UE to at least delay some NAS procedures until the feeder link is resumed. The time information associated with the opposite transition (from normal mode to S&F mode) was also agreed in the RAN2#129bis [4] meeting, and that time information is at least used by the NAS layer.

In the post-meeting email discussion [Post129bis][310][R19 IoT NTN] RRC CR (Huawei) [5], the rapporteur listed two options on the table for signaling the transition time from the normal to the S&F mode.

(RRC-1) RAN2 to further discuss which of the following options to choose for indicating the transition time from normal mode to S&F mode: Option 1: (5/9) It is up to NW implementation to set the legacy t-Service as the transition time from normal mode to S&F mode. Option 2: (5/9) Using the agreed time information in SIB31 for both directions of transition. UE determines which direction it is based on whether the S&F indication is present.

We think Option 1 is not feasible because it would trigger the neighbor cell measurement and cell reselection too early, resulting in unnecessary UE power consumption. Figure 1 illustrates such a case, where the UE capable of the S&F operation triggers the neighbor cell measurement at t1, which is much earlier than the timing that the UE is supposed to perform the measurement (i.e., right before t2).

Figure 1. The UE capable of S&F operation would trigger the neighbor cell measurement too early, if t-Service is used to indicate when the S&F mode will start.

(RRC-1) If t-Service is used to indicate when the S&F mode will start, S&F-capable UEs could trigger the neighbor cell measurement and cell reselection too early, resulting in unnecessary UE power consumption.

The proponent of Option 1 claims that setting t-Service as the transition time from normal mode to S&F mode can trigger legacy UEs to reselect other normal-mode cells before the serving cell loses its feeder link connection, but this is not going to happen as legacy UEs will still remain in the serving cell due to the qualified service link quality. The best way to influence legacy UEs is to set up the barring bit at appropriate time to direct these UEs to other cell.

(RRC-1) Legacy UEs will still remain in the serving cell after t-Service, even if t-Service is used to indicate when the S&F mode will start.

(RRC-1) The best way to direct legacy UEs to other normal-mode cells is to rely on the legacy barring bit.

Consequently, to have a simpler/cleaner procedure without negatively influencing the Rel-19 UE capable of S&F, the start time of S&F mode should be provided in SIB31, keeping its application separate from t-Service. To optimize the signaling design, there is no need to introduce a new parameter in addition to the agreed parameter (t-ModeSwitching). That is, Option 2 (Using the agreed time information in SIB31 for both directions of transition. UE determines which direction it is based on whether the S&F indication is present) can be considered as the baseline.

(RRC-1) When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

Regarding the usage of the transition time from the normal to S&F mode, RAN2 considered it is at least useful to the UE NAS according to the RAN2#129bis agreement. However, if such transition time can be properly utilized by the UE AS, we believe it is also beneficial at least to the Rel-19 UEs not supporting the S&F operation, as these UEs can determine whether to establish, reestablish, or resume an RRC connection based on whether there is sufficient time to fulfill the access purpose. Without this information, the UEs not supporting the S&F operation may initiate an RRC connection establishment/reestablishment/resume procedure right before the satellite turns into the S&F mode, and then will be released by the network soon after the satellite turns into the S&F mode, which consumes considerable radio resources and UE energy.

In addition, for the UEs not supporting the S&F operation, the principle is to make these UEs stay in the cells operating in the normal mode for as long as possible, even if they are allowed to camp on the cell in the S&F mode. Therefore, while camping on the cell operating in the normal mode and indicating when S&F mode will start, the Rel-19 UEs not supporting the S&F operation should trigger the intra-/inter-frequency measurement before the S&F mode starts. Similar to the case of t-service in Rel-18, these UEs should trigger the intra-/inter-frequency measurement regardless of other triggering conditions (e.g., the serving cell quality, the distance to the reference location).

UE AS can utilize the transition time from the normal mode to S&F mode to determine: 1) when to trigger the neighbor cell measurement, and 2) whether to establish/reestablish/resume an RRC connection.

To increase the likelihood for the UE to detect, measure, and reselect a cell operating in the normal mode (i.e., non-S&F mode), the network can provide a list of cell (PCIs) that are not operating in the S&F mode as part of cell reselection information. The UE can then prioritize the cells in the list while making the cell reselection decision. This enhancement was also listed as an open issue in the post-meeting email discussion [Post129bis][310][R19 IoT NTN] RRC CR (Huawei) [5].

(RRC-2) RAN2 to further discuss whether to introduce the following assistance information for the neighbour cells: - (5/9) Operation mode. - (3/9) Mode transition time

(RRC-2) To prioritize the cells operating in normal mode during the cell reselection, a list of cells not operating in the S&F mode can be provided in the system information.

# Conclusion

In this paper, we discuss the time information on when a cell will transition from the normal to the S&F mode, and have the following observations. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Observation 1	(RRC-1) If t-Service is used to indicate when the S&F mode will start, S&F-capable UEs could trigger the neighbor cell measurement and cell reselection too early, resulting in unnecessary UE power consumption.

Observation 2	(RRC-1) Legacy UEs will still remain in the serving cell after t-Service, even if t-Service is used to indicate when the S&F mode will start.

Observation 3	(RRC-1) The best way to direct legacy UEs to other normal-mode cells is to rely on the legacy barring bit.

Proposal 1	(RRC-1) When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

Proposal 2	UE AS can utilize the transition time from the normal mode to S&F mode to determine: 1) when to trigger the neighbor cell measurement, and 2) whether to establish/reestablish/resume an RRC connection.

Proposal 3	(RRC-2) To prioritize the cells operating in normal mode during the cell reselection, a list of cells not operating in the S&F mode can be provided in the system information.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

R2-2500002, Report of 3GPP TSG RAN WG2 meeting #128, ETSI MCC.

R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, ETSI MCC.

Draft Report of 3GPP TSG RAN WG2 meeting #129bis, ETSI MCC.

[Post129bis][310][R19 IoT NTN] RRC CR, Huawei.