3GPP TSG-RAN WG2 Meeting#130                               	R2-2503799

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.9.4

Source: 	Google

Title: 	Open issues on the support for PWS in NB-IoT

Document for:	Discussion and Decision

# Introduction

The support for the broadcast of PWS messages in NB-IoT was discussed in the RAN2#129 and RAN2#129bis meeting [1][2], resulting in the following agreements.

Agreements in RAN2#129: - Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells. - Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. - Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB. - We don’t introduce a new SIB for carrying geographical information for emergency areas. - Send a LS to CT1 and CT4 indicating that RAN2 currently supports a maximum of 64 segments for SIB11-NB and SIB12-NB with a maximum TBS of 680bits and asking to provide feedback if they think we need to work on changes to allow longer messages (that would require more segments). Also indicate some possible drawbacks. Agreements in RAN2#129bis: - In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle).

Based on the above agreements and the status of the post-meeting email discussion [Post129bis][310][R19 IoT NTN] RRC CR (Huawei) [3], several important open issues are explored in this paper, with the TPs being proposed at the end of the paper.

# Discussion

## (RRC-6) Handling of PWS segments after switching to a new cell

It has been agreed that the LTE SIBs for PWS are used as a baseline for designing NB-IoT SIBs for PWS. In LTE, if a UE (in the idle or connected mode) switching to a new cell has not fully received a segmented PWS message in the original cell, it discards any previously buffered segments, as specified below in 3GPP TS 36.331[2]. This is because the segmentation of the same PWS message can vary across base stations.

- 5.2.2.4 System information acquisition by the UE <unrelated part omitted> 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11; <unrelated part omitted> 1> if the UE is CMAS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

(RRC-6) An LTE UE switching to a new cell would discard any previously buffered PWS segments, if it has not fully received a segmented PWS message in the original cell.

However, in NB-IoT NTN, it is very likely the segmentation of the PWS message in the original cell and in the new cell are performed by the same base station, especially in the transparent satellite payload architecture and when the cells are earth-moving cells. Therefore, discarding previously buffered PWS segments is not always a preferable approach, as it would prolong the PWS reception time significantly, resulting in greater UE power consumption.

(RRC-6) Most likely, an UE in the idle mode would connect to the same eNB while moving from one NTN cell to another. Discarding previously buffered PWS segments in such instances would lead to significant inefficiencies in both resource utilization and power consumption.

In view of this, the rapporteur of the RRC running CR has listed this as an open issue [3].

(RRC-6) RAN2 to further discuss whether to allow UE to receive and assemble PWS segments from different cells during mobility.

To address this issue, we propose that the UE should retain previously buffered PWS segments and resume PWS reception when transitioning to a new NTN cell from a previously served NTN cell. From an implementation perspective, the UE would utilize a more sophisticated condition to assure whether the old and new cells belong to the same eNB, thereby enabling the continuation of PWS reception.

(RRC-6) UE is allowed to retain previously buffered PWS segments and resume PWS reception when transitioning to a new NTN cell from a previously served NTN cell.

If P1 above can be agreed, the TP appended in the Annex A at the end of this paper can be considered as the baseline for updating the current 36.331 running CR.

(RRC-6) If P1 is agreed, RAN2 to adopt the TP in Annex A into the current 36.331 running CR.

## (RRC-4) Inclusion of PWS area information in Paging

The Cell Broadcast Entity (CBE) first takes care of all the aspects of formatting alerts messages, including the splitting of an alter message into a number of pages. The alerts messages are then passed all the way to the MME, and then eventually reach the eNB operating an NTN cell. The UE, under the coverage of the NTN cell and upon receiving the paging message including the CMAS indication, proceeds to acquire SIB12 and obtains the information in SIB12. If the geo-fencing information (i.e., warningAreaCoordinatesSegment) is also included in SIB12, the UE is able to determine whether to take actions (e.g., display the alert messages) based on whether the UE is within the alert area(s) or not. Since the cell coverage of the NTN cell is much larger than the alert area, most of the UEs within the NTN cell do not need to respond to the CMAS message, due to out of the alert area. However, as these UEs (within the NTN cell) still receive the paging message including a CMAS notification, they need to further acquire and store SIB1/SIB12 to determine whether to take further actions, which is a significant power waste for these UEs.

(RRC-4) The likelihood of paging an irrelevant UE in an NTN cell for a PWS alert is very high.

To alleviate the aforesaid UE power waste, the simplest way is to duplicate the geo-fencing information (i.e., warningAreaCoordinatesSegment) in SIB12 and put the same geo-fencing information also in the paging message. However, as the geo-fencing information can include the coordinates of up to 100 vertices, embedding such detailed geo-fencing information in the paging message could bring significant signalling overheads. To achieve good trade-off between the UE power saving and the signalling overheads, the paging message may include the coarse information regarding the affected area of the PWS, where the coarse information is able to indicate affected area in a coarse level, but requires much less bits. For example, as illustrated in Figure 1, an NTN cell can be divided equally into four quadrants in accordance with the reference location of the NTN cell. The paging message includes the coordinate of the reference location and also a 4-bit long bitmap indicating whether there is any alert area within each quadrant, if the CMAS indication (i.e., cmas_Indication) is also present. The four quadrants are mapped to the 1st bit, 2nd bit, 3rd bit, and 4th bit in the bitmap, respectively, and the value ‘1’ in each bit indicates there is at least one alert area while the value ‘0’ indicates there is no alert area in the corresponding quadrant. The reference location can be even omitted in the paging message, and in such case, the UE will acquire and use the reference location in SIB31 as the basis to divide the NTN cell. In the example illustrated in Figure 1, as the alert area falls within the 3rd quadrant, the values of the bitmap are {0, 0, 1, 0}.

(RRC-4) PWS area information in coarse level can be signaled together with a PWS indication in Paging-NB. The UE not within the area determined by the PWS area information can skip acquiring the system information relevant to the PWS.

If P3 above can be agreed, the TP appended in the Annex B at the end of this paper can be considered as the baseline for updating the current 36.331 running CR.

(RRC-4) If P3 is agreed, RAN2 to adopt the TP in Annex B into the current 36.331 running CR.

Figure 1.The coarse geographical information in the Paging-NB divides an NTN cell into four quadrants.

# Conclusion

In this paper, we discuss two open issues for supporting the broadcast of PWS messages in NB-IoT, among the open issues listed by the RRC running CR rapporteur. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

(RRC-6) Handling of PWS segments after switching to a new cell:

Observation 1	(RRC-6) An LTE UE switching to a new cell would discard any previously buffered PWS segments, if it has not fully received a segmented PWS message in the original cell.

Observation 2	(RRC-6) Most likely, an UE in the idle mode would connect to the same eNB while moving from one NTN cell to another. Discarding previously buffered PWS segments in such instances would lead to significant inefficiencies in both resource utilization and power consumption.

Proposal 1	(RRC-6) UE is allowed to retain previously buffered PWS segments and resume PWS reception when transitioning to a new NTN cell from a previously served NTN cell.

Proposal 2	(RRC-6) If P1 is agreed, RAN2 to adopt the TP in Annex A into the current 36.331 running CR.

(RRC-4) Inclusion of PWS area information in Paging:

Observation 3	(RRC-4) The likelihood of paging an irrelevant UE in an NTN cell for a PWS alert is very high.

Proposal 3	(RRC-4) PWS area information in coarse level can be signaled together with a PWS indication in Paging-NB. The UE not within the area determined by the PWS area information can skip acquiring the system information relevant to the PWS.

Proposal 4	(RRC-4) If P3 is agreed, RAN2 to adopt the TP in Annex B into the current 36.331 running CR.

# Reference

R2-2501702, Report of 3GPP TSG RAN WG2 meeting #129, ETSI MCC.

Draft Report of 3GPP TSG RAN WG2 meeting #129bis, ETSI MCC.

[Post129bis][310][R19 IoT NTN] RRC CR, Huawei.

# Annex A: TP for resuming the PWS segment reception

--------------------------------------------- TP on the running CR for TS 36.331[3] ---------------------------------------------------

5.2.2.4	System information acquisition by the UE

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1 or BR-BCCH configuration defined in 9.1.1.8;

1>	if the procedure is triggered by a system information change notification:

2>	if the UE uses an idle DRX cycle longer than the modification period:

3>	start acquiring the required system information, as defined in 5.2.2.3, from the next eDRX acquisition period boundary;

2>	else

3>	start acquiring the required system information, as defined in 5.2.2.3, from the beginning of the modification period following the one in which the change notification was received;

NOTE 1:	The UE continues using the previously received system information until the new system information has been acquired.

1>	if the UE is in RRC_IDLE and enters a cell for which the UE does not have stored a valid version of the system information required in RRC_IDLE, as defined in 5.2.2.3:

2>	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_IDLE, as defined in 5.2.2.3;

1>	following successful handover completion to a PCell for which the UE does not have stored a valid version of the system information required in RRC_CONNECTED, as defined in 5.2.2.3:

2>	acquire, using the system information acquisition procedure as defined in 5.2.3, the system information required in RRC_CONNECTED, as defined in 5.2.2.3;

2>	upon acquiring the concerned system information:

3>	discard the corresponding radio resource configuration information included in the radioResourceConfigCommon previously received in a dedicated message, if any;

1>	following a request from CDMA2000 upper layers:

2>	acquire SystemInformationBlockType8, as defined in 5.2.3;

1>	neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT, SystemInformationBlockType22-NB;

1>	not initiate the RRC connection establishment/resume procedure subject to EAB until the UE has a valid version of SystemInformationBlockType14, if broadcast;

1>	if the UE is ETWS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3> if the UE is not connected to NTN:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB);

2>	when the UE acquires SystemInformationBlockType1(-NB) following ETWS indication, upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType10(-NB) is present:

4>	if the UE is in CE:

5>	start or resume acquiring SystemInformationBlockType10;

4>	else if the UE is an NB-IoT UE:

5>	start or resume acquiring SystemInformationBlockType10-NB immediately;

4>	else

5>	start acquiring SystemInformationBlockType10 immediately;

3>	if schedulingInfoList indicates that SystemInformationBlockType11(-NB) is present:

4>	start or resume acquiring SystemInformationBlockType11(-NB) immediately;

NOTE 2:	UEs shall start acquiring SystemInformationBlockType10(-NB) and SystemInformationBlockType11(-NB) as described above even when systemInfoValueTag in SystemInformationBlockType1(-NB) has not changed.

1>	if the UE is CMAS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment:

3> if the UE is not connected to NTN:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12 associated with the discarded warningMessageSegment;

2>	when the UE acquires SystemInformationBlockType1(-NB) following CMAS indication, upon entering a cell during RRC_IDLE, following successful handover and upon connection re-establishment:

3>	if schedulingInfoList indicates that SystemInformationBlockType12(-NB) is present:

4>	start or resume acquiring SystemInformationBlockType12(-NB);

NOTE 3:	UEs shall start acquiring SystemInformationBlockType12(-NB) as described above even when systemInfoValueTag in SystemInformationBlockType1(-NB) has not changed.

# Annex B: TP for inclusion of PWS area information in Paging-NB

--------------------------------------------- TP on the running CR for TS 36.331[3] ---------------------------------------------------

- Paging-NB

The Paging-NB message is used for the notification of one or more UEs.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: PCCH

Direction: E-UTRAN to UE

Paging-NB message

-- ASN1START

Paging-NB ::=						SEQUENCE {

pagingRecordList-r13				PagingRecordList-NB-r13		OPTIONAL,	-- Need ON

systemInfoModification-r13			ENUMERATED {true}				OPTIONAL,	-- Need ON

systemInfoModification-eDRX-r13		ENUMERATED {true}				OPTIONAL,	-- Need ON

nonCriticalExtension				Paging-NB-v1610-IEs				OPTIONAL

}

Paging-NB-v1610-IEs	::=				SEQUENCE {

pagingRecordList-v1610				PagingRecordList-NB-v1610		OPTIONAL,	-- Need ON

nonCriticalExtension				Paging-NB-v1900-IEs				OPTIONAL

}

Paging-NB-v1900-IEs	::=				SEQUENCE {

etws-Indication-r19					ENUMERATED {true}				OPTIONAL,	-- Need ON

cmas-Indication-r19					ENUMERATED {true}				OPTIONAL,	-- Need ON

alertAreaInformation				BIT STRING (SIZE (4))			OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

PagingRecordList-NB-r13 ::=			SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-NB-r13

PagingRecordList-NB-v1610 ::=		SEQUENCE (SIZE (1..maxPageRec)) OF PagingRecord-NB-v1610

PagingRecord-NB-r13 ::=				SEQUENCE {

ue-Identity-r13						PagingUE-Identity,

...

}

PagingRecord-NB-v1610 ::=			SEQUENCE {

mt-EDT-r16							ENUMERATED {true}			OPTIONAL	-- Need ON

}

-- ASN1STOP

Paging-NB field descriptions

cmas-Indication If present: indication of a CMAS notification.

etws-Indication If present: indication of an ETWS primary notification and/ or ETWS secondary notification.

mt-EDT Indication of mobile-terminated EDT.

pagingRecordList If E-UTRAN includes pagingRecordList-v1610, it includes the same number of entries, and listed in the same order, as in pagingRecordList (i.e. without suffix).

systemInfoModification If present: indication of a BCCH modification other than for SystemInformationBlockType14-NB (SIB14-NB), SystemInformationBlockType16-NB (SIB16-NB), SystemInformationBlockType31-NB (SIB31-NB) and SystemInformationBlockType33-NB (SIB33-NB). This indication does not apply to UEs using eDRX cycle longer than the BCCH modification period.

systemInfoModification-eDRX If present: indication of a BCCH modification other than for SystemInformationBlockType14-NB (SIB14-NB), SystemInformationBlockType16-NB (SIB16-NB), SystemInformationBlockType31-NB (SIB31-NB) and SystemInformationBlockType33-NB (SIB33-NB). This indication applies only to UEs using eDRX cycle longer than the BCCH modification period.

ue-Identity Provides the NAS identity of the UE that is being paged.

alertAreaInformation Provides the alert area information for the CMAS, ETWS primiary, and/or ETWS secondary notifcation in a coarse level. FFS on the details.