3GPP TSG-RAN2#130 	R2-2503819

St. Julian’s, Malta, 19th May – 23th May, 2025

Agenda item:		8.4.4 (NR_LPWUS-Core)

Source:	LG Electronics Inc.

Title: 	Remainng issues on LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 made some agreements for LP-WUS in RRC_CONNECTED as follows.

For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. If configured, the UE can signal a preferred time offset via UAI signalling. Ask RAN1 for further information regarding their conclusions. FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

In post e-mail discussion [1], RAN2 made MAC running CR based on RAN2 agreements, but there are some remaining issues. In this contribution, we further discuss remaining issues on LP-WUS in RRC_CONNECTED.

2.	Discussion

In the last meeting, whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX groups is remained as FFS. In addition, in the last RAN1 meeting, RAN1 also discussed this issue, but there is no consensus.

In Rel-16, the feature of dual DRX groups was introduced for FR1+FR2 CA case, where FR1 cells are configured to default DRX group and FR2 cells are configured to secondary DRX group. Secondary DRX group is configured with shorter drx-onDurationTimer and drx-InactivityTimer, which make secondary DRX group to enter non-Active time quickly than default DRX group, so the power consumption of FR2 cell can be reduced.

Considering that both dual DRX groups and LP-WUS are introduced for power saving, enabling of both feature is more beneficial from power saving point of view. For example, when data scheduling is expected, the UE power is reduced by dual DRX groups, otherwise, the UE power is saved by LP-WUS.

Proposal 1. RAN2 supports to enable dual DRX groups and LP-WUS simultaneously.

If both dual DRX groups and LP-WUS are enabled simultaneously, which DRX group monitors LP-WUS needs to be discussed, i.e., LP-WUS is monitored in each DRX group or one of DRX group. If LP-WUS is monitored in each DRX group, drx-onDurationTimer and lpwus_PDCCHMonitoringTimer can be handled per each DRX group, and this may bring more power saving benefit.

However, in our view, LP-WUS monitoring in each DRX group is not preferable because this introduces the complexity on the UE behaviour and RAN2 needs to discuss many things. Thus, we think that LP-WUS should be monitored in one of DRX group, and it is straightforward to monitor LP-WUS in PCell of the primary DRX group.

Proposal 2. When dual DRX groups and LP-WUS are enabled simultaneously, LP-WUS is monitored in PCell of the primary DRX group.

The next question is how dual DRX groups are operated with LP-WUS. We think there are four case to be considered.

- Case#1. Both DRX groups are not in Active Time

In Case#1, LP-WUS is monitored in default DRX group and LP-WUS occasion is not overlapped with Active time of both DRX groups.

We believe that there is no doubt about that both DRX groups operate based on LP-WUS in this case, i.e., if LP-WUS is detected, the UE starts drx-onDurationTimer and lpwus_PDCCHMonitoringTimer for both DRX groups, if not, the UE does not start drx-onDurationTimer and lpwus_PDCCHMonitoringTimer for both DRX groups.

Proposal 3. If LP-WUS occasion is not overlapped with Active time of any DRX group,

- the UE monitors LP-WUS on LP-WUS occasion

- the UE starts drx-onDurationTimer and lpwus_PDCCHMonitoringTimer for both DRX groups based on LP-WUS

- Case#2. Primary DRX group is in Active Time but Secondary DRX group is not in Active time

In Case#2, LP-WUS is monitored in default DRX group and LP-WUS occasion is overlapped with Active time of default DRX group.

In RAN1#120, RAN1 already agreed that the UE does not monitor LP-WUS during C-DRX active time. Thus, LP-WUS is not monitored in default DRX group.

Agreement For RRC connected mode, UE does not monitor LP-WUS during C-DRX active time. - FFS FAR and MDR impact.

Then, in the last meeting, RAN2 made WA that in Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected) and FFS for Option 1-2. Thus, it would be desirable to start drx-OnDurationTimer for both DRX groups as if LP-WUS is detected, and lpwus_PDCCHMonitoringTimer is FFS.

Proposal 4. If LP-WUS is monitored in default DRX group and LP-WUS occasion is overlapped with Active time of default DRX group,

- the UE does not monitor LP-WUS occasion

- the UE starts drx-onDurationTimer for both DRX groups as if LP-WUS is detected. FFS for lpwus_PDCCHMonitoringTimer.

- Case#3. Primary DRX group is not in Active Time but Secondary DRX group is in Active time

In Case#3, LP-WUS is monitored in default DRX group and LP-WUS occasion is not overlapped with Active time of default DRX group but overlapped with Active time of secondary DRX group.

In this case, if the UE can monitor LP-WUS in default DRX group, the UE starts drx-onDurationTimer based on the received LP-WUS. However, it is not clear whether the UE can monitor LR and MR simultaneously, so RAN2 asked RAN1 about this in LS [2]. This would be discussed later based on RAN1 response.

Proposal 5. For the case where LP-WUS is monitored in default DRX group and LP-WUS occasion is not overlapped with Active time of default DRX group but overlapped with Active time of secondary DRX group, RAN2 would discuss later based on RAN1 response.

- Case#4. Both DRX groups are in Active Time

In Case#4, LP-WUS is monitored in default DRX group and LP-WUS occasion is overlapped with Active time of both DRX groups.

Similar to Case#2, the UE does not monitor LP-WUS occasion in default DRX group. Thus, it would be desirable to start drx-OnDurationTimer for both DRX groups as if LP-WUS is detected. lpwus_PDCCHMonitoringTimer is FFS.

Proposal 6. If LP-WUS is monitored in default DRX group and LP-WUS occasion is overlapped with Active time of both DRX groups,

- The UE does not monitor LP-WUS occasion

- the UE starts drx-onDurationTimer for both DRX groups as if LP-WUS is detected. FFS for lpwus_PDCCHMonitoringTimer.

Meanwhile, based on the last meeting contribution, we think there are some issues left to be discussed.

In RAN1#116, RAN1 agreed to enable/disable LP-WUS functionality by RRC signaling, and whether to support activation/deactivation is left for further study point.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Based on this, in the last meeting, it was proposed to support activation/deactivation of LP-WUS functionality in contributions.

However, considering that LP-WUS is introduced for UE power saving, LP-WUS functionality may not be enabled/disabled or activated/deactivated frequently. Thus, activation/deactivation of LP-WUS functionality based on L1/L2 signaling is not needed.

In addition, since the UE reports CSI reporting in Active time and may report measurement report in cell edge, the network can know the UE’s condition whether the UE can receive LP-WUS or not. Based on the network decision, the network may enable/disable LP-WUS functionality by RRC signaling.

Therefore, we think other than RRC signaling is not needed to enable/disable or activate/deactivate LP-WUS functionality, and UE assistance information is also not needed.

Proposal 7. LP-WUS functionality is enabled/disabled by RRC signaling only.

Proposal 8. No additional method to activate/deactivate LP-WUS functionality and no UE assistance information are needed.

In addition, it was also proposed to discuss whether the UE wakes up MR for SPS/CG operation while the UE monitors LP-WUS on LR. From power saving point of view, it may be reasonable to deactivate SPS/CG while the UE monitors LP-WUS.

However, if CG is deactivated, the UE cannot transmit UL data on CG until LP-WUS is received, but the network does not know whether the UE have UL data or not. In addition, if SPS is deactivated, the network needs to wait until the next LP-WUS to wake up the UE. Thus, deactivation of SPS/CG would introduce the latency and we think this is not reasonable.

Therefore, we propose that LP-WUS is not impacted to SPS/CG operation.

Proposal 9. LP-WUS is not impacted to SPS/CG operation.

3.	Conclusion

In this document, we discuss issues on LP-WUS in RRC_CONNECTED, and make proposals as follows.

Proposal 1. RAN2 supports to enable dual DRX groups and LP-WUS simultaneously.

Proposal 2. When dual DRX groups and LP-WUS are enabled simultaneously, LP-WUS is monitored in PCell of the primary DRX group.

Proposal 3. If LP-WUS occasion is not overlapped with Active time of any DRX group,

- the UE monitors LP-WUS on LP-WUS occasion

- the UE starts drx-onDurationTimer and lpwus_PDCCHMonitoringTimer for both DRX groups based on LP-WUS

Proposal 4. If LP-WUS is monitored in default DRX group and LP-WUS occasion is overlapped with Active time of default DRX group,

- the UE does not monitor LP-WUS occasion

- the UE starts drx-onDurationTimer for both DRX groups as if LP-WUS is detected. FFS for lpwus_PDCCHMonitoringTimer.

Proposal 5. For the case where LP-WUS is monitored in default DRX group and LP-WUS occasion is not overlapped with Active time of default DRX group but overlapped with Active time of secondary DRX group, RAN2 would discuss later based on RAN1 response.

Proposal 6. If LP-WUS is monitored in default DRX group and LP-WUS occasion is overlapped with Active time of both DRX groups,

- the UE does not monitor LP-WUS occasion

- the UE starts drx-onDurationTimer for both DRX groups as if LP-WUS is detected. FFS for lpwus_PDCCHMonitoringTimer.

Proposal 7. LP-WUS functionality is enabled/disabled by RRC signaling only.

Proposal 8. No additional method to activate/deactivate LP-WUS functionality and no UE assistance information are needed.

Proposal 9. LP-WUS is not impacted to SPS/CG operation.

4.	Reference

- [Post129bis][210][LPWUS] Running CR for 38.321 (Apple)

- R2-2503187	LS on LP-WUS in RRC_CONNECTED	RAN2