3GPP TSG-RAN WG2 Meeting #130	R2-2503827

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.2

Source:	Sharp

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion and Decision

# Introduction

In RP#102 meeting, the following objective was approved as part of the WID [1].

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Moreover, the following agreements were made by RAN2#129bis meeting [2].

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

In this contribution, we are going to discuss remaining issues on on-demand SSB (OD-SSB) SCell operation.

# Discussions

In the current specifications, the measurement configuration (i.e., measConfig) may include measurement objects, reporting configurations, and measurement identities, where each measurement identity associates one measurement object with one reporting configuration. For serving cell measurement purpose, when a cell is configured, the UE may be further configured with a measurement object indication (i.e., servingCellMO) indicating the cell is associated with the measurement object indicated by servingCellMO. Then the UE may perform serving cell measurement based on the parameters (e.g., smtc1, ssb-ToMeasure) provided in the associated measurement object, derive measurement results based on the L3 filtering parameters provided in the associated quantity configuration as well as perform measurement reporting based on the parameters (e.g., reportInterval, reportAmount, timeToTrigger) provided in the associated reporting configuration(s).

For Case 2, always-on SSB is SSB supported in Rel-18 specifications, and the UE may perform always-on SSB based measurement as mentioned above. However, after on-demand SSB is activated, the UE may incorrectly perform on-demand SSB based measurement since the legacy parameters (e.g., smtc1, ssb-ToMeasure) provided in the associated measurement object are configured for always-on SSB instead of on-demand SSB. To resolve this, one option is to configure additional corresponding parameters (e.g., smtc1, ssb-ToMeasure) which can be enabled when on-demand SSB is activated. Thus, we suggest RAN2 WG discusses what additional parameter(s) needs to be provided in the measurement object for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated. Similarly, whether the additional parameter(s) (e.g., reportInterval, reportAmount, timeToTrigger) provided in the associated reporting configuration(s) and additional L3 filtering parameters provided in the associated quantity configuration are needed for on-demand SSB can be further studied.

Proposal 1: For Case 2, RAN2 WG to discuss what additional parameter(s) needs to be provided in the measurement object/report configuration/quantity configuration for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated.

As shown below, according to Section 5.5.3.1 of TS 38.331 [3], the UE shall perform SSB-based measurements (e.g., RSRP, RSRQ, SINR) for a serving cell when the serving cell is configured with servingCellMO. Moreover, in the RAN2#129bis meeting, we made a working assumption to introduce a new servingCellMO in ServingCellConfig to indicate the measurement object for OD-SSB, in cases where always-on SSB (A-SSB) and OD-SSB have different center frequencies. We understand this means the network can configure a serving cell with two servingCellMOs, indicating separate measurement objects for A-SSB and OD-SSB are associated with the serving cell. As a result, based on the current specification, the UE would measure both AO-SSB and OD-SSB when a serving cell is configured with two servingCellMOs.

TS 38.331 Section 5.5.3.1 The UE shall: 1> whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows: 2> if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO: 3> if the reportConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 4> derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 3> derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3; Text omitted 1> for each serving cell for which servingCellMO is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity: 2> if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO: 3> if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: 4> derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a; 3> derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3; Text omitted

However, this contradicts the agreement reached by RAN1 at the RAN1#120 meeting, as detailed below. Therefore, we propose that RAN2 discuss ways to resolve this conflict.

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Proposal 2: For Case 2, RAN2 to discuss the UE behavior for the serving cell measurement when a serving cell is configured to be associated with separate measurement objects of A-SSB and OD-SSB.

In RAN1#120bis meeting, the following agreement on Scenario#3B was achieved:

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

It means after SCell activation completion and OD-SSB activation, a MAC CE is used for only updating OD-SSB parameters. Now there is a MAC CE for activation/deactivation of OD-SSB, we think this MAC CE should also be used for updating, i.e., a single MAC CE format should be considered for both activation/deactivation of OD-SSB and updating OD-SSB parameters.

Proposal 3: A single MAC CE format is used for both activation/deactivation of OD-SSB and updating OD-SSB parameters.

# Conclusions

Based on the discussion above, we have the following proposals:

Proposal 1: For Case 2, RAN2 WG to discuss what additional parameter(s) needs to be provided in the measurement object/report configuration/quantity configuration for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated.

Proposal 2: For Case 2, RAN2 to discuss the UE behavior for the serving cell measurement when a serving cell is configured to be associated with separate measurement objects of A-SSB and OD-SSB.

Proposal 3: A single MAC CE format is used for both activation/deactivation of OD-SSB and updating OD-SSB parameters.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator).

- “Draft Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China”, Chairman (InterDigital).

- TS 38.331 V18.4.0, Radio Resource Control (RRC) protocol specification, 2024-12.