3GPP TSG-RAN WG2 Meeting #130	R2-2503828

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.5.4

Source:	Sharp

Title:	Discussion on PRACH adaptation

Document for:	Discussion and Decision

# Introduction

In RP#102 meeting, the following objective was approved as part of the WID [1].

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we are going to discuss adaptation of PRACH in time domain.

# Discussions

In the current specification, when an RA procedure is initiated, the UE first determines the availability of the configured set(s) of RA resources. From the available RA resource set(s), the UE selects one set for the ongoing RA procedure. The UE then determines the RA type (i.e., 4-step or 2-step) based on the selected set of RA resources. Once the RA type is determined, the UE performs the Msg1 transmission, during which it selects the SSB, determines the preamble index, and selects a RO to transmit the determined preamble.

In addition, the following agreement was made:

Agreement from RAN2#129 Follow the legacy mechanism for RACH resource selection.

We understand that this agreement implies there is no prioritization between legacy RA resources and additional RA resources. Therefore, we assume that, when additional RA resources are configured, the UE treats both the legacy and the additional RA resources as belonging to the same RA resource set without applying prioritization. In other words, a set of RA resources can comprise either legacy RA resources only or both legacy RA resources and additioanl RA resources.

Proposal 1: When additional RA resources are configured, the UE treats the legacy and additional RA resources as belonging to the same set of RA resources without applying prioritization.

Moreover, RAN1 agreed that additional RA resources are activated via DCI (using P-RNTI or C-RNTI) as shown below, which means the MAC layer has no way of knowing whether the additional RA resources are activated.

Agreement from RAN1#119 Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement from RAN1#120 For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication

Thus, it becomes necessary that, once an RA procedure is triggered, for the initial Msg1 transmission, if the selected RA resource set includes additional RA resources, the MAC shall determine their availability based on an indication from the lower layer.

Proposal 2: For the initial Msg1 transmission, if the selected set of RA resources includes additional RA resources, the MAC shall determine their availability based on an indication from the lower layer.

Additionally, it is noted that the MAC determines the RO for Msg1 retransmission as well. According to RAN1’s agreement, additional RA resources are only valid within a specific period, meaning they may not be available at the time of Msg1 retransmission. Therefore,  the availability of additional RA resources must also be verified during Msg1 retransmission.

Proposal 3: Upon Msg1 retransmission, if the selected set of RA resources includes additional RA resources, the MAC shall determine their availability based on an indication from the lower layer.

So far, RAN1 and RAN2 have agreed that PRACH adaptation is supported for at least 4-step RA.

Agreement from RAN1#120b For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon Agreement from RAN2#129 RAN2 starts 4-step RACH adaptation.

However, it remains unclear whether this support extends to scenarios where the RA type switched from 2-step RA to 4-step RA. We see no issues in supporting this case. Thus, we propose that PRACH adaptation should also support this scenario.

Proposal 4: PRACH adaptation is supported for 4-step RA that is switched from 2-step RA.

# Conclusions

Based on the discussion above, we have the following proposals:

Proposal 1: When additional RA resources are configured, the UE treats the legacy and additional RA resources as belonging to the same set of RA resources without applying prioritization.

Proposal 2: For the initial Msg1 transmission, if the selected set of RA resources includes additional RA resources, the MAC shall determine their availability based on an indication from the lower layer.

Proposal 3: Upon Msg1 retransmission, if the selected set of RA resources includes additional RA resources, the MAC shall determine their availability based on an indication from the lower layer.

Proposal 4: PRACH adaptation is supported for 4-step RA that is switched from 2-step RA.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator).