3GPP TSG-RAN WG2 Meeting #130			     R2-2503831

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.6 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining open issues on rate control signaling for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses remaining open issues on XR UL rate control.

Discussion

Following open issues are identified during the MAC running CR discussion.

- [MAC-04] Which type of ID should be used to identify a QoS flow, e.g. DRB ID + QFI ID or some other identifier - [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows - [MAC-06] Any additional fields that should be included in the Rate Control MAC CE - [MAC-07] Format of the Rate Control MAC CE - [MAC-08] Handling of triggered UL rate queries (e.g. multiplexing, transmission and cancelation, etc) - [MAC-09] How UE should handle the Rate Control MAC CE in DC configuration - [MAC-10] Behavior of bitRateQueryProhibitTimer - [MAC-11] Whether to apply the same design for UL rate control design to DL data transmission - [MAC-12] How to indicate in the Rate Control MAC CE that a query from UE is for available bit rate.

The first issue is which type of ID should be used to identify a QoS flow. Based on contributions in the last RAN2 meeting, two options can be considered:

- Option 1: PDU session ID + QoS flow ID;

- Option 2: DRB ID + QoS flow ID.

Both options work well since QoS flow ID is unique in one PDU session and QoS flow ID is unique in one DRB. The difference is length of each ID, i.e., 8bits for PDU session ID and 5bits for DRB ID. The DRB ID is already used for several MAC CEs but the PDU session ID is never used so far in the MAC specification. Thus, the option 2 is preferred for this issue.

Proposal 1. DRB ID + QoS flow ID is used to indicate a QoS flow in the rate control MAC CE.

The next is whether the Rate Control MAC CE includes single or multiple QoS flows. One of main XR traffic characteristic is that one XR service has multiple QoS flows. Typically, if one XR service needs UL rate control, multiple QoS flows may need to be controlled. In this sense, it would be good to include information for multiple QoS flows in a rate control MAC CE.

Proposal 2. The Rate Control MAC CE can include information for multiple QoS flows.

For the details of UL rate queries, it would be good to reuse Recommended Bit Rate procedure as much as possible, but the granularity should be different since UL rate control MAC CE can control per QoS flow. With this in mind, the prohibit timer is configured per the QoS flow. If the prohibit timer for a QoS flow is configured and it is not running and the allocated UL resources can accommodate a rate control MAC CE, the MAC entity instructs the Multiplexing and Assembly procedure to generate the rate control MAC CE for the QoS flow and starts the prohibit timer for the QoS flow, then the MAC entity cancels UL rate queries for the QoS flows included in the rate control MAC CE.

Proposal 3. The prohibit timer is configured per QoS flow.

Proposal 4. When a UL rate query for a QoS flow is triggered and not cancelled,

- if the prohibit timer for the QoS flow is configured and it is not running and the allocated UL resources can accommodate a rate control MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1, the MAC entity shall

  - instruct the Multiplexing and Assembly procedure to generate the rate control MAC CE for the QoS flow

  - start the prohibit timer for the QoS flow included in the rate control MAC CE

  - cancel UL rate queries for the QoS flows included in the rate control MAC CE

One more discussion point is about a priority of a rate control MAC CE for bit rate query. As shown in the below box, each MAC CE should have a priority for the LCP and we think that the rate control MAC CE for bit rate query should have same priority as MAC CE for Recommended bit rate query which is lower than data from any logical channels. In this situation, sufficient uplink resources may not be allocated for a rate control MAC CE for bit rate query and a truncated MAC CE for bit rate query may be considered.

Proposal 5. The rate control MAC CE for bit rate query has same priority as MAC CE for Recommended bit rate query for the LCP procedure.

- data from any Logical Channel, except data from UL-CCCH; - MAC CE for Recommended bit rate query; - MAC CE for BSR included for padding;

The next issue is how to use the index 0 in the new table. In RBR MAC CE, the index 0 in the bit rate field table has special meaning, i.e., “For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given”. During the MAC running CR discussion, similar wording was added as NOTE 1 for the index 0 in the new table (now it is removed, but FFS), i.e., “For bit rate recommendation, this index is used to indicate that no new recommendation on bit rate is given. For bit rate query, this index is used to indicate that the UE is not requesting a specific bit rate.”, but companies indicates that it doesn’t give any value to rate control MAC CE. This is because if network indicates that there is no new recommendation on the bit rate, the network would send the same recommended bit rate as previous one to the UE. It is also indicated that if the previous rate control MAC CE has not been successfully received by the UE, the UE cannot correctly understand the network intention for the index 0. We agree with these observations and don’t see need for this NOTE 1 for the index 0.

For the available bit rate query, we don’t think that the available bit rate query is useful for uplink rate control MAC CE due to similar reason addressed above. Rather, it is better to send desired bit rate to the network and then if the network can accept this desired bit rate, the network can allow to use this desired bit rate, but if the network cannot accept this desired bit rate, the network send a recommended bit rate which is different from the desired bit rate in the rate control MAC CE. We think that this is simple and sufficient for XR rate control and the available bit rate query is not needed.

Proposal 6. The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit.

Proposal 7. The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate.

The last issue is how UE should handle the Rate Control MAC CE in DC configuration. When the DC is configured, the anchor point is CU-UP and the bit rate of each QoS flow should be measured at the CU-UP. The CU-UP can determine the recommended bit rate of a QoS flow and this recommended bit rate of each QoS flow would be transferred to the DU who generates/transmits a rate control MAC CE including this recommended bit rate of each QoS flow to the UE. In this situation, it is not sure why MN and SN transfer two different recommended bit rates for same QoS flow to the UE. We think that no enhancement is needed considering DC configuration and it would be sufficient to just apply the received rate control MAC CE without any further condition.

Proposal 8. No special handling for UL split bearer is needed for rate control MAC CE.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Proposal 1. DRB ID + QoS flow ID is used to indicate a QoS flow in the rate control MAC CE.

Proposal 2. The Rate Control MAC CE can include information for multiple QoS flows.

Proposal 3. The prohibit timer is configured per QoS flow.

Proposal 4. When a UL rate query for a QoS flow is triggered and not cancelled,

- if the prohibit timer for the QoS flow is configured and it is not running and the allocated UL resources can accommodate a rate control MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1, the MAC entity shall

  - instruct the Multiplexing and Assembly procedure to generate the rate control MAC CE for the QoS flow

  - start the prohibit timer for the QoS flow included in the rate control MAC CE

  - cancel UL rate queries for the QoS flows included in the rate control MAC CE

Proposal 5. The rate control MAC CE for bit rate query has same priority as MAC CE for Recommended bit rate query for the LCP procedure.

Proposal 6. The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit.

Proposal 7. The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate.

Proposal 8. No special handling for UL split bearer is needed for rate control MAC CE.