3GPP TSG-RAN WG2 Meeting #130	R2-2503834

Malta, 19 – 23 May, 2025

Agenda item:	8.7.4.2

Source:	InterDigital

Title:	DSR enhancements for UL scheduling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2#129 made the following agreements wrt. the DSR enhancement:

- One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

# 2	Discussion

## 2.1	New DRX MAC CE format applicability

In the previous meeting, it was left FFS whether the new DSR MAC CE format will be always applied when at least one LCG is configured with multiple thresholds. Actually, since the new agreed DSR MAC CE format to report multiple reporting thresholds does not suffer from any higher overhead compared to legacy DSR MAC CE format when any LCG is configured with only one reporting threshold, there is no reason not to apply the new DSR MAC CE format always when at least one LCG is configured even with a single reporting threshold.

Proposal 1: New DSR MAC CE format will always be used when at least one LCG is configured with one or more reporting thresholds.

## 2.2	Reporting control PDUs and re-tx data in the new DSR MAC CE

In the previous RAN2#129-Bis meeting, RAN2 discussed the reporting of re-transmission data (PDCP or RLC) and control PDUs (PDCP or RLC) in the new DSR MAC CE, the following agreement was reached.

During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

Comparing to the Release 18 behaviour wrt. control PDU/re-tx data reporting in DSR MAC CE, if the LCG has no pending DSR for any associated LCH, the DSR is not reported for that LCG. Since the DSR can be triggered only by new delay critical data in the PDCP, the control PDU/re-tx data defined delay critical data alone would never be reported. This is specified in TS 38.321 as in the following:

### 5.4.9 Delay status reporting If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel. #### 6.1.3.72 Delay Status Report MAC CE The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. The Remaining Time, the BT, and the Buffer Size fields for an LCG shall be reported in two consecutive octets. These three fields for different LCGs shall be included in a DSR MAC CE in ascending order based on the LCGi.

Observation 1: In Release 18, if an LCG has no pending DSR(s) when the DSR MAC CE is being built, the DSR is not reported for that LCG. Hence, control PDU(s)/Re-Tx data alone would never be reported for an LCG.

Thus, it seems simplest approach is to follow the same principle for Release 19 new DSR MAC CE reporting as well and use the smallest reported threshold of an LCG for any control PDUs / Re-transmission data. This avoids any special cases where control PDUs and re-transmission data may need to be reported but no other delay critical data exists out from which the remaining time value could be deduced for the LCG in the new DSR MAC CE.

Proposal 2: The control PDUs and re-transmission data for an LCG are only reported in the new DSR MAC CE provided that there is a pending DSR for the LCG.

Proposal 3: The additional delay critical data amount from control PDUs and re-transmission data is included into the smallest reported threshold in the new DSR MAC CE.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: New DSR MAC CE format will always be used when at least one LCG is configured with one or more reporting thresholds.

Observation 1: In Release 18, if an LCG has no pending DSR(s) when the DSR MAC CE is being built, the DSR is not reported for that LCG. Hence, control PDU(s)/Re-Tx data alone would never be reported for an LCG.

Proposal 2: The control PDUs and re-transmission data for an LCG are only reported in the new DSR MAC CE provided that there is a pending DSR for the LCG.

Proposal 3: The additional delay critical data amount from control PDUs and re-transmission data is included into the smallest reported threshold in the new DSR MAC CE.