3GPP TSG-RAN WG2 Meeting #130			         R2-2503838

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.4.3

Source: 	LG Electronics Inc.

Title:         	Remaining issues on measurement offloading and relaxation

Document for: 	Discussion and Decision

Introduction

In this contribution we discuss the remaining issues on the RRM measurement offloading and relaxation.

Discussion

[Open issue 38304-2] FFS enhancements on R16 relaxed measurement criteria

Since the Rel-16 relaxed measurement criteria, i.e. for UE with low mobility and UE not at cell edge, are based on Srxlev. UE supporting both Rel-16 RRM relaxation and Rel-19 RRM offloading will not be able to derive Srxlev during RRM offloading, and therefore will not be able to start or stop the Rel-16 relaxed measurement.

There are two possible solutions to address this issue. The first is to disallow Rel-16 relaxed measurement during RRM offloading. If the network configures Rel-19 RRM relaxation together, then Rel-19 UEs would not need to perform Rel-16 RRM relaxation. The second approach is to introduce a LR threshold that allows UEs to determine whether to perform Rel-16 relaxed measurement based on the LR measurements. However, it does not seem desirable that network configures separate configuration to enable Rel-16 RRM relaxation for Rel-19 UEs, although it supports Rel-19 RRM relaxation also. Therefore, we propose not to consider the scenario where network configures RRM offloading and Rel-16 RRM relaxation without Rel-19 RRM relaxation, and if Rel-19 RRM relaxation and Rel-16 RRM relaxation are configured together, Rel-19 UEs performs Rel-19 RRM relaxation only.

Proposal 1	No enhancements of Rel-16 relaxed measurement criteria for UEs supporting RRM offloading.

Proposal 2	Do not consider the scenario where RRM offloading and Rel-16 RRM relaxation are configured together without Rel-19 RRM relaxation.

Proposal 3	If Rel-19 RRM relaxation and Rel-16 RRM relaxation are configured together, Rel-19 UEs performs Rel-19 RRM relaxation only.

The MR measurement results of the serving cell in RRC_IDLE/INACTIVE are used not only for Rel-16 relaxed measurement criteria but also for various other procedures. It is necessary to discuss whether to support LR measurement-based UE behaviour in each case and what spec changes are required for that. If supporting LR measurements is not appropriate, then we need to consider what constraints are necessary in the spec to prevent that.

LR measurement-based cell-ranking criterion Rs for serving cell

Although there is no agreement on this yet, may companies already have suggested in their papers that the threshold for RRM offloading to LR should be higher than the threshold for neighbor cell measurements, i.e. S-intraSearchP. This is because it is no appropriate to directly compare the LR measurement results, which have different accuracy performance, with neighbor cell measurements performed by MR. Therefore, by enforcing this network configuration constraint, it is possible to prevent support for cell-ranking criteria based on the LR measurements.

The mobility performance should not be compromised for power saving, and the mobility decision should be made based on reliable MR measurements. Therefore, the LR measurements based cell reselection criterion should not be allowed under the assumption that the threshold for measurement offloading is always higher than the threshold for skipping intra-frequency measurement.

Accordingly, we propose to specify that the RRM offloading to LR threshold should always be set higher than S-IntraSearchP in the corresponding field description in TS 38.331.

Proposal 4	Specify that the threshold for RRM offloading to LR should always be set higher than S-IntraSearchP in the field description.

LR measurement-based measurement rules for cell re-selection

The measurement rules for cell re-selection use four thresholds for evaluating the serving cell’s measurement results: s-IntraSearchP, s-IntraSearchQ, s-NonIntraSearchP, and s-NonIntraSearchQ. Since s-IntraSearchP is always greater than s-NonIntraSearchP, and s-IntraSearchQ is always greater than s-NonIntraSearchQ, if Proposal 1 is accepted, the UE will always be able to determine the measurement rule based on the MR measurement results. Therefore, the RRM offloading to LR will have no impact on the measurement rules for cell re-selection.

Observation 1	If Proposal 4 is accepted, the RRM offloading to LR will have no impact on the measurement rules for cell re-selection.

LR measurement-based uplink carrier selection for RA/SDT (NUL or SUL)

Upon initiation of random access of SDT, UE selects uplink carrier between NUL and SUL based on the measurement results of the serving cell, i.e. the RSRP of the downlink pathloss reference. The uplink carrier should be selected regardless of whether the serving cell quality is good or poor, so UE should be able to select the uplink carrier even when the serving cell is measured by LR only. Since the transmit power of the SSB and that of the LP-SS may differ, it is not appropriate to use the existing threshold intended for comparing the SSB measurement results, i.e. rsrp-ThresholdSSB-SUL, for comparing the LR measurement. Accordingly, a separate threshold, e.g. rsrp-ThresholdLP-SS-SUL, is needed to enable the UE to select the uplink carrier based on the LR measurements.

Alternatively, if RRM offloading is stepped and the serving cell measurement using MR is started when the random access or SDT is initiated, the UE will be able to select the uplink carrier based on the MR measurements. However, the UL carrier selection may be delayed due to the time it takes to reach a state where the MR measurement is possible and the time required to acquire a stable MR measurement result for the serving cell. Therefore, we propose to discuss which option is preferred and choose one of them.

Proposal 5	For uplink carrier selection between NUL and SUL, choose one of the following two options:

- Option 1. Introduce a LR threshold, e.g. rsrp-ThresholdLP-SS-SUL, that allows UE to select the uplink carrier for random access or SDT based on LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of the random access or SDT.

LR measurement-based SDT condition is met

For a UE configured with SDT, the decision to perform SDT is made by comparing the SSB based measurements, i.e. RSRP of the downlink pathloss reference, and the SSB thresholds, i.e. sdt-RSRP-Threshold, cg-SDT-RSRP-ThresholdSSB, mt-SDT-RSRP-Threshold. If UE starts to perform the serving cell measurement using MR upon initiation of SDT as in Option 2 in Proposal 2, LR thresholds would not be necessary. However, if we go with Option 1 in Proposal 2, we would similarly need to introduce LR thresholds for SDT.

Proposal 6	For initiation of SDT, choose one of the following two options:

- Option 1. Introduce LR thresholds, e.g. sdt-LR-RSRP-Threshold, cg-SDT-LR-RSRP-ThresholdLP-SS, and mt-SDT-LR-RSRP-Threshold, that allows UE to determine whether to initiate SDT based on the LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of SDT.

LR measurement-based conditions for resuming RRC connection for multicast reception

Network can configure a threshold, i.e. ThresholdMBS, for UEs receiving a multicast session in RRC_INACTIVE, and if the serving cell quality becomes below that the threshold, the UE transition to RRC_CONNECTED via RRC resume. Since this threshold is also designed for comparison with SSB measurements, it is not appropriate for use while performing the RRM offloading to LR.

However, considering that the purpose of the RRM offloading to LR is to prevent UE’s MR from waking up solely for serving cell measurement, it is difficult to expect power saving through the RRM offloading to LR for UEs receiving MBS session using MR. In fact, the LR measurement might consume even more power. Therefore, we propose selecting one of the following two options.

Proposal 7	For UEs supporting Rel-18 multicast reception in RRC_INACTIVE and the RRM offloading, choose one of the following two options:

- Option 1. Introduce LR threshold, e.g. ThresholdMBS-LR, that allows UEs receiving multicast in RRC_INACTIVE to determine whether to perform RRC resume based on LR measurements.

- Option 2. UEs receiving MBS session in RRC_IDLE/INACTVEI do not perform the RRM offloading.

LR measurement-based Early Measurement Reporting

The legacy RRM relaxation is not allowed for a frequency if EMR is configured for that frequency.

TS 38.304 The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

Similar to the legacy relaxed measurement, the LR measurement is also not appropriate for EMR purpose. Only normal measurement using MR should be reported for EMR.

Proposal 8	If EMR is configured for the serving frequency, the UE does not perform the RRM offloading to LR.

Proposal 9	UE does not perform Rel-19 relaxed measurement for frequencies for which EMR is configured.

[Open issue RRC-9] FFS on whether/how RRM relaxation is applicable for high priority frequency

RAN4 has decided to support relaxed measurement for higher priority frequencies in case 1, where the UE measures the serving cell using only low resolution (LR). Whether to support relaxed measurement for higher priority frequencies in case 3 is still under discussion.

@RAN4#114bis - Considerations on higher priority frequency layer - Support relaxed higher priority frequency layer search at case1

RAN2 has designed RRM offloading and RRM relaxation as separate function. That is, if the RRM offloading condition is satisfied, UE can perform RRM offloading. To perform RRM relaxation, the RRM relaxation condition must be satisfied. We may consider changing this principle depending on the RAN4 agreement. For example, if the RRM offloading condition is met, the UE measures the serving cell using LR and performs relaxed measurements for higher priority frequencies.

However, the meaning of the RAN4 agreement is that, while the UE is measuring the serving cell using LR, the measurement requirements allow the UE to also perform relaxed measurements for higher priority frequencies, and thus it seems feasible to maintain this principle as well. Namely, the UE performs LR measurement for serving cell and relaxed measurement for higher priority frequencies according to the measurement requirements defined by RAN4, if both the RRM offloading and RRM relaxation conditions are satisfied.

The running CR has already been written based on this principle, and in the CR ‘offloading’ does not include any relaxed measurement. In order to minimize further work on the CR, we prefer to maintain the principle that the UE performs relaxed measurements only when the RRM relaxation condition is met.

Therefore, we propose to wait until RAN4 completes its discussion on case 3. If RAN4 decides to support relaxed measurement for higher priority frequencies in case 3 as well, we can reflect the RAN4 agreement in TS 38.304 by simply specifying that “if the RRM relaxation condition is met, the UE performs relaxed measurements for all cells including higher priority frequencies.”

Proposal 10	Wait until RAN4 completes its discussion on RRM relaxation for high priority frequency in case 3.

[Open issue RRC-7/38304-3] FFS on exit condition for serving cell RRM relaxation

While the UE is performing LP-WUS monitoring or RRM offloading, it does not have MR measurement results. Therefore, an exit condition based on LR measurement results is necessary. This is why we have defined separate exit conditions for LP-WUS monitoring and RRM offloading, in addition to the entry condition.

However, since the UE can still obtain MR measurement results while performing RRM relaxation, it can determine whether to discontinue RRM relaxation based on both MR and LR measurement results, just as it does for the entry condition. As in R16 RRM relaxation, the UE can stop RRM relaxation if the entry condition is no longer met without separate exit condition. Therefore, we propose following:

Proposal 11	No separate exit condition other than ‘not fulfilling the entry condition’ for Rel-19 RRM relaxation.

[Open issue RRC-10/38304-6] FFS on low mobility criteria

Sone companies have expressed concerns regarding the radio switching of RRM offloading/relaxation status for UEs in a high mobility state. However, this issue can be mitigated to some extent by configuring separate thresholds for entry condition and exit condition. I.e. higher threshold in entry condition. Furthermore, even if the ping-pong happens, it should not be problematic, as it only results in UE changing the receiver used for serving cell measurement or a change in the measurement requirements. Therefore, the low mobility criteria do not need to be considered in Rel-19 RRM relaxation/offloading.

Proposal 12	Do not consider the Low mobility criteria for Rel-19 RRM offloading/relaxation.

[Open issue RRC-8/38304-4] FFS on whether/how to reduce the threshold number for LP-WUS/WUR

In the last meeting, we started discussion on reducing the total number of thresholds that the network needs to configure for LP-WUS operation by introducing a common entry/exit condition across different LP-WUS related procedures. While no conclusion was reached, we observed that companies were considering the following two options:

- Option 1. merge LPWUS monitoring and serving cell offloading

- Option 2. merge LPWUS monitoring and Rel-19 RRM relaxation

Since the serving cell offloading typlically uses a higher threshold than R19 RRM measurement relaxaiton, if all three procedures - LP-WUS monitoring, serving cell offloading, and R19 RRM relaxation- are configured under option 1, the UE would be unable to monitor LP-WUS while the R19 RRM relaxation.

However, even if the serving cell quality is not sufficient to perform the RRM offloading to LR, there is no reason to prevent the UE from monitoring LP-WUS if the LP-WUS reception quality remains reliably high during RRM relaxation. Therefore, if we really need to reduce the number of thresholds for LP-WUS, we prefer to go with option 2.

That said, the exit condition for R18 RRC relaxation is still FFS. If it is eventually defined as ‘not fulfilling the entry condition’, this alone could help reduce the threshold complexity. Therefore, we propose supporting a unified entry condition for LP-WUS monitoring and R19 RRM relaxation.

Proposal 13	Support a unified entry condition for LP-WUS monitoring and r19 RRM relaxation.

The purpose of this discussion is not to reduce the signaling required for LP-WUS configuration, but rather to reduce the total number of thresholds that the network needs to configure when operating LP-WUS. Therefore, there is no need to consider a separate solution for cases where RRM relaxation is not configured. In other words, if the network configures LP-WUS monitoring and serving cell offloading without enabling RRM relaxation, it is appropriate to define separate entry/exit conditions for those two features.

Proposal 14	Do not unify the entry/exit condition for LP-WUS monitoring and RRM offloading. (I.e. when network configures LP-WUS monitoring and RRM offloading without Rel-19 RRM relaxation, separate entry/exit conditions are configured for those two features.)

Conclusion

Proposal 1	No enhancements of Rel-16 relaxed measurement criteria for UEs supporting RRM offloading.

Proposal 2	Do not consider the scenario where RRM offloading and Rel-16 RRM relaxation are configured together without Rel-19 RRM relaxation.

Proposal 3	If Rel-19 RRM relaxation and Rel-16 RRM relaxation are configured together, Rel-19 UEs performs Rel-19 RRM relaxation only.

Proposal 4	Specify that the threshold for RRM offloading to LR should always be set higher than S-IntraSearchP in the field description.

Proposal 5	For uplink carrier selection between NUL and SUL, choose one of the following two options:

- Option 1. Introduce a LR threshold, e.g. rsrp-ThresholdLP-SS-SUL, that allows UE to select the uplink carrier for random access or SDT based on LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of the random access or SDT.

Proposal 6	For initiation of SDT, choose one of the following two options:

- Option 1. Introduce LR thresholds, e.g. sdt-LR-RSRP-Threshold, cg-SDT-LR-RSRP-ThresholdLP-SS, and mt-SDT-LR-RSRP-Threshold, that allows UE to determine whether to initiate SDT based on the LR measurements.

- Option 2. UE stops RRM offloading and starts serving cell measurement using MR upon initiation of SDT.

Proposal 7	For UEs supporting Rel-18 multicast reception in RRC_INACTIVE and the RRM offloading, choose one of the following two options:

- Option 1. Introduce LR threshold, e.g. ThresholdMBS-LR, that allows UEs receiving multicast in RRC_INACTIVE to determine whether to perform RRC resume based on LR measurements.

- Option 2. UEs receiving MBS session in RRC_IDLE/INACTVEI do not perform the RRM offloading.

Proposal 8	If EMR is configured for the serving frequency, the UE does not perform the RRM offloading to LR.

Proposal 9	UE does not perform Rel-19 relaxed measurement for frequencies for which EMR is configured.

Proposal 10	Wait until RAN4 completes its discussion on RRM relaxation for high priority frequency in case 3.

Proposal 11	No separate exit condition other than ‘not fulfilling the entry condition’ for Rel-19 RRM relaxation.

Proposal 12	Do not consider the Low mobility criteria for Rel-19 RRM offloading/relaxation.

Proposal 13	Support a unified entry condition for LP-WUS monitoring and r19 RRM relaxation.

Proposal 14	Do not unify the entry/exit condition for LP-WUS monitoring and RRM offloading. (I.e. when network configures LP-WUS monitoring and RRM offloading without Rel-19 RRM relaxation, separate entry/exit conditions are configured for those two features.)