3GPP TSG-RAN WG2 Meeting #130	R2-2503848

Malta, May, 19th- 23rd 2025

Agenda item:	8.1.2.2

Title:	Continuous Discussion on LCM for UE side Model

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#129bis, following agreements were reached[1]

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via otherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

Further consideration is provided in this contribution.

Discussion

  - On applicability reporting

In RAN2#129bis, it was agree following agreement:

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

As to how to capture the cause value, there are two distinct approaches.

Approach 1: To provide a highly abstracted cause value which indicates whether the functionality is inapplicable temporarily or not. If UE indicates this cause value, the network can expect the functionality becomes applicable later.

The reason that cause the temporary inapplicability may include:

- The applicable server is not available locally, and UE is trying to install it. Note that having model in UE server does not mean the UE has memory size to install it. If UE cannot install it, the network does not need to wait for it becomes applicable.

- Other reasons, e.g. due to UE internal implementation reason.

So in this approach, only one cause value is enough:

- Functionality is inapplicable temporarily.

If the cause value is absent, it means the functionality is inapplicable for other reasons(not temporary reasons).

Approach 2: compared to approach 1, this approach provides more detailed cause values.

The specific cause values can be:

  - NW side additional condition not applicable

  - UE side additional condition not applicable

  - Lack of hardware capability

  - Waiting to fetch applicable model

  - Other cause values.

If the UE indicates one of first 3 cause values, it means the functionality is not applicable and there is applicable model in either UE or UE server. The fourth cause value means there is applicable model but it is not available in UE locally. UE is trying to fetch and install it, i.e. the functionality is inapplicable temporarily.

We understand both approach can work, i.e. in the approach 2, network can tell which cause value means the functionality is inapplicable temporarily.

Observation 1: Both approaches of highly abstracted cause value and more specific cause values can indicate whether the functionality is inapplicable temporarily, which is useful for network to determine following behavior. The difference between the two approaches is the information provided by the cause value.

Considering the more specific cause values may be utilized by network for other processing in the future, approach 2 is more future proof thus is more preferred.

Proposal 1: Following causes can be defined as inapplicability cause:

  - NW side additional condition not applicable

  - UE side additional condition not applicable

  - Lack of hardware capability

  - Waiting to fetch applicable model

One open issue in the post email discussion is related to how to enable applicability reporting for a specific functionality[2]:

Issue description: It is not yet clarified what the content (if any) of the UAI configuration should be, to enable the UE to report applicability in UAI, e.g. applicability updates/changes as agreed for option A. For instance, this configuration could be just a flag or could contain further information. The issue is captured as an editor’s note in the running CR, clause 6.3.4.

Related RAN2 agreements are copied as following:

In RAN2#127, it was agreed UE is allowed to do UAI reporting via otherConfig in step 3:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via otherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

In RAN2#129, it was agreed UAI can be sent to update applicability for a full inference configuration.

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

However, it is not crystal clear for us whether the information of a specific functionality must be configured in otherConfig to enable applicability reporting via UAI for that functionality.

Observation 2: It was agreed UE is allowed to do UAI reporting via otherConfig and UAI can be sent to update applicability. However, it is no crystal clear whether the information of a specific functionality must be configured in otherConfig to enable applicability reporting via UAI for that functionality.

There are two different options of understandings regarding this issue.

Option 1: As long as the inference configuration is configured for a functionality and UE is allowed to report change of applicability (via a flag included in otherConfig for all configured functionalities), the UE can report the change of applicability of the functionality via UAI. Thus there is no need to add additional information in otherConfig for a specific functionality.

Option 2: One functionality is enabled to report applicability via UAI only if its information is also included in otherConfig, even if full inference configuration is already configured to the UE. Thus if network want to receive change of applicability of a functionality, it shall include the information of the functionality in otherConfig. The information of the functionality can be the CSI-ReportConfigId of the full inference configuration.

The advantage of option 2 is that the configuration of inference configuration and enabling of applicability reporting via UAI is decoupled.

With option 2, when network configures inference configuration for a functionality, the UE includes initial applicability report in RRCReconfigurationComplete message. However, whether reporting the change of applicability for the functionality depends on whether it is also configured in the otherConfig.

With this kind of approach, the network has more flexibility in configuring which functionality is allowed to do applicability reporting via UAI, i.e. not all functionalities need to report the change of applicability via UAI. From network point of view, performance monitoring can also be used to determine whether to deconfigure a functionality.

Based on the discussion provided above, we propose:

Proposal 2: RAN2 to decide one of following options on how to enable applicability reporting via UAI for a functionality.

Option 1:Applicability reporting via UAI is enabled for a functionality as long as the full inference configuration is configured for the functionality.

Option 2: Applicability reporting via UAI is enabled for a functionality only if it is also configured in otherConfig, e.g. to include the associated CSI-ReportConfigId in otherConfig.

In current RRC running CR, it seems the applicability of all configured inference configuration are included in every RRCReconfigurationComplete message. The relevant part of the running CR is copied below:

#### 5.3.5.3 Reception of an RRCReconfiguration by the UE /// skipped part 2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6: 3> include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration; 2> for each serving cell configured with at least one reportConfigId associated to a CSI-ReportConfig including a configuration for measurement predictions: 3> include an entry in the applicabilityReportList and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including a configuration for measurement predictions: 5> include an entry in the applicabilityReportConfigIdLit and set the content as follows: 6> set the applicabilityReportConfigId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration for measurement predictions corresponding to the applicabilityReportConfigId; 6> if the applicabilityStatus is set to inapplicable: 7> set the inapplicabilityCause for the configuration for measurement predictions to the cause of inapplicability; /// skipped part

There are two issues related to the applicability reporting in RRCReconfigurationComplete message:

Issue 1: whether applicability reporting is included in every RRCReconfigurationComplete message, no matter there is inference configuration included in corresponding RRCReconfiguration message.

Issue 2: whether the applicability of all configured inference configuration shall be mandatory included in each applicability reporting.

In our understanding, it is unnecessary to include applicability reporting of all configured inference configuration in every RRCReconfigurationComplete message.

Observation 3: It is unnecessary to include applicability report of all configured inference configuration in every RRCReconfigurationComplete message.

Regarding what can/shall be included in the applicability reporting in a RRCReconfigurationComplete message, there are following options.

Option 1: To only include applicability of the inference configuration that are configured/reconfigured in this RRCReconfiguration message, if any.

Option 2: To include applicability of the inference configuration that are configured/reconfigured in this RRCReconfiguration message , if any. In addition, to include the change of applicability of the inference configuration that are previously configured in previous RRCReconfiguration message, if any.

We think option 1 is baseline according to following RAN2 agreement reached in RAN2#129:

3. Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

As to option 2, it depends on whether it is allowed to include the update of applicability of previously configured inference configuration.

One relevant use case is the previous configured functionality may become inapplicable because the CPU/memory are preempted for the new configured inference configuration. In this case, the UE can report inapplicability of the previous configured inference configuration and the applicability of the new configured inference configuration in the same RRCReconfigurationComplete message. Otherwise, an additional UAI will be triggered to report inapplicability of the previous configured inference configuration

Based on above discussion, we have following proposal:

Proposal 3: RAN2 to decide one of following options regarding the applicability reporting in RRCReconfiguration message for applicability reporting option A.

Option 1: To only include applicability report of the inference configuration that are configured/reconfigured in this RRCReconfiguration message, if any.

Option 2: To include applicability report of the inference configuration that are configured/reconfigured in this RRCReconfiguration message, if any. In addition, to include the change of applicability of the inference configuration that are configured in previous RRCReconfiguration message, if any.

  - On applicability report and activation during RRC resume

As part of serving cell configuration, CSI-ReportConfig that is used as inference configuration will be stored in Inactive AS context when UE is released to RRC_INACTIVE state. During RRC resume procedure, the stored RRC configuration will be restored. More specifically, the UE restores RRC configuration including MCG/SCG configuration upon receiving RRC resume message. As a result, the configured inference configuration will also be restored.

Observation 4: A configured inference configuration is stored as part of Inactive AS context when UE is released to RRC_INACTVIE state, and is restored upon UE receives RRCResume message.

At the same time, the RRCResume message can include MCG/SCG configuration which contains serving cell configurations which is same as RRCReconfiguration message. One difference between RRCReconfiguration and RRCResume message is that otherConfig cannot be included in RRCResume message for MCG. Thus the question is whether network can configure new inference configuration in the RRCResume message.

If yes, the UE can report the initial applicability of the new configured inference configuration in RRCResumeComplete message. And same as RRCReconfiguration procedure, the UE can activate the periodic inference configuration if UE report applicability in RRCResumeComplete message.

Proposal 4: RAN2 to agree that new inference configuration can be configured in RRCResume message and initial applicability report can be included in RRCResumeComplete message.

Till now, it is unclear how UE will treat the inference configuration restored from Inactive AS context during and RRC resume procedure.

When UE resumes to a serving cell, the restored inference configuration may be applicable or inapplicable. However, at least before RRCResumeComplete message, UE cannot report applicability of restored inference configuration to network because of the limited size of RRCResumeRequest message.Thus the applicability of restored inference configuration is unclear for the network.

Observation 5: At least before receiving RRCResumeComplete message, the network cannot get aware of the applicability of restored inference configuration.

In such situation, how to treat the restored inference configuration, especially the periodic CSI report configuration will have impact on whether network needs to prepared network side resource for the inference configuration. For example, if UE activate the periodic CSI report configuration during RRC resume procedure, the network will prepare the resource before transmitting RRC resume message.

Observation 6: How UE treat restored inference configuration during RRC resume procedure has impact on whether network needs to prepare network resources for the restored inference configuration before transmitting RRCResume message.

Regarding the network configuration and UE behavior related to restored inference configuration, we have following options:

Option 1: UE autonomously releases the restored inference configuration, or network is expected to deconfigure the restored inference configuration via RRCResume message.

With this option, the network does not need to prepare any resource for the restored inference configuration. And the network can still configure new inference configuration via RRCResume or RRCReconfiguration message. Obviously this option 1 is not efficient in terms of signaling overhead.

Option 2: UE reports applicability of the restored inference configuration in RRCResumeComplete message, and keeps the restored inference configuration as deactivated no matter it is applicable or inapplicable. Network is expected to release or reconfigure the inference configuration based on the applicability report.

With option 2, network can decide whether to release or to reconfigure the restored inference configuration according to the applicability report. Then the risk that UE automatically activate an inapplicable inference configuration can be avoided. Compared to option 1, the advantage of option 2 is the restored inference configuration is not always released. Thus signaling overhead can be reduced.

Option 3: UE reports applicability of the restored inference configuration in RRCResumeComplete message. If the inference configuration is periodic CSI report configuration and is applicable, the UE activates it automatically after transmitting RRCResumeComplete message. Otherwise, UE keeps the (inapplicable or aperiodic/semi-persistent) inference configuration in deactivated.

This option 3 is similar to UE automatic activation in step 4 in application report option A. Different to option 2, the network has to prepare the network resource before transmitting RRCResume message in case the restored inference configuration is applicable when UE initiate RRC resume procedure. This is feasible because network knows when to prepare the resource when it receives RRCResumeRequest message.

The advantage of option 3 is the fast activation of applicable periodic inference configuration. Because UE needs not to waiting for another RRCReconfiguration message to reconfiguration it.

Therefore, we propose RAN2 to decide the UE behavior on how to treat the restored inference configuration and take above options into account.

Proposal 5: RAN2 to decide one of following options regarding how to treat the restored inference configuration during RRC resume procedure:

Option 1: UE autonomously releases the restored inference configuration, or network is expected to deconfigure the restored inference configuration via RRCResume message.

Option 2: UE reports applicability of the restored inference configuration in RRCResumeComplete message, and keeps the restored inference configuration as deactivated no matter it is applicable or inapplicable. Network is expected to release or reconfigure the inference configuration based on the applicability report.

Option 3: UE reports applicability of the restored inference configuration in RRCResumeComplete message. If the inference configuration is periodic CSI report configuration and is applicable, the UE activates it automatically after transmitting RRCResumeComplete message. Otherwise, UE keeps the inference configuration in deactivated.

  - On data collection request

Agreements on data collection configuration

The UE can request measurement configuration for data collection of AI/ML based beam management.The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end.

Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request

According to the agreements as shown in above, the legacy UAI framework would be applied for UE to request the measurement configuration of UE side data collection.

Observation 7: The UAI framework is agreeable to be used for UE to request the measurement configuration of the UE side data collection.

As agreed in the previous meeting, before UE providing the preference, NW shall provide a list of candidate configurations for UE to choose the preferred ones, in our understanding, the candidate configuration can be contained in the otherConfig for triggering the UAI.

Proposal 6: The candidate configuration list is contained in the OtherConfig of the RRCReconfiguration to trigger UE to report the UAI for requesting the measurement configuration.

In our understanding , the candidate configurations shall be at least one of the following formation:

- Formation 1: UE side data collection configuration list, e.g. for beam management case, each UE side data collection configuration includes CSI-ResourceConfig of set A and set B, and corresponding associated Id(s)

- Formation 2: Inference configuration list, e.g. for beam management case, CSI-ReportConfig or CSI-ReportConfigId.

For formation 1, it is a direct way to configure some CSI-ResourceConfig into otherConfig for NW to freely providing the candidate configuration list.

For formation 2, using the inference configuration is also workable, in this formation, the intention is mainly for those CSI-ReportConfig that is indicated as inapplicable in the applicability reporting , in some case, those inapplicability CSI-ReportConfig may be due to lack of data for training. Compare to formation 1, the gain is the reduced signaling overhead when using the CSI-ReportConfigId to indicate the inference configuration.

In our understanding, both formations are workable, it is proposed that:

Proposal 7: RAN2 is asked to decide which of the following candidate configuration list can be contained in the otherConfig for triggering UAI

- UE side data collection configuration list where each configuration contains CSI-ResourceConfig for both Set A and Set B, and corresponding associated Id(s)

- Inference configuration list where each inference configuration can be a CSI-ReportConfig or a CSI-ReportConfigId.

After UE receiving the RRCReconfiguration where the OtherConfig containing the candidate configuration list, UE may send the UAI to the NW, and according to the agreements, it is said that UE shall report the start/stop indication on UE side data collection in the UAI, but in our understanding, there is no need for NW to know whether UE to start or stop UE side data collection since this configuration is just for UE side data collection and no logged data report to NW is expected..

Proposal 8: The start/stop indication on UE side data collection in the UAI is not needed.

As for the preferred configuration provided in the UAI, it can be represent by the configuration index that is contained in the candidate configuration list.

Proposal 9: One or more configuration indexes are contained in the UAI to indicate the UE preferred configuration for UE side data collection.

Conclusion

Observation 1: Both approaches of highly abstracted cause value and more specific cause values can indicate whether the functionality is inapplicable temporarily, which is useful for network to determine following behavior. The difference between the two approaches is the information provided by the cause value.

Observation 2: It was agreed UE is allowed to do UAI reporting via otherConfig and UAI can be sent to update applicability. However, it is no crystal clear whether the information of a specific functionality must be configured in otherConfig to enable applicability reporting via UAI for that functionality.

Observation 3: It is unnecessary to include applicability report of all configured inference configuration in every RRCReconfigurationComplete message.

Observation 4: A configured inference configuration is stored as part of Inactive AS context when UE is released to RRC_INACTVIE state, and is restored upon UE receives RRCResume message.

Observation 5: At least before receiving RRCResumeComplete message, the network cannot get aware of the applicability of restored inference configuration.

Observation 6: How UE treat restored inference configuration during RRC resume procedure has impact on whether network needs to prepare network resources for the restored inference configuration before transmitting RRCResume message.

Observation 7: The UAI framework is agreeable to be used for UE to request the measurement configuration of the UE side data collection.

Proposal 1: Following causes can be defined as inapplicability cause:

  - NW side additional condition not applicable

  - UE side additional condition not applicable

  - Lack of hardware capability

  - Applicable model is not available locally

Proposal 2: RAN2 to decide one of following options on how to enable applicability reporting via UAI for a functionality.

Option 1:Applicability reporting via UAI is enabled for a functionality as long as the full inference configuration is configured for the functionality.

Option 2: Applicability reporting via UAI is enabled for a functionality only if it is also configured in otherConfig, e.g. to include the associated CSI-ReportConfigId in otherConfig.

Proposal 3: RAN2 to decide one of following options regarding the applicability reporting in RRCReconfiguration message for applicability reporting option A.

Option 1: To only include applicability report of the inference configuration that are configured/reconfigured in this RRCReconfiguration message, if any.

Option 2: To include applicability report of the inference configuration that are configured/reconfigured in this RRCReconfiguration message, if any. In addition, to include the change of applicability of the inference configuration that are configured in previous RRCReconfiguration message, if any.

Proposal 4: RAN2 to agree that new inference configuration can be configured in RRCResume message and initial applicability report can be included in RRCResumeComplete message.

Proposal 5: RAN2 to decide one of following options regarding how to treat the restored inference configuration during RRC resume procedure:

Option 1: UE autonomously releases the restored inference configuration, or network is expected to deconfigure the restored inference configuration via RRCResume message.

Option 2: UE reports applicability of the restored inference configuration in RRCResumeComplete message, and keeps the restored inference configuration as deactivated no matter it is applicable or inapplicable. Network is expected to release or reconfigure the inference configuration based on the applicability report.

Option 3: UE reports applicability of the restored inference configuration in RRCResumeComplete message. If the inference configuration is periodic CSI report configuration and is applicable, the UE activates it automatically after transmitting RRCResumeComplete message. Otherwise, UE keeps the inference configuration in deactivated.

Proposal 6: The candidate configuration list is contained in the OtherConfig of the RRCReconfiguration to trigger UE to report the UAI for requesting the measurement configuration.

Proposal 7: RAN2 is asked to decide which of the following candidate configuration list can be contained in the otherConfig for triggering UAI

- UE side data collection configuration list where each configuration contains CSI-ResourceConfig for both Set A and Set B, and corresponding associated Id(s)

- Inference configuration list where each inference configuration can be a CSI-ReportConfig or a CSI-ReportConfigId.

Proposal 8: The start/stop indication on UE side data collection in the UAI is not needed.

Proposal 9: One or more configuration indexes are contained in the UAI to indicate the UE preferred configuration for UE side data collection.

References

- RAN2#129bis chairman notes

- R2-xxxxxx RRC open issues AIML PHY