3GPP TSG-RAN WG2 Meeting #130	R2-2503857

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.12.2

Source:	Nokia

Title:	Discussion on asymmetric DL sTRP/UL mTRP

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID RP-242394.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Based on the open issue summary of [Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson) and [Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)), the following  proposals have been agreed to discuss via contributions in RAN2#130 on asymmetric DL sTRP/UL mTRP:

RRC:

- How to capture PL offset behaviour in RRC for handover and resume case

MAC:

- Discuss the scenario asymmetric DL sTRL/UL mTRP with no PL offset (i.e. the UL TRP may transmit SSB) based on RAN1/RAN4 LS (R1-2503091) and spec. impact regarding 2TA for sDCI mTRP.

- Discuss to support sDCI mTRP 2TA for intra-DU LTM, in the same way as Rel-18 mDCI mTRP 2TA, i.e., the TA included in the LTM cell switch command is applied to the TAG configured for the indicated TCI state in LTM cell switch command.

- Regarding PL offset in LTM, discuss

i) how to apply the PL offset in intra-DU LTM when PL offset is provided in LTM configuration (e.g., the PL offset for the indicated TCI state in the LTM cell switch command is applied for the first PUSCH transmission in RACH-less LTM cell switch);

ii) whether to support PL offset in intra-DU LTM when PL offset is not provided in LTM configuration for the target cell.

- Discuss whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP.

In this document, we present Nokia’s views regarding the open issues listed.

# 2	Discussion

## 2.1	Handling of PL offset during RRC resume and handover

Last meeting RAN2 agreed that (from RAN2 point of view) the UE shall apply the latest PL offset value received in RRC or MAC CE. However, this agreement could be problematic/ambiguous when applied to the scenario where a UE configured with PL offset(s) is sent to RRC inactive and then resumes its connection.

During the resumption procedure, the UE first applies default L1 parameters and restores the last RRC configuration. Resuming the connection using the last RRC-configured PL offset is problematic for two reasons:

1)	UE does not restore configurations received from lower layers. The restored PL offset from the RRC configuration could be out-of-date relative to subsequent MAC CE-indicated PL offset(s) that had been received prior to going to inactive state.

2)	Even in cases where the network never sent an updated PL offset via MAC CE, the UE could have moved elsewhere in the cell while inactive and the old (RRC-configured) PL offset configuration would still be out-of-date.

Observation 1: Following resumption from inactive state, the applicable PL offset configuration could be ambiguous or out-of-date.

One or two companies suggested that PL offsets received from MAC CE can be sent “upward” to RRC, presumably so they can be stored in the RRC context, but this is not typical behaviour between L2-L3 and could cause other unforeseen issues.

The much simpler solution is to ensure that PL offset is provided by the network during the RRC resume procedure. This could be done via the masterCellGroupConfig configuration within the RRCResume message, or even by adding a new IE within RRCResume to directly update the PL offsets (in case there is no other reconfiguration needed for the cell group).

In the former case, i.e. updating the PL offsets during RRC resume procedure based on masterCellGroupConfig configuration, at least some conditional presence would need to be added to the pathlossOffset-r19 field to state that it is mandatory during RRC resume. In the past, similar conditional statements were captured for the RRC resume case, e.g. for SyncAndCellAdd and MRB-Initialization.

Proposal 1: Network should indicate the applicable PL offset configuration to the UE during the RRC resume procedure. RAN2 to consider the following options:

-	Option 1: Make pathlossOffset-r19 conditionally mandatory during RRC resume so network includes masterCellGroupConfig with pathlossOffset-r19 within the RRCResume message.

-	Option 2: Add a new IE within RRCResume to directly update the PL offsets without requiring masterCellGroupConfig configuration.

A similar issue was identified in R2-2502834 for the case of inter-gNB handover, where the target gNB would be informed of the last RRC-configured PL offset from the source gNB but would be unaware of any subsequent MAC CE-configured PL offsets. This leads to the UE and target gNB having potentially different understandings of the configured PL offsets: e.g. during handover, if the target gNB thought the last RRC-configured PL offset was appropriate, then it might not send an updated configuration (based on delta signalling target gNB would omit the PL offset and  expect UE to keep using the RRC configuration); however, UE would instead keep using the MAC CE configured PL offset.

To avoid ambiguity, there could similarly be a requirement to include the PL offset during handover (i.e. when masterCellGroupConfig includes ReconfigurationWithSync). As with Option 1 above, this could be handled by a conditional presence field.

Observation 2: During handover, the UE and target gNB could assume that different PL offsets are applicable.

Note that our understanding is that from RAN1 perspective, the applicability of PL offset has only been considered in the intra-cell scenario, i.e. not during handover. However, as this enhancement would be very minor, it seems reasonable to discuss it in RAN2.

Proposal 2: RAN2 to discuss how to handle PL offset during handover, e.g. by making pathlossOffset-r19 conditionally mandatory when masterCellGroupConfig includes ReconfigurationWithSync.

## 2.2	Support for asymmetric TRP during LTM procedures

Another issue raised during offline discussion was whether to support PL offset during LTM cell switch procedure involving asymmetric TRP candidate cells.

LTM candidate TCI states are configured by RRC (LTM-Candidate > LTM-TCI-Info), which supports configuration based on the unified TCI framework (joint/separate DL/UL TCI states) and includes the PL RS configuration to be used. LTM-TCI-Info IE was defined in an extensible manner, so in theory, if RAN2 sees the need, it could support addition of PL offset configuration for the candidate TCI states associated with asymmetric TRP cells.

On the other hand, the LTM Cell Switch Command MAC CE (as well as the Candidate Cell TCI States Activation/Deactivation MAC CE), which apply the TCI state(s) configured by LTM-TCI-Info, do not have spare fields to accommodate updating the PL offset configuration simultaneously with the MAC CE commands.

In our view, it would be an overoptimization to introduce new MAC CEs to support simultaneously updating the PL offset with the LTM procedures. This means that if LTM candidate cell activation and LTM cell switching are to be supported for asymmetric TRP cells, the use would be limited to the case where the PL offset is pre-configured by RRC (assuming PL offset can be added to LTM-TCI-Info configuration). Depending on the time delta between the RRC configuration and MAC CE indication for candidate cell switch/activation, the configured PL offset may or may not be valid anymore, so the overall benefit of supporting PL offset in this scenario is quite questionable.

Observation 3: It is complicated to enhance the LTM cell switch and candidate cell activation MAC procedures for asymmetric TRP; limiting the candidate LTM TCI state enhancements to RRC is feasible, but has questionable benefits if MAC cannot also be enhanced.

Proposal 3: RAN2 does not specify enhancements to support PL offset for asymmetric TRP during LTM cell switch or candidate cell activation procedures.

## 2.3	TCI state activation MAC CE for asymmetric DL sTRP/UL mTRP

RAN1 has made a conclusion for asymmetric DL sTRP/UL mTRP.

Conclusion RAN1#117 For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

From our perspective, the Release-18 activation/deactivation MAC CE (shown in Figure 1 below) can support the combination agreed upon by RAN1. As a result, the legacy MAC CE format can be reused in the asymmetric DL sTRP/UL mTRP scenario. This reuse has no impact on the joint TCI mode. However, in the separate TCI mode, since there is no DL TCI state associated with the UL-only TRP, the corresponding Fi,j field should be set to 0, and the UE should disregard this field.

Figure 1: Release-18 TCI State activation/deactivation MAC Control Element

Proposal 4: Reuse the Release-18 TCI State activation/deactivation MAC Control Element (CE) to support both joint and separate TCI modes in the asymmetric mTRP scenario. In the case of separate TCI mode, where a UL-only TRP is present without a corresponding DL TCI State, the associated Fi,j field in the MAC CE shall be set to 0, and the UE shall ignore this field.

# 3	Conclusion

Observation 1: Following resumption from inactive state, the applicable PL offset configuration could be ambiguous or out-of-date.

Observation 2: During handover, the UE and target gNB could assume that different PL offsets are applicable.

Observation 3: It is complicated to enhance the LTM cell switch and candidate cell activation MAC procedures for asymmetric TRP; limiting the candidate LTM TCI state enhancements to RRC is feasible, but has questionable benefits if MAC cannot also be enhanced.

Proposal 1: Network should indicate the applicable PL offset configuration to the UE during the RRC resume procedure. RAN2 to consider the following options:

-	Option 1: Make pathlossOffset-r19 conditionally mandatory during RRC resume so network includes masterCellGroupConfig with pathlossOffset-r19 within the RRCResume message.

-	Option 2: Add a new IE within RRCResume to directly update the PL offsets without requiring masterCellGroupConfig configuration.

Proposal 2: RAN2 to discuss how to handle PL offset during handover, e.g. by making pathlossOffset-r19 conditionally mandatory when masterCellGroupConfig includes ReconfigurationWithSync.

Proposal 3: RAN2 does not specify enhancements to support PL offset for asymmetric TRP during LTM cell switch or candidate cell activation procedures.

Proposal 4: Reuse the Release-18 TCI State activation/deactivation MAC Control Element (CE) to support both joint and separate TCI modes in the asymmetric mTRP scenario. In the case of separate TCI mode, where a UL-only TRP is present without a corresponding DL TCI State, the associated Fi,j field in the MAC CE shall be set to 0, and the UE shall ignore this field.