3GPP TSG-RAN WG2 #130	R2-2503858

Malta, May 19th – 23rd, 2025

Agenda item:	8.6.2

Source: 	Ericsson

Title:  	Inter-CU LTM and Fast Recovery

Document for:	Discussion and Decision

# Introduction

During the previous meeting there has been some outstanding issues on security and fast recovery. This contribution proposes our view on how to proceed.

# Discussion

## Terminology for LTM and CLTM

According to the current running CR, there is still confusion on what terminology to use when describing things for LTM and CLTM. Since LTM and CLTM have a lot of similarities and share pretty much the same ASN.1 signaling and procedural text, the easiest way would be to use the term “LTM” by default for anything which is common for LTM and CLTM. In this case, the term “CLTM” needs to be used only for something that applies specifically for CLTM (and not to LTM), such as for example the conditional execution conditions.

Use the term “LTM” for anything which is common for LTM and CLTM while the term “CLTM” is only used for something which apply only to CLTM.

## Indicating sk-counter to the MN

In current procedural text of the RRC running CR for inter-CU LTM and CLTM, when the UE selects an sk-counter, this is indicated to the MN via the RRCReconfigurationComplete message.

2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6: 3> if this RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup or via SRB3: 4> include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration; 3> else if this RRCReconfiguration message was received via SRB1 but not within the nr-SCG within mrdc-SecondaryCellGroup: 4> if a new sk-Counter value has been selected due to the LTM cell switch execution procedure: 5> include selectedSK-Counter and set its value to the selected sk-Counter value; Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

The problem with the current text is that, in case of inter-SN LTM, there is actually no MN RRCReconfigurationComplete message which is sent to the MN, as the UE will just apply the SN RRCReconfiguration message which is related to the SN. In such a case, the SN RRCReconfigurationComplete message will just simply be received by the MN via the ULInformationTransferMRDC, and the MN will simply forward it to the SN.

In case of inter-SN LTM, the SN RRCReconfigurationComplete message will just simply be received by the MN via the ULInformationTransferMRDC, and the MN will simply forward it to the SN.

Therefore, current text is actually wrong but there is also the issue on how to inform the MN about which sk-counter have been selected by the UE in case of an inter-SN LTM. According to this, there are two possible options to solve this issue:

- Option 1: When an inter-SN LTM is performed, the selected sk-counter is always included in the SN RRCReconfigurationComplete message and the SN forward the sk-counter to the MN.

- Option 2: The selected sk-counter is included within the ULInformationTransferMRDC when an inter-SN LTM is executed.

RAN2 to discuss how the UE informs the MN about the selected sk-counter in case of inter-SN LTM.

  - The selected sk-counter is always included in the SN RRCReconfigurationComplete message and the SN forward the sk-counter to the MN.

  - The selected sk-counter is included within the ULInformationTransferMRDC when an inter-SN LTM is executed.

## Which sk-counter to use for inter-MN LTM

Another open issue is related to which sk-counter to use in case of an inter-MN LTM. As typically for LTM, also for the case of inter-MN LTM we need to support subsequent LTM.

Subsequent LTM needs to be supported also for the case of inter-MN LTM.

Let’s consider the following scenario (also shown in Figure 1):

- UE is operating in a cell which is also an inter-MN LTM candidate configuration 1 with an SCG (no LTM on the SCG). In this case the inter-MN LTM candidate configuration has an sk-counter to be used when the key is refreshed.

- UE goes to an inter-MN LTM candidate configuration 2, which decides to release the SCG. The security is refreshed by no sk-counter from LTM candidate configuration 2 is used (or even present)

- UE goes back to the inter-MN LTM candidate configuration 1 which has an SCG and thus there will be an SCG addition and, since the security is refreshed, the UE will use the sk-counter which is included within the LTM candidate configuration 1

- UE goes to an inter-MN LTM candidate configuration 3, which decides to release the SCG. The security is refreshed by no sk-counter from LTM candidate cell 2 is used (or even present).

- UE goes back to the inter-MN LTM candidate configuration 1 which has an SCG, an sk-counter is needed, but this was already used during the second LTM cell switch.

Figure 1. Scenario considered

According to this scenario shown in Figure 1, the main problem is that when the fourth inter-MN LTM cell switch happen, the sk-counter within the LTM candidate configuration 1 has been already used and thus the UE cannot derive any secondary key. In this case, one may say that after the second LTM cell switch the inter-MN LTM candidate configuration 1 can be re-configured with a new sk-counter, but this will violate the principle of subsequent LTM. Therefore, according to this, the easiest solution would be to re-use the list of sk-counter even for the case of inter-MN LTM.

The list of sk-counter is used also for the case of inter-MN LTM.

## Fast Recovery

Fast recovery was discussed in RAN2#129bis and we took the following agreement:

RAN2#129bis agreements: At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed. NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

Based on the open issues in the running CR, but also from discussions and agreements from last meeting, there is an unclear behavior during the fast recovery procedure when a cell switch to another CU which requires a security key update. Consider Figure 2 below as an illustration of the problem.

Figure 2; Fast recovery example

In the above example, a Rel-19 UE is in cell111, with five LTM candidates configured (cell 112, cell131, cell211, cell221 and cell 311). The corresponding IDs for how the UE should treat clearing of buffers and security updates are configured as described by Rel-18 ID and Rel-19 ID.

The fast recovery can happen either due to a problem before or after the LTM Cell Switch command, and depending on when, the table below describes the outcome according to previous agreements.

Table 1; Fast Recovery examples

|  | Selected cell for fast recovery | Result | Comment |
| --- | --- | --- | --- |
| Before LTM Cell Switch Command | Cell112 | Trivial, UE can select | Rel-19 ID same Rel-18 ID same |
|  | Cell131 | Trivial, UE can select | Rel-19 ID same Rel-18 ID different |
|  | Cell211 | UE can’t select | No NCC has been distributed yet |
|  | Cell221 | UE can’t select | No NCC has been distributed yet |
|  | Cell 311 | UE can’t select | No NCC has been distributed yet |
| After LTM Cell Switch Command to cell211 with NCC for gNB2 | Cell112 | FFS according to editor’s note | NCC for gNB2 already applied |
|  | Cell131 | FFS according to editor’s note | NCC for gNB2 already applied |
|  | Cell211 | N/A | Target, but failed in cell switch |
|  | Cell221 | Trivial, UE can select | Rel-19 ID same as the intended target cell. |
|  | Cell 311 | FFS according to agreements | No NCC for gNB3 available |

Based on Table 1, let’s have a look at the current FFS in the running CR:

Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration. Rapporteur comment: It is still not crystal clear how the fast recovery procedure would work, and especially when the UE should revert back to the previous PCell configuration. We suggest to discuss this based on companies’ contribution.

As shown in Table 1, the fast recovery for the case of inter-CU LTM is quite tricky because there are several cases that needs to be considered. According to this, it is evident that it is difficult to have a unified solution to cover all these cases and, since what we are trying to handle is a failure scenario anyway, probably RAN2 should not strive to over-optimize this.

The support of fast recovery for the case of inter-CU LTM is challenging as there are several cases that needs to be considered.

Because of this, it would probably be easiest to agree on the simplest solution which is to re-use what we have in Rel-18 LTM. This would mean that fast recovery is only possible in case of RLF and LTM cell switch failure, when the UE selects a cell which Rel-19 ID is the same as the one of the source PCell. This case requires minimum standardization effort and make also the whole feature easier to implement. Thus, we propose:

RAN2 to revert the previous agreement and to agree that fast recovery is only possible in case of RLF and LTM cell switch failure, when the UE selects a cell which Rel-19 ID is the same as the one of the source PCell.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In case of inter-SN LTM, the SN RRCReconfigurationComplete message will just simply be received by the MN via the ULInformationTransferMRDC, and the MN will simply forward it to the SN.

Observation 2	Subsequent LTM needs to be supported also for the case of inter-MN LTM.

Observation 3	The support of fast recovery for the case of inter-CU LTM is challenging as there are several cases that needs to be considered.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the term “LTM” for anything which is common for LTM and CLTM while the term “CLTM” is only used for something which apply only to CLTM.

Proposal 2	RAN2 to discuss how the UE informs the MN about the selected sk-counter in case of inter-SN LTM.

a.	The selected sk-counter is always included in the SN RRCReconfigurationComplete message and the SN forward the sk-counter to the MN.

b.	The selected sk-counter is included within the ULInformationTransferMRDC when an inter-SN LTM is executed.

Proposal 3	The list of sk-counter is used also for the case of inter-MN LTM.

Proposal 4	RAN2 to revert the previous agreement and to agree that fast recovery is only possible in case of RLF and LTM cell switch failure, when the UE selects a cell which Rel-19 ID is the same as the one of the source PCell.

# References

R2-25xxxxx- Running RRC CR for L1 measurements, vivo, RAN2#130, May 2025.