3GPP TSG-RAN WG2 Meeting #130 	R2-2503861

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.3

Source:	Panasonic

Title:	Discussion on truncated L1 measurement reporting

Document for:	Discussion

# Introduction

In previous meeting, the following has been agreed for L1-RSRP measurement reporting.

For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

In this paper, we discuss the FFS - how to handle truncated MAC CE.

# Discussion

For the differential L1-RSRP reporting, it is natural that the strongest beam among all the reported beams is taken as the reference beam. This reference beam would be included as the first beam in the report and represented as absolute value, and the rest of beams are reported by differential values. This has been reflected in the first agreement above. The merit is to avoid encoding both negative and positive differential values.

However, this strongest beam might not be the triggered beam (e.g. it does not satisfy the TTT, although RSRP is high). This may create some burden for truncated measurement report. As capture in the second agreement above, for truncated measurement report, UE need to include triggered beams first. Then in previous meeting, some companies commented that with agreement#2, if UL grant is too small to accommodate full report, UE has to “rebuild” a truncated measurement report instead of “truly truncated” the full report. Then the FFS was captured to give more time for companies to think about.

From our understanding, UE need not prepare a measurement report before knowing the UL grant size. In other words, if there is a triggered event, UE should send SR to request a UL grant first. After UE gets UL grant, UE then starts to build the measurement report. Then depending on the TBS in the UL grant, either full report or short (“truncated”) report is built. As a result, the same procedure as described in the above agreement#1 can be applied for full report or truncated MAC CE. This understanding matches the description in the running CR [1], see below yellow highlighted part.

### 5.x.4 Measurement report The purpose of this procedure is to transfer L1 measurement results from the UE to the network. RRC controls the event triggered L1 beam level measurement reporting by configuring the following parameter: - reportInterval: the periodicity of the event-triggered periodic measurement report; - reportAmount: number of measurement reports needs to be transmitted after the event is triggered; - maxNumberOfReportedBeams: number of beams whose measurements can be reported in the event triggered L1 measurement report by MAC CE; - allowReportAnyBeam: whether the UE could report the measurement results for the beams not satisfying the conditions of the events; - reportCurrentBeam: whether the UE could report the measurement result of the current beam; - ltm-CandidateReportConfigId: LTM candidate cell ID for which the UE is required to measure reference signal and perform LTM event evaluation; - candidateSpecificOffset: offset for event condition that is applicable for all the reference signals belonging to the candidate cell with the candidate cell ID ltm-CandidateReportConfigId; - reportQuantity: the report quantity for the CSI report For the event triggered L1 measurement reporting, the MAC entity shall: 1> if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled: 2> if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE; 3> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId for the RS signal triggered the report by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; 3> cancel the the triggered L1 measurement report; 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE; 3> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId for the RS signal triggered the report by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; 2> else: 3> if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured: 4> trigger the SR using the dedicated SR configuration for L1 measurement report; 3> else: 4> trigger the SR using the SR configuration for L1 measurement report.

Proposal 1. For truncated L1 measurement report MAC CE, the best quality beam among the beams included in truncated L1 measurement report MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam.

# Conclusion

In this paper, we have discussed differential L1-RSRP reporting for truncated MR MAC CE. We have the following proposal.

Proposal 1. For truncated L1 measurement report MAC CE, the best quality beam among the beams included in truncated L1 measurement report MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam.

# Reference

[1] MAC running CR for enhanced mobility Ph4, www.3gpp.org / ftp / Email_Discussions / RAN2 / [RAN2#129bis] / [POST129bis][117][MOB] (Vivo)