3GPP TSG-RAN WG2 Meeting #130	R2-2503863

St. Julians, Malta, 19 – 23 May 2025

Agenda item:	8.2.3

Source:	Nokia

Title:	Aspects on random access for AIoT

WID/SID:	Ambient_IoT_solutions - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution handles the open issues within AIoT, in particular;

- Contention free access procedure

- Signalling of msg1 resource(s)

- General signalling of D2R resource(s)

- Procedures for re-access

# 2	Discussion

## 2.1	CF(R)A

In RAN2#129bis it was agreed that for CFRA, the first command message would be the message in which the AS ID would be assigned.

7	For CFRA, command message is used for AS ID assignment

Furthermore, it was discussed that it would be FFS whether RAN2 assumes multiple device scenario, since this may have impact on the multiple device scenario.

Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

The latest WID on AIoT states that RAN2 should aim at designing the paging format in such a way that multiple identifiers can be included, at least for forward compatibility purpose, but we believe that this contradicts the current agreements.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

We understand that clarity may be needed on the current agreement. Our assumption is, that the ‘command’ used for AS ID assignment should neither be the read nor write command, but rather a dedicated AS ID command. As such, assuming this command follows the inventory reply from the device (msg2) without any interruptions, the AS ID may be referenced during any later read/write commands. This would allow multiple UEs to be addressed in a CFRA manner first by the inventory and then later by subsequent read/write commands, without spending unnecessary time waiting for the inventory resources.

Observation 1: The WID states that forward compatibility should be included when designing (at least) paging messages.

Observation 2: The usage of AS ID ensures forward compatible multi-devices paging in CFRA without the need to use the full device ID.

Observation 3: If AS ID is assigned using the read or write command, the later devices in a multi-device CFRA procedure ends up waiting a significant time before the msg1 resources are provided.

Proposal 1: A separate AS ID assignment command shall be adopted to support multiple device scenario. FFS whether this can be multiplexed with a read/write command.

In the paging discussion, the big topic is on whether a device should react to other pagings i.e. different transaction ID before it finalises its own procedure. Since CFRA is a dedicated procedure, we assume that the AIoT-F has a clear intention with the data, and thus we see it may be more urgent/relevant for a device to reply to a dedicated CFRA procedure than a CBRA. In order to ensure this behaviour, we believe that a device should reply to a CFRA procedure, irrespective of whether it has an ongoing CBRA procedure. Such behaviour may also help resolve any potential deadlocks.

Proposal 2: RAN2 to discuss whether CFRA takes precedence over CBRA in any given case i.e. a device may interrupt any ongoing CBRA initiated procedure when a CFRA paging has the device ID. FFS on whether this is applicable during a segmented command.

## 2.2	Msg1 resources

In RAN2#129bis, RAN2 reached agreements on the behaviour w.r.t. allocating at least msg1 resources to the device.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

During the meeting, it was discussed whether the paging provides only the Q value, or specific number of time and frequency resources. Each solution has it’s own drawbacks and advantages, where

- Providing only the Q value;

  - enables freedom in the reader to have msg1 resources with different number of frequency resources. This looks to be a forward compatible solution, where the number of frequency resources could be varied based on the number of readers in the area.

  - However, for device 1’s, the complexity of determining the correct timeslot now comprises of keeping track of all frequency resources within each timeslot, in order to determine the correct timeslot wherein the selected resource is located

- Providing Q value and fixed number of frequency slots per msg1 resource

  - Enables easy implementation in the device, as the timeslot is simply calculated as the random slot number divided by the number of frequency slots per msg1 resource

  - Has less flexibility in more advanced scenarios

We believe that the optimal procedure, also regarding forward compatibility, would be to have the paging indicate only the Q factor and allowing msg1 to indicate the frequency resources. However, given the complexity of such operation, we propose that the paging will carry the Q factor and fixed number of frequency resources per msg1 timeslots.

Proposal 3: The R2D transmission determining the Msg1 resources will indicate at least number of frequency resources per time slot to the device.

In order to enable the reader to indicate contention is resolved and a resource is given to one or multiple devices within a given timeslot, it should be able to indicate to multiple devices within msg2. Allowing multiplexing of multiple device IDs within a single message instead of a dedicated unicast message, enables optimal resource usage.

The simplest way of allowing access to a device within a given time/frequency slot would be the reader providing a list of RN16’s which won the contention, and the device then uses the frequency resource which it randomly selected to transmit msg3 and subsequent messages. However, this may bring multiple issues such as;

- The device would need to store the selected frequency slot to be used for transmission of msg3.

- In some cases, two devices may select the same frequency slot but the reader may be able to resolve contention anyway in case not all frequency slots within the timeslot were contented. In such cases it may be beneficial to allow the reader to provide one of the devices with one of the free frequency slots instead of relying on re-access

- Due to various gNB implemented procedures, it may determine that different frequencies are better for the device to transmit on in order to optimise performance. Such goals could be to;

  - Interference mitigation by spacing devices to accommodate for the bad internal filters,

  - Frequency hopping due to security reasons, or others.

Proposal 4: The device should not be expected to store the frequency index in which it sent Msg1, i.e. reader must always indicate current resources.

How the reader indicates such frequency slot may be further discussed, but some options are;

- An ordered list of RN16s of the devices, in which each entry indicates a dedicated frequency slot.

  - The disadvantage is that padding must occur in case fewer devices has requested access within the timeslot than how many frequency slots are available. Furthermore, in case a reader may choose to only provide a subset of frequencies of what is granted, no padding means ambiguity on which slot is referred.

- An RN16 followed by a frequency slot number or bitmap indicating the frequency slot

  - For few frequency slots, this may cause more overhead, but for a larger number of frequency slots this will perform better than padding.

Proposal 5: RAN2 to design a procedure for a reader to assign a dedicated frequency resource for a device within Msg1.

Lastly, a question arises whether the device should use the same frequency during the entire procedure in case a command is indicated. In our view, this would again require the device to store which frequency resource should be used, and thus we believe that any R2D resource carrying an D2R should also carry the frequency index/bitmap.

Proposal 6: RAN2 to discuss whether the frequency index provided in Msg1 is persistent for all subsequent D2R communication within the transaction or whether each R2D transmission providing a D2R resource will provide also a frequency index.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: The WID states that forward compatibility should be included when designing (at least) paging messages.

Observation 2: The usage of AS ID ensures forward compatible multi-devices paging in CFRA without the need to use the full device ID.

Observation 3: If AS ID is assigned using the read or write command, the later devices in a multi-device CFRA procedure ends up waiting a significant time before the msg1 resources are provided.

Proposal 1: A separate AS ID assignment command shall be adopted to support multiple device scenario. FFS whether this can be multiplexed with a read/write command.

Proposal 2: RAN2 to discuss whether CFRA takes precedence over CBRA in any given case i.e. a device may interrupt any ongoing CBRA initiated procedure when a CFRA paging has the device ID. FFS on whether this is applicable during a segmented command.

Proposal 3: The R2D transmission determining the Msg1 resources will indicate at least number of frequency resources per time slot to the device.

Proposal 4: The device should not be expected to store the frequency index in which it sent Msg1, i.e. reader must always indicate current resources.

Proposal 5: RAN2 to design a procedure for a reader to assign a dedicated frequency resource for a device within Msg1.

Proposal 6: RAN2 to discuss whether the frequency index provided in Msg1 is persistent for all subsequent D2R communication within the transaction or whether each R2D transmission providing a D2R resource will provide also a frequency index.