3GPP TSG-RAN WG2 Meeting #130 	R2-2503864

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Discussion on A-IoT paging

Document for:	Discussion

# Introduction

In the previous meeting, the following agreements have been made for A-IoT paging.

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to take this into account for their design.

This paper is intended to discuss some remaining issues on paging, including first FFS in the above agreements (device behavior if it gets a new service request while one procedure is still ongoing), whether/how to define “end of procedure”, reader ID and skip response due to energy constraint.

# Discussion

## “End of procedure” and device behaviour if multiple requests are received in parallel

In the previous meeting, the following proposal has been discussed without conclusion:

Proposal 1: Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure, or (b) terminate the ongoing procedure and respond to the latest request.

In the following, we first discuss whether/how to define end of procedures for both options, and then provide a way forward for selection between options a and b.

### Whether/how to define end of procedure

It is clear that for option a, end of procedure needs to be defined. In the previous meeting discussion, one argument of supporting option b was that it removes the need to define end of procedure, because device would always respond to the latest request as long as it receives the paging.

However, even for option b, there is still a need for device to conclude whether the previous procedure has ended successfully or failure, in case that the device has been interrupted by a new service request and hence respond to this new request. For example, device has sent a Msg3 corresponding to one paging with transaction ID#n. Now the device receives another paging with transaction ID#n+1. Option b requires the device to abandon the procedure related to transaction ID#n and respond to new paging with transaction ID#n+1. In this case, device still needs to conclude whether the procedure related to transaction ID#n has been successful or not, based on which device would decide whether to respond to subsequent paging with transaction ID#n in the future time instant. If the transaction ID#n is regarded as successful, device would not respond to future paging with transaction ID#n. If concluded otherwise (i.e. transaction ID#n has failed), device would need to respond to the transaction ID#n in future paging. The same question can be asked if the device is interrupted by a new paging with transaction ID#n+1 after sending Msg1 for CBRA (and waiting for Msg2) in response to a paging with transaction ID#n.

Logically speaking, if an “ongoing” procedure is interrupted without completion, the transaction should be regarded as a failure. This goes back to the question on how to define a procedure is ongoing. In the example mentioned above, if a procedure related to transaction ID#n is interrupted after Msg1 for CBRA, since the contention has not been resolved (implying that the procedure is not ended), it makes sense to treat this transaction ID#n as failure. On the other hand, if the end of procedure is defined as “device has sent Msg3”, then the transaction ID#n in the above example would be regarded as a success.

With the above considerations, it can be seen that regardless of options a or b, end of procedure needs to be defined. Let us first discuss from reader perspective then from device perspective.

Depending on service type (e.g. “inventory only” or “inventory + command”) and contention-based or contention-free random access, the definition of completion of the procedure would be different. See the following table 1 from reader perspective:

Table 1: Procedure completion from reader perspective

| Case number | Case description | Procedure completion from reader perspective |
| --- | --- | --- |
| 1 | Inventory with CBRA | Msg3 is successfully received |
| 2 | Inventory with CFRA | First D2R data transmission (i.e. Msg1) is successfully received |
| 3 | Inventory and command with CBRA | The scheduled D2R by R2D command is successfully received |
| 4 | Inventory and command with CFRA | The scheduled D2R by R2D command is successfully received |

However, from device perspective, more discussion is required on how the procedure can be seen as completed. Notice that we (RAN2) concluded that service type (e.g. “inventory only”, “inventory + command”) will not be included in paging message. Therefore, after being triggered device may not know the triggered procedure is for “inventory only” or “inventory +command”. Device needs to follow the R2D message in order to figure out whether it needs to transmit a next D2R. In the following, we separate the discussion between CBRA and CFRA as an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message was agreed.

Device determination of a successful/failed/pending procedure for CBRA

The following figure 1 illustrates possible steps for “inventory only” and “inventory + command” for CBRA and how device assumes the procedure status after each step.

Figure 1: Determination of a procedure status for CBRA

We would like to mention the following remarks with respect to figure 1.

-	Remark 1: A timer is assumed for device to expect Msg2 after sending Msg1. If no Msg2 is received within time window, device assumes the contention resolution has failed (e.g. random ID collision). Device should look for another access opportunity provided/triggered by the reader for re-access.

-	Remark 2: After sending Msg3, if reader receives Msg3 successfully and does not have follow-up command, reader need not send anything. Device would assume the procedure has ended successfully.

As Msg3 failure case, if the reader wants to request a retransmission of Msg3, reader must send the R2D within the time window until the next paging or trigger message (see the following agreement made in previous meeting). Reader can also use the time window to send follow-up command or simply inform NACK to device. This removes the need for introducing a timer for the time window after Msg3. .

Agreements on NACK 2. For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging

- 	Remark 3: For the follow-up command responses (see steps 5 and 7), device assumes that the procedure ends successfully after sending the command response. If reader does not receive it, reader can re-send upper layer command again. Device would need to monitor R2D anyway as long as it has energy.

Device determination of a successful/failed/pending procedure for CFRA

For the CFRA case, the steps 1 and 2a/2b in figure 1 (CBRA) will be skipped, because they are for contention resolution. The first D2R transmission starts with step 3 in figure 1, or referred to as “Msg1” for CFRA. Furthermore, RAN2 agreed in previous meeting that for CFRA, NACK feedback and re-access is not support in the previous meeting. Consequently, device can assume the procedure ends successfully after sending D2R. Since device still needs to monitor R2D, device would not miss the follow-up command from the reader even if device has assumed the procedure has ended successfully. The result is figure 2 below.

Figure 2: Determination of a procedure status for CFRA

To summarize, we propose the following:

Proposal 1. For CBRA, after sending Msg1 in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing (i.e. pending), until one of the following events occurs:

      - Msg2 is not received within a time window, in which case the procedure is concluded as failure

      - After sending Msg3 (i.e. inventory response),

        - Device receives NACK for re-access determination until next paging or trigger message, in which case the procedure is concluded as failure

        - No R2D is received until next paging or trigger message, in which case the procedure is concluded as successful

  - After sending command response (e.g. Msg5/Msg7/etc), in which case the procedure is concluded as successful

Proposal 2. For CFRA, after sending any D2R data , device assumes that the procedure associated with this paging ends successfully.

### Options a vs. b

Both options may have its own drawback. For option b, since it requires device to always respond to the latest service request and abandon the ongoing procedure, it can result in inefficiency in radio resource utilization. For example, after sending Msg1, device is waiting for Msg2 to know whether the contention is resolved. If device responds to another paging and abandons the current one, the radio resource for Msg1 would be wasted. Further, option b can cause potential ping-pong between different services or readers. Device could end up with interrupting between different services without being able to complete one procedure.

For option a, the concern is the deadlock situation as discussed in previous meeting [1]. Basically, deadlock could happen when device falls in a situation resulting in no response to paging with same or different service request.

In view of the above drawbacks, it could be beneficial to give reader control when to apply option a or option b. For example, if reader has reason to believe that the device is in a deadlock, reader can apply option b (i.e. force device to respond to new paging). Another use case of option b could be that reader wants to prioritize a new service request over the unfinished transaction with lower priority and therefore request device to abandon the current procedure in order to respond to the new service request. In other scenarios, reader can apply option a to avoid ping-pong or procedure interruption.

One straightforward way to realize the above reader control is to introduce some control information in the paging message. Device can simply follow the control information to decide whether to abandon the ongoing procedure and respond to the latest paging (i.e. option b) or ignore the paging (i.e. option a).

Proposal 3. Paging message includes control information to indicate whether device should follow option-a or option-b.

## Reader ID

The necessity of reader ID information hasnot been discussed in previous meeting due to lack of time. As summarized by rapporteur in [2], 11 companies had the view that providing reader ID information may be needed or beneficial.

For example, in a multi-reader overlapping area, device may receive paging messages from different readers with the same or different service request from CN. Depending on the service requirements, it may be preferable for device to respond to one single reader only, or multiple readers (duplicated responses). For example, for inventory service, it may be sufficient for device to respond to one (“best”) reader and ignore the paging from other readers to save the energy and signaling overhead. However, for proximity determination, it is useful for the network to locate the device if the device responds to multiple readers. Therefore, network should be able to control whether a device should respond to single reader or multiple readers.

If the transaction ID is generated by the reader without reader coordination, different readers would generate transaction ID independently. It is likely that different transaction IDs will be used by different readers even if the paging is associated with the same CN request. If no other information to control the duplicated responses is included in paging except the transaction ID, it is effectively requested that the device needs to respond to multiple readers. On the other hand, if network would like to control the device to avoid duplicated responses to multiple readers, reader ID can be included in paging. Similar to the transaction ID, device now needs to maintain the reader ID after a successful access procedure (i.e. store it in the memory). If the new paging has a different reader ID, device should ignore the paging, regardless the same or different paging ID. In this way, the device is required to respond to one reader only. If the new paging has the same reader ID as the store one, device then compares the transaction ID to decide whether it is the subsequent paging or new paging.

Proposal 4. In additional to transaction ID, a reader ID can be optionally indicated in paging message. Device needs to maintain the reader ID (if indicated) and transaction ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 5. On deciding whether or not to respond to a paging, device compares the stored IDs with the IDs in the paging as follows:

| New transaction ID | New Reader ID | Device behavior | Intended scenario |
| --- | --- | --- | --- |
| Same as stored one | Not included | Not respond | Subsequent paging from same reader |
| Different than stored one | Not included | respond | device is requested to respond to multiple readers (assuming each reader generates transaction ID independently) |
| Same as stored one | Same as stored one | Not respond | Subsequent paging from same reader |
| Different as stored one | Same as stored one | respond | New paging from same reader |
| Same or different | Different than stored one | Not respond | Device is requested to respond to one reader (assuming reader-device association is fixed) |

## Skip response to paging due to energy constraint

In the study item phase, it has been discussed that if the device does not have sufficient energy to perform the remaining or follow-up procedure, the device can send energy status report to reader in a D2R message (e.g. MSG1, MSG3 and command response message). However, according to the WID, energy status reporting is not included in the scope.

If device can identify lack of energy for the follow-up communication, instead of sending energy status report, another option is that the device can simply skip responding to the paging in the first place, since sending a D2R message itself also consumes non-negligible amount of energy and negatively impacts the energy harvesting at the device. Therefore, a more important aspect is that the A-IoT device should be allowed to skip responding if the device does not have sufficient energy to complete the remaining access procedure triggered by the paging. Whether the energy is sufficient can be judged by device implementation. This procedure can be illustrated using Fig.3. below.

Fig.3. Device skips responding based on judgement of its energy

Proposal 6. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

# Conclusion

In this paper, we have discussed some remaining issues on paging, including end of procedure, device behavior if multiple requests are received in parallel, reader ID and skip response due to energy constraint. The proposals are as follows:

Proposal 1. For CBRA, after sending Msg1 in response to an A-IoT paging, device assumes the procedure associated with this paging is ongoing (i.e. pending), until one of the following events occurs:

      - Msg2 is not received within a time window, in which case the procedure is concluded as failure

      - After sending Msg3 (i.e. inventory response),

        - Device receives NACK for re-access determination until next paging or trigger message, in which case the procedure is concluded as failure

        - No R2D is received until next paging or trigger message, in which case the procedure is concluded as successful

  - After sending command response (e.g. Msg5/Msg7/etc), in which case the procedure is concluded as successful

Proposal 2. For CFRA, after sending any D2R data , device assumes that the procedure associated with this paging ends successfully.

Proposal 3. Paging message includes control information to indicate whether device should follow option-a or option-b.

Proposal 4. In additional to transaction ID, a reader ID can be optionally indicated in paging message. Device needs to maintain the reader ID (if indicated) and transaction ID in the memory after successful completion of the access procedure triggered by a paging.

Proposal 5. On deciding whether or not to respond to a paging, device compares the stored IDs with the IDs in the paging as follows:

| New transaction ID | New Reader ID | Device behavior | Intended scenario |
| --- | --- | --- | --- |
| Same as stored one | Not included | Not respond | Subsequent paging from same reader |
| Different than stored one | Not included | respond | device is requested to respond to multiple readers (assuming each reader generates transaction ID independently) |
| Same as stored one | Same as stored one | Not respond | Subsequent paging from same reader |
| Different as stored one | Same as stored one | respond | New paging from same reader |
| Same or different | Different than stored one | Not respond | Device is requested to respond to one reader (assuming reader-device association is fixed) |

Proposal 6. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

# Reference

[1] R2-2503155	Offline discussion report: [AT129bis][010][AIoT] Paging	Qualcomm

[2] R2-2502211 Email discussion report: [POST129][035][AIoT]Paging, Qualcomm,