3GPP TSG-RAN WG2 Meeting #130										R2-2503867

St Julian’s, Malta, 19 – 23 May 2025

Agenda item		: 8.4.2

Title				: Open issues on LP-WUS in IDLE/INACTIVE mode

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

This contribution discusses the open issues related to LP-WUS operation in IDLE and INACTIVE mode.

# Discussion

2.1 Value of X in UEID calculation:

During the discussion most of the companies (10 for option 2 and 7 for option 3) preferred to either go with

- (Option 2) the legacy approach of PEI which calculates X based on applicability of eDRX or not

- Or (Option 3) a single value considering the max X value.

Both these options can work and to keep the implementation simple, a single largest value of X (X= 1048576) can be applied to calculate the UE_ID.

Proposal 1: In UE_ID based LP-WUS subgrouping, UE_ID is calculated as 5G-S-TMSI mod X, X is 1048576.

2.2 Handling LP-WUS and Group paging:

Since LP-WUS in IDLE/INACTIVE mode follows the PEI as baseline, in rel17, it was agreed that UE is not required to monitor PEI is UE is expecting to receive a group paging. We should extend the same principle for LP-WUS and agree explicitly that if UE is waiting for a group paging message, UE does not use LP-WUS.

In order to support LP-WUS for group paging, there has to significant specification work needed (changes to the UE Index sent as part of group paging message at NGAP, XNAP etc needs to be modified) and its not preferable at this late stage in this release.

Proposal 2: UE is not expected to use LP-WUS (if supported and configured) , if UE is expecting/monitoring for a group paging notification.

2.3 Evaluation period for Entry/Exit condition:

The main advantage of using LR to monitor wake up indication is that MR can move to ultra-deep sleep/ deep sleep state to save power. The currently agreed entry and exit conditions only consider the signal qualities of LP-SS/SSB measured by LR/MR. Considering just the thresholds might lead to ping-pong effect of moving between use of LR and MR resulting in UE not having sufficient time to move to between sleep states. This can even result in adverse effect as there is power penalty associated with transition between sleep states. In order to overcome this, we propose to reuse the similar principle used in Rel16 relaxed measurement criteria where there is a time period configured for which the evaluation criteria need to be satisfied to consider the condition as fulfilled.

Proposal 3: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

2.4 Entry/Exit condition configuration:

During the draft RRC CR discussion, an issue (Open issue RRC-2) was raised on if the Entry/Exit configuration is mandatory as part of the broadcasted LP-WUS configuration. The coverage of LP-WUS is agreed to be at least that of msg-3 coverage (PUSCH coverage) but this doesn’t always translate to a downlink coverage and this could in most of the cases be smaller than MR’s coverage. Also, without a defined exit conditions to check, it could result in a case where the LR goes out of coverage of the cell and cannot return back to using MR since there is no exit condition to evaluate. This case would then have to be handled by implementation or other standardised way such as missing of consecutive LP-SS occasions to declare something like “RLF” to return back to using MR. These kinds of solutions require further specification effort. Since the principles of entry/exit conditions were already studied and agreed, it would be simpler to make it mandatory to have a reliable working of LP-WUS feature.

Proposal 4: Entry/Exit condition is mandatorily present as part of the LP-WUS configuration broadcasted by network to enable LP-WUS feature

# Conclusion

Proposal 1: In UE_ID based LP-WUS subgrouping, UE_ID is calculated as 5G-S-TMSI mod X, X is 1048576.

Proposal 2: UE is not expected to use LP-WUS (if supported and configured) , if UE is expecting/monitoring for a group paging notification.

Proposal 3: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

Proposal 4: Entry/Exit condition is mandatorily present as part of the LP-WUS configuration broadcasted by network to enable LP-WUS feature