3GPP TSG-RAN2 Meeting #130	R2-2503872

# St Julian’s, Malta, May 19th - May 23th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2031 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRS hopping in positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson, CATT, Samsung, vivo, Xiaomi, Qualcomm, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For RedCap UE positioning with SRS hopping, RAN1 agreed to introduce a UTW (uplink time window) to resolve the collision between SRS hopping and other UL/DL channel/signals. The UTW is only used for resolving collision, i.e., even if the UTW is not configured, the SRS with hopping can still be transmitted in RRC_CONNECTED and RRC_INACTIVE. Agreement in RAN1#114-bis For RedCap UEs positioning transmitting the UL SRS with frequency hopping, regarding the collisions between other UL and DL signals/channels and the UL SRS with frequency hopping, support both of the following options - Option 1: UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit FH SRS for positioning. - FFS details of an UL time window - Note: it implies that UE drops the transmission of other signals/channels and transmits SRS for positioning - Option 2: new collision rules between the UL SRS with frequency hopping and other UL and DL signals/channels/. Option 2 can apply without or outside UL time window (i.e. option 1) - FFS: details on the collision rules Note: it is understood that option 2 is a component of the feature for UL SRS Tx hopping (FG 41-5-2), and option 1 is a separate feature group. Note: UE is not expected to be configured with a SRS for positioning hopping cycle partially overlapping with UTW.. Based on above: 1. Neither RAN1 nor RAN2 introduces ‘UplinkTimeWindowTimer’ in Rel-18. 2. The UTW does not have any MAC spec impact. After RRC layer configures SRS hopping parameter and UTW, UE’s physical layer will transmit SRS hopping with the corresponding restriction, MAC does not need to translate UTW location to the physical layer. Furthermore, such UE behavior towards UTW is already specified in RAN1 specification, 38.214, 6.2.1.4.1, wherein the physical layer gets the UTW location directly based on/according to RRC configured parameters. Further RAN2#129-bis has agreed the following: Agreement: The definition of the UTW boundaries is captured in RRC field descriptions, including the formula which will be added once we have the RAN1 reply on startSFN. The CR in R2-2502080 will be implemented in the next meeting (removal of section 5.32 from MAC).. |
| Summary of change: |  | - Delete UplinkTimeWindowTimer related description in MAC spec - Delete UTW formula in MAC spec Impact analysis Impacted 5G architecture options: SA Impacted functionality: Positioning SRS frequency hopping for RedCap UE Inter-operability: - If the UE is implemented according to this CR while the NW is not, NW will expect UE to run a UplinkTimeWindowTimer but UE wouldn’t, so gNB will not receive correct SRS hopping; - If the NW is implemented according to this CR while the UE is not, UE will wrongly run an UplinkTimeWindowTimer which is not configured and expected by gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature that has not been introduced in Rel-18 will exist in Rel-18 MAC specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 5.26.1, 5.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is a revision of the postponed CR R2-2502080, with coversheet modification |  |  |  |  |  |  |  |  |

# START OF CHANGE

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

A2X communication: A communication to support A2X services leveraging PC5 reference points, as defined in TS 23.256 [31]. A2X services are realized by various types of A2X applications, e.g., BRID or DAA.

Air to Ground network: An NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000 m and take-off/landing altitudes down to 3000 m.

BWP for SRS for positioning Tx frequency hopping: For SRS for positioning Tx frequency hopping, separate BWP configuration outside BWP configuration for data transmission.

Dedicated SL-PRS resource pool: A sidelink resource pool which can be used for the transmission of SL-PRS and cannot be used for the transmission of PSSCH.

Dormant BWP: The dormant BWP is one of downlink BWPs configured by the network via dedicated RRC signaling. In the dormant BWP, the UE stop monitoring PDCCH on/for the SCell, but continues performing CSI measurements, Automatic Gain Control (AGC) and beam management, if configured.

DRX group: A group of Serving Cells that is configured by RRC and that have the same DRX Active Time.

eRedCap UE: A UE with enhanced reduced capabilities as specified in clause 4.2.22.1 of TS 38.306 [25].

HARQ information: HARQ information for DL-SCH, for UL-SCH, or for SL-SCH transmissions consists of New Data Indicator (NDI), Transport Block Size (TBS), Redundancy Version (RV), and HARQ process ID.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes.

Listen Before Talk: A procedure according to which transmissions are not performed if the channel is identified as being occupied, see TS 37.213 [18].

LTM candidate cell: A candidate cell configured for LTM as defined in TS 38.331 [5].

Msg3: Message transmitted on UL-SCH containing a C-RNTI MAC CE or CCCH SDU, submitted from upper layer and associated with the UE Contention Resolution Identity, as part of a Random Access procedure.

Multi-path: Mode of operation of a UE in RRC_CONNECTED configured with one direct path on which the UE connects to gNB using NR Uu, and one indirect path on which the UE connects to the same gNB via another UE using PC5 unicast link or non-3GPP connection (N3C).

Multi-PUSCH configured grant: A configured grant configuration configured with nrOfSlotsInCG-Period (see TS 38.331 [5]). It includes multiple consecutive configured uplink grants within a single periodicity.

N3C indirect path: In Multi-path, the indirect path using Non-3GPP Connection between remote UE and relay UE.

NCR-Fwd: NCR-node function, which performs amplifying-and-forwarding of UL/DL RF signals between gNB and UE. The behavior of the NCR-Fwd is controlled according to the side control information received by the NCR-MT from a gNB.

NCR-MT: NCR-node entity which communicates with a gNB via a control link to receive side control information. The control link is based on NR Uu interface.

NCR-node: RAN node comprising NCR-MT and NCR-Fwd.

Non-terrestrial network: An NG-RAN consisting of gNBs, which provide non-terrestrial NR access to UEs by means of an NTN payload embarked on an airborne or space-borne NTN vehicle and an NTN Gateway.

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [19] and ProSe communication (including ProSe non-Relay, UE-to-Network Relay and UE-to-UE Relay communication (including ProSe UE-to-UE Relay communication with integrated discovery)) as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink discovery: AS functionality enabling ProSe non-Relay discovery, ProSe UE-to-Network Relay discovery and ProSe UE-to-UE Relay discovery for Proximity based Services as defined in TS 23.304 [26], between two or more nearby UEs, using NR technology but not traversing any network node.

NR sidelink transmission: Any NR Sidelink-based transmission, including transmission for NR sidelink discovery, transmission for NR sidelink communication, transmission for Ranging/Sidelink Positioning, and transmission for A2X communication.

PDCCH occasion: A time duration (i.e. one or a consecutive number of symbols) during which the MAC entity is configured to monitor the PDCCH.

Positioning SRS Bandwidth Aggregation: Transmission of positioning SRS on multiple carriers in RRC_CONNECTED and RRC_INACTIVE where the positioning SRS resources are linked in RRC configuration as defined in TS 38.331 [5].

PRS Processing Window: A time window during which UE may perform PRS measurement inside the active DL BWP with the same numerology as the active DL BWP without measurement gap.

Ranging/Sidelink Positioning: AS functionality enabling ranging-based services and sidelink positioning as specified in TS 23.586 [30].

RB set: A RB set refers to a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum as defined in TS 37.213 [18].

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1 in TS 38.306 [25].

Serving Cell: A PCell, a PSCell, or an SCell in TS 38.331 [5].

Shared SL-PRS resource pool: A sidelink resource pool which can be used for the transmission of both SL-PRS and PSSCH.

Sidelink transmission information: Sidelink transmission information included in an SCI for an SL-SCH transmission or SL-PRS transmission with or without SL-SCH transmission on Shared SL-PRS resource pool as specified in clause 8.3 and 8.4 of TS 38.212 [9] consists of Sidelink HARQ information including NDI, RV, Sidelink process ID, HARQ feedback enabled/disabled indicator, Sidelink identification information including cast type indicator, Source Layer-1 ID and Destination Layer-1 ID, and Sidelink other information including CSI request, SL-PRS request, SL-PRS resource ID, a priority, a communication range requirement and Zone ID and COT sharing information.

SL-PRS delay budget: Delay budget before which the SL-PRS is expected to be transmitted by the Tx UE.

SL-PRS transmission information on Dedicated SL-PRS resource pool: SL-PRS transmission information on Dedicated SL-PRS resource pool is included in an SCI for an SL-PRS transmission on Dedicated SL-PRS resource pool, as specified in TS 38.212 [9], consisting of

-	SL-PRS identification information, including cast type indicator, source ID and destination ID;

-	SL-PRS transmission other information, including SL-PRS priority, SL-PRS request, SL-PRS resource ID and resource reservation period.

SRS positioning validity area: An area consisting of a list of cells within which the corresponding positioning SRS configuration is considered as valid.

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG depending on if the MAC entity is associated to the MCG or the SCG, respectively. Otherwise the term Special Cell refers to the PCell. A Special Cell supports PUCCH transmission and contention-based Random Access, and is always activated.

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

UE-gNB RTT: For non-terrestrial networks, the sum of the UE's Timing Advance value (see TS 38.211 [8] clause 4.3.1) and kmac.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [20], between nearby UEs, using E-UTRA technology but not traversing any network node.

NOTE 1:	A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until it is stopped or expires (e.g. due to BWP switching). When the MAC entity applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise.

NOTE 2:	In this version of the specification, the SRS in the procedural description includes Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE as in clause 5.26. Positioning SRS except for the Positioning SRS for transmission in RRC_INACTIVE is treated the same as SRS by the UE unless explicitly stated otherwise.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2X	Aircraft-to-Everything

AP	Aperiodic

BFR	Beam Failure Recovery

BRID	Broadcast Remote Identification

BSR	Buffer Status Report

BWP	Bandwidth Part

CE	Control Element

CG	Cell Group

CG-SDT	Configured Grant-based SDT

CI-RNTI	Cancellation Indication RNTI

CSI	Channel State Information

CSI-IM	CSI Interference Measurement

CSI-RS	CSI Reference Signal

CS-RNTI	Configured Scheduling RNTI

DAA	Detect And Avoid

DAPS	Dual Active Protocol Stack

DCP	DCI with CRC scrambled by PS-RNTI

DL-PRS	DownLink-Positioning Reference Signal

DSR	Delay Status Report

DTX	Discontinuous Transmission

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

IAB	Integrated Access and Backhaul

INT-RNTI	Interruption RNTI

LBT	Listen Before Talk

LCG	Logical Channel Group

LCP	Logical Channel Prioritization

LTM	L1/L2 Triggered Mobility

MBS	Multicast/Broadcast Services

MCCH	MBS Control Channel

MCCH-RNTI	MBS Control Channel RNTI

MCG	Master Cell Group

MO-SDT	Mobile Originated SDT

MPE	Maximum Permissible Exposure

MTCH	MBS Traffic Channel

MT-SDT	Mobile Terminated SDT

N3C	Non-3GPP Connection

NCD-SSB	Non Cell Defining SSB

NCR	Network-Controlled Repeater

NSAG	Network Slice AS Group

NUL	Normal Uplink

NZP CSI-RS	Non-Zero Power CSI-RS

PDB	Packet Delay Budget

PEI-RNTI	Paging Early Indication RNTI

PHR	Power Headroom Report

PQI	PC5 QoS Identifier

PS-RNTI	Power Saving RNTI

PSI	PDU Set Importance

PTAG	Primary Timing Advance Group

PTM	Point to Multipoint

PTP	Point to Point

QCL	Quasi-colocation

PPW	PRS Processing Window

PRS	Positioning Reference Signal

RA-SDT	Random Access-based SDT

RRH	Remote Radio Head

RS	Reference Signal

SCG	Secondary Cell Group

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SI	System Information

SL-PRS-CS-RNTI	SL-PRS-Configured Scheduling-RNTI

SL-PRS-RNTI	SL-PRS-RNTI

SL-CS-RNTI	Sidelink-Configured Scheduling-RNTI

SL-PRS	Sidelink-PRS

SL-RNTI	Sidelink-RNTI

SpCell	Special Cell

SP	Semi-Persistent

SP-CSI-RNTI	Semi-Persistent CSI RNTI

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRI	SRS Resource Indicator

SS	Synchronization Signals

SSB	Synchronization Signal Block

STAG	Secondary Timing Advance Group

STx2P	Simultaneous Transmission with 2 Panels

SUL	Supplementary Uplink

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TPC-SRS-RNTI	Transmit Power Control-Sounding Reference Signal-RNTI

TRIV	Time Resource Indicator Value

TRP	Transmit/Receive Point

TRS	CSI-RS for tracking

U2N	UE-to-Network

U2U	UE-to-UE

UCI	Uplink Control Information

UTO-UCI	Unused Transmission Occasion - UCI

V2X	Vehicle-to-Everything

ZP CSI-RS	Zero Power CSI-RS

<Omitted>

5.26	Positioning SRS transmission in RRC_INACTIVE

5.26.1	General

Periodic and semi-persistent Positioning SRS with or without positioning SRS bandwidth aggregation can be configured for Positioning SRS transmission in RRC_INACTIVE.

SRS for positioning Tx frequency hopping can also be configured for Positioning SRS transmission in RRC_INACTIVE.

The MAC entity shall,

1>	if the TA of the configured Positioning SRS is valid according to clause 5.26.2, and the conditions for positioning SRS transmission in clause 7.3.1 of TS 38.213 [6] and clause 6.2.1.4 of TS 38.214 [7] are satisfied:

2>	instruct to the lower layer according to TS 38.214 [7] to transmit Periodic or Semi-Persistent Positioning SRS that is activated according to clause 5.18.17 or clause 5.18.37.

5.32	Void

# END OF CHANGE