3GPP TSG-RAN WG2 Meeting #130                                      R2-2503874

St Julian's, Malta, 19 - 23 May 2025

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID [1] of Rel-19 SBFD related to random access is given as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

This contribution provides our view on random access procedures in SBFD technique.

- Discussion

SBFD RACH configuration in NUL carrier

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP can be allocated on NUL carrier or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE performs random access in SBFD symbols, the ROs should occupy or overlap with both SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than NUL DL carrier to provide larger UL coverage, in order to occupy or overlap with resources in the UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 1. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Proposal 1: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

RAN2#127 has made the following agreements to discuss the RACH configuration in RRC signaling:

RAN2#127 Agreement RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

Wherein the SBFD RACH configuration in the SIB is for RRC_INACTIVE/IDLE UE to perform CBRA (or CFRA for SI request), and in dedicated RRC signaling is for RRC_CONNECTED UE to perform CFRA (or CBRA when fallback).

CBRA RACH resource is provided in SIB1 -> BWP-UplinkCommon for RRC_IDLE/INACTIVE UEs, or in RRCReconfiguration -> BWP-UplinkCommon for RRC_CONNECTED UEs. This section provides our thoughts on the RRC RACH configuration for SBFD-aware UEs in CBRA.

Fallback from 2-step legacy RO to 4-step SBFD RO

RAN2 has achieved the agreement:

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

The motivation of introducing legacy RO to fall back to SBFD RO is to increase the RA attempt for better successful rate. From this point of view, the legacy 4-step RO and legacy 2-step RO should both support to fall back to SBFD RO. Thus, the maximum failure number threshold should be configured in both RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA.

Proposal 2: Support both legacy 4-step RO and legacy 2-step RO to fall back to 4-step SBFD RO. The maximum failure number threshold should be configured in both RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA.

Figure 2. Fallback from legacy RO to SBFD RO when 2-step RACH is also configured

In legacy mechanism, if the UE initially selects 2-step RACH resource, after msgA-TransMax-r16 times of failure, UE will fall back to try 4-step RACH resource for the rest of times. RAN2 should discuss how to handle the co-existence between this new maximum failure number threshold and the legacy msgA-TransMax-r16. There can be following candidate configuration restrictions:

- gNB should not configure this maximum failure number threshold and msgA-TransMax-r16 together.

- gNB can configure this maximum failure number threshold and msgA-TransMax-r16 together , but this value should not be equal to msgA-TransMax-r16.

Proposal 3: For 2-step legacy RO fallback to 4-step SBFD RO, RAN2 should discuss how to handle the co-existence between this new maximum failure number threshold and the legacy msgA-TransMax-r16. Considering one of the following options:

- gNB should not configure this maximum failure number threshold and msgA-TransMax-r16 together.

- gNB can configure this maximum failure number threshold and msgA-TransMax-r16 together , but this value should not be equal to msgA-TransMax-r16.

Another issue is that, whether fallback from 2-step RACH resource -> 4-step RACH resource -> 4-step SBFD RACH resource can be supported. RAN2 should confirm this issue for the complete MAC procedure.

Proposal 4: RAN2 should discuss whether fallback from 2-step RACH resource -> 4-step RACH resource -> 4-step SBFD RACH resource in one RA process can be supported.

Set selection after RO type switch

RAN2 has agreed the working assumption that selecting RO type happens before selecting set:

RAN2#129-bis Working assumption For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

So when the RO type switches due to exceeding the maximum transmission number, the UE has to change from RO type 1 to RO type 2 and use the corresponding RACH partition configuration in RO type 2. There can be 2 kinds of UE behaviors:

- UE should perform a set reselection on the RO type 2 thoroughly, using all the wanted features/feature combinations. For example, UE wants to adopt SDT+slicing, but UE only selects a set with SDT in RO type 1. When switching to RO type 2, UE can still use SDT+slicing to select a set in RO type 2.

- UE should just check the set availability in RO type 2, using the final feature/feature combination associated with the already selected set of the RO type 1. For example, UE wants to adopt SDT+slicing, but UE only selects a set with SDT in RO type 1. When switching to RO type 2, UE just check whether there is a set associating with SDT in RO type 2. If exist, UE can fallback to use the set; if not exist, UE cannot fallback.

Proposal 5: When UE switches from RO type 1 to RO type 2, RAN2 to confirm which of the following is the desired UE behavior of set reselection:

- UE should perform a set re-selection on the RO type 2 thoroughly, using all the wanted features/feature combinations;

- UE should just check the set availability in RO type 2, using the final feature/feature combination associated with the already selected set of the RO type 1. If such set exists within RO type 2, UE can fallback to use that set; if does not exist, UE cannot fallback.

Early identification of SBFD-aware UE

RAN2#127-bis meeting has agreed the following:

Agreement RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

That is to say Msg1 based early identification is not supported in legacy ROs. The question is whether to support Msg3 based early identification when legacy RO is selected.

Figure 3. Example of initial access timeline

Firstly, as the figure indicates, if early identification is achieved in Msg3, the benefit is only on Msg5 UL, NAS security UL and AS security UL (see green arrows), these UL transmission can be scheduled on UL subband. When gNB acquires UE capability, the support of SBFD feature will be directly known by gNB. However the NAS security and AS security set up procedure is only applicable in RRC_IDLE, i.e., even if UE resumes from RRC_INACTIVE, these two steps are not needed. So the scenario of scheduling UL transmission in UL subband becomes more limited.

Secondly, Msg3 based early identification will achieve less benefit than Msg1 based early identification. If companies thinks early identification is not an urgent feature to support for Msg1 based early identification, then Msg3 based early identification shouldn’t be supported at all due to the lower benefit.

Thirdly, if the NW indication and RSRP threshold are not configured, SBFD-aware UE is better to use SBFD RO by default. So early identification can also be achieved when SBFD-aware UE always selects SBFD RO in Msg1/A, the motivation of enhancing Msg3 is further weakened.

Based on the above, we have the following proposal:

Proposal 6: Do NOT support Msg3 based early identification for SBFD-aware UE.

RA-RNTI collision issue

RAN2 has discussed RA-RNTI collision issue and there is no consensus. We acknowledge that such scenario may exist (i.e., RRC SBFD RACH configuration option 2 may configure additional ROs which is overlapped with the legacy ROs), however RAN1 has made the previous conclusion that:

RAN1#120 Conclusion For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1: - Alt-1: do not optimize for this case

Therefore, whether to solve the RO overlapping issue depends on RAN1’s decision. If RAN2 cannot make consensus, we suggest to ask RAN1 for clarification to avoid endless RAN2 debate.

Proposal 7: Send LS to RAN1 to ask whether there is a need to solve the RA-RNTI collision issue for RRC SBFD RACH configuration option 2, and if yes, what the recommended solution is.

CFRA in SBFD ROs

LTM CFRA with SBFD RO

RAN2#120-bis has achieved the following agreement:

Agreement Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

Using SBFD RO to initiate RA just provide another set of radio resources for accessing. If a target cell is configured with more SBFD resources than legacy resource, it will be quicker for the UE to access successfully to the target cell. Since LTM is also introduced with the motivation of reducing handover latency, using SBFD RO to access for LTM also can further reduce latency to access to target cell. So the motivation is well-aligned.

LTM early sync, which is a part of whole LTM cell switch procedure, also gets benefit from earlier SBFD RO to reduce latency as much as possible. Therefore, the CFRA of LTM early sync should also support SBFD RO.

Observation 1: SBFD RO and LTM share same intention to reduce handover latency.

Further, the specification impact of SBFD RO+ LTM can be rather small. RAN2’s main work is to let UE know whether to use SBFD RO or legacy RO to initiate LTM CFRA, so source cell that sends the handover command should also send the RO type indication to UE.

Further, supporting LTM+SBFD RO does not have RAN3 impact. Candidate cell sends the CFRA resource to the source cell before handover, and the CFRA resource sent in current F1 interface just cites the RAN2 IE RACH-ConfigDedicated and EarlyUL-SyncConfig. For example, if RAN2 indicates RO type in EarlyUL-SyncConfig, when the EarlyUL-SyncConfig is generated by candidate cell and sent to source cell, it means that candidate cell decides to let UE sending preamble on SBFD ROs to the candidate cell. See below yellow highlight RAN2 container in F1AP message:

- 38.473-i50 - 9.2.2.7 UE CONTEXT MODIFICATION REQUEST This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. LTM Information Modify; 0..1; YES; reject. …. LTM CFRA Resource Config List; 0..1; YES; ignore. >LTM CFRA Resource Config Item IEs; 1 .. <maxnoofLTMCells>; EACH; ignore. >>Cell ID; M; NR CGI 9.3.1.12; -. >>LTM CFRA Resource Configuration; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8].; -. >>LTM CFRA Resource Configuration for SUL; O; OCTET STRING; Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier.; -. LTM Configuration ID Mapping List; O; 9.3.1.294; YES; reject. Early Sync Candidate Cell Information List; 0..1; YES; ignore. >Early Sync Candidate Cell Information Item IEs; 1 .. <maxnoofLTMCells>; EACH; ignore. >>Cell ID; M; NR CGI 9.3.1.12; -. >>TCI States Configurations List; O; OCTET STRING; Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8].; -. >>Early UL Sync Configuration; O; 9.3.1.328; -. …. - 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329.

Therefore, we have the following observations and proposal:

Observation 2: The RAN2 specification impact of supporting SBFD RO and LTM is small. There is no RAN3 spec impact on LTM+SBFD since current F1AP transfers RAN2 CFRA configuration signaling as a container.

Proposal 8: Support using SBFD RO in LTM cell switch and LTM early sync CFRA case.

LTM cell switch with SBFD RO

For UE performing CFRA for LTM cell switch, a LTM cell switch commend MAC CE is introduced to indicate CFRA resource in LTM:

TS38.321-i30: - 6.1.3.75 LTM Cell Switch Command MAC CE Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

Even if UE takes the RACH-ConfigDedicated from the target cell as the baseline RACH parameters, it is source cell’s decision to utilize SBFD RO or legacy RO during CFRA to the target cell (at least for intra-DU LTM case). This is similar to the Rel-18 case where source cell decides to perform a specific Msg1 repetition number and includes it in the MAC CE. So the RO type indication should be contained in the LTM cell switch MAC CE.

Then, UE can select RO type for the LTM CFRA case directly based on the indication in this LTM cell switch MAC CE, rather than based on the RACH-ConfigDedicated of the target cell.

Proposal 9: Include 1-bit RO type indication in the LTM cell switch MAC CE.

LTM early sync with SBFD RO

For LTM early sync, the serving cell provides RRC parameter EarlyUL-SyncConfig for each of the candidate cells to the UE. UE totally relies on the EarlyUL-SyncConfig and to send preambles to each of the candidate cells without actually accessing to the candidate cells nor reads SIB of the candidate cell. If UE is scheduled to send preamble on SBFD ROs (via PDCCH order) to the candidate cell, the corresponding EarlyUL-SyncConfig of the candidate cell should provide the SBFD time and frequency location, so that UE can know the SBFD RO location of the candidate cell.

Since the EarlyUL-SyncConfig is provided by candidate cell to the source cell in advance, if the SBFD time and frequency location is included in the EarlyUL-SyncConfig by the candidate cell, it means candidate cell is available to receive preambles on the SBFD ROs. With this permission, the source cell can schedule UE using SBFD RO via PDCCH order.

Proposal 10: For LTM early sync, include the SBFD time and frequency resource of each candidate cell in the RRC signaling EarlyUL-SyncConfig.

SBFD configuration type in CFRA

RAN1 has achieved several agreements/conclusions to describe the SBFD RACH configuration option 1 and option 2, and what kind of additional RO that the RACH configuration option 1 and option 2 actually contains:

RAN1#116-bis Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration), - For the legacy-ROs, including the ROs in non-SBFD symbols and the ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon (if any), the legacy SSB-RO mapping is followed. - For the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, separate SSB-RO mapping will be used RAN1#117 Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs RAN1#117 Agreement A RO across SBFD symbols and non-SBFD symbols in the same slot or across slots is invalid by default. A configured RO starting from SBFD symbol and ending in non-SBFD symbol either in the same slot or across different slots can be valid based on network configuration. This is only supported for RACH configuration option 2 and only supported for the ROs configured by the additional RACH configuration. If network configures such RO as a valid RO, UE should treat the RO as an additional-RO in SBFD symbols, and the followings are assumed by network and UE: - The same frequency resources are used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL transmit power is used for both the SBFD segment and non-SBFD segment of the PRACH. - The same UL spatial domain filter is used for both the SBFD segment and non-SBFD segment of the PRACH. - UE doesn’t stop PRACH transmission in the transition period/gap (if any) between SBFD and non-SBFD symbols - There are no phase coherency requirements on the UE between the SBFD segment and non-SBFD segment of the PRACH. - Other assumptions are not precluded. NOTE: For FR2, network may need to ensure that the additional-RO and the legacy RO, which overlap with each other in time domain, are mapped to the same SSB. RAN1#118 Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs). RAN1#118 Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

Based on the above RAN1 agreements, we have the following observations:

- Each of the additional RO in option 1 should have at least one symbol that is a legacy D symbol, and all the symbols of each of the additional RO should be SBFD symbol; (i.e., an additional RO in option 1 does not allow to be all in legacy F symbols, and one additional RO does not allow to extent from SBFD symbol to non-SBFD symbol)

- Each of the additional RO in option 2 can be either legacy D symbol or legacy F symbol, and at least one symbol of each of the additional RO should be SBFD symbol. (i.e., an additional RO in option 2 can be all legacy D symbols, all legacy F symbols, or legacy D symbol + legacy F symbol, and one additional RO can extent from SBFD symbol to non-SBFD symbol)

Therefore, even if with the same RACH-ConfigGeneric and same TDD configuration and same SBFD time/frequency location, RRC SBFD RACH configuration option 1 and option 2 will derive different additional RO location for a SBFD-aware UE, see the following figure:

Figure 4. With same RACH-ConfigGeneric and same TDD/SBFD time/frequency location, option 1 and option 2 will derive different additional RO location

Observation 3: Even if with the same RACH-ConfigGeneric and same TDD/SBFD time/frequency location, RRC SBFD RACH configuration option 1 and option 2 will derive different additional RO location for SBFD-aware UE.

Based on above analysis, UE should know which of the SBFD RACH configuration type (i.e., either option 1 or option 2) is provided to the UE, then UE can decide how to derive the location of additional RO:

- In CBRA, gNB can explicit provide either SBFD RACH configuration option 1 or option 2 to the UE, so it will have no problem.

- In CFRA, UE should also know SBFD RACH configuration option 1 or option 2 and the indicated RO type for each of the CFRA cases. However, currently option 1 and option 2 will not be reflected in each CFRA case since UE does not need to perform complex selection procedure between SBFD RO and legacy RO. To solve the issue, there can be several solutions:

  - One method is to adopt the corresponding common SBFD RACH configuration type for CBRA (if corresponding common RACH resource also provides SBFD RACH configuration).

  - However option 1 and option 2 may be both absent in common RACH parameters, i.e. CBRA does not configure any SBFD resources. In this case if CFRA indicates to use SBFD RO, there will be no reference on which SBFD RACH configuration type the UE should refer to. So under this scenario, the SBFD RACH configuration type should be explicitly provided for the specific CFRA case.

  - Or, UE and gNB can always assume one RACH configuration type for each of the CFRA cases, no matter the SBFD RACH configuration type in common RACH parameters is provided or not.

So RAN2 should firstly confirm that UE also needs to know the SBFD RACH configuration type for each of CFRA cases. To solve the issue, RAN2 should further discuss how to let UE know the SBFD RACH configuration type for each of CFRA cases.

Proposal 11: If a CFRA configuration indicates UE to use SBFD RO, UE should know the SBFD RACH configuration type (option 1 or option 2). FFS how UE knows SBFD RACH configuration type in each CFRA cases.

CSI-RS used in SBFD RO in CFRA

Currently the CSI-RS beam can be used on legacy RO in BFR and ReconfigurationWtihSync kind of CFRA. From technical perspective, it is feasible to support using CSI-RS and RO mapping, and we see no reason to block this for SBFD RO to achieve more flexibility and more RA chances. Therefore, when SBFD RO is indicated in CFRA, UE should also be able to examine the availability of CSI-RSRP to determine whether the CSI-RS beam can be used or not.

Further, there is no need to design separate SSB-RSRP or CSI-RSRP threshold than legacy, since UE measures SSB or CSI-RS in advance, so the measured SSB or CSI-RS can be in legacy symbol.

Proposal 12: Support using CSI-RS beam for SBFD RO in BFR and ReconfigurationWtihSync kind of CFRA, and there is no need to design separate SSB-RSRP or CSI-RSRP threshold than legacy.

Fallback from CBRA to CFRA

RAN2#129-bis has agreed the following working assumption:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

If CFRA indicates to use SBFD RO, UE will select SBFD RO in the first step, then UE selects set within the RACH partition configuration of the SBFD RO. For the CFRA case, UE also has to select a set from RACH partition configuration in the beginning of RA procedure, for fallback use. However, the CBRA may not even configure any SBFD ROs, so the fallback set in this case should be selected based on the RACH partition configuration on the legacy RO.

Considering this, the example procedure of CFRA fallback to CBRA is as follows:

Figure 5. Procedure of UE selecting SBFD RO in a CFRA case

Proposal 13: If a CFRA case indicates to use SBFD RO, when selecting set for the CFRA, UE should:

- If common RACH parameters of the same BWP provides SBFD ROs, UE selects set within the RACH partition of SBFD RO;

- If common RACH parameters of the same BWP does not provide SBFD ROs, UE selects set within the RACH partition of legacy RO.

Msg1 repetition in SBFD ROs

RAN1#118 has made the following agreement regarding Msg1 repetition in SBFD ROs:

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases.

It can be seen that Msg1 repetition on SBFD ROs is supported in RAN1, so RAN2 should discuss solutions and specification impacts on this aspect.

Fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD RO

First issue is whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA. In legacy mechanism, UE can fallback from lower repetition number to higher repetition number if several RA attempts fail, in order to increase the RA successful rate of the poor coverage UE. For the similar purpose, the UE should also be able to fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols.

The spec also restricts that UE should select a fallback set that has the same associated feature combination:

TS38.321-i40, 5.1.4: 2> if the Random Access procedure is not completed: 3> if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure: 4> if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or 4> if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1: 5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

To reduce the specification effort, the fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD symbols should also be associated the same feature combination, i.e., UE does not re-select RACH resource set when such fallback happens.

Proposal 14: Support fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD ROs, and such fallback should be associated with the same feature/feature combination.

RO type switch with Msg1 repetition

RAN2 has agreed the switch between RO types. RAN1/2 also agrees to perform Msg1 repetition on SBFD ROs. So RAN2 should further consider whether the two enhancement can work together or not, i.e., whether to allow UE firstly perform Msg1 repetition on a RO type for several times, and fallback from low Msg1 repetition number to higher Msg1 repetition number on this RO type, then switch to another RO type to continue Msg1 repetitions.

We think this is a reasonable and feasible solution to improve the RA successful rate. Therefore RAN2 should support this.

Proposal 15: Support UE firstly perform Msg1 repetition on one RO type for several times and fallback from low Msg1 repetition number to higher Msg1 repetition number, then fallback to another RO type to continue Msg1 repetitions.

- Conclusion

In this contribution, we propose the following observation and proposals:

SBFD RACH configuration in NUL carrier

Proposal 1: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

Proposal 2: Support both legacy 4-step RO and legacy 2-step RO to fall back to 4-step SBFD RO. The maximum failure number threshold should be configured in both RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA.

Proposal 3: For 2-step legacy RO fallback to 4-step SBFD RO, RAN2 should discuss how to handle the co-existence between this new maximum failure number threshold and the legacy msgA-TransMax-r16. Considering one of the following options:

- gNB should not configure this maximum failure number threshold and msgA-TransMax-r16 together.

- gNB can configure this maximum failure number threshold and msgA-TransMax-r16 together , but this value should not be equal to msgA-TransMax-r16.

Proposal 4: RAN2 should discuss whether fallback from 2-step RACH resource -> 4-step RACH resource -> 4-step SBFD RACH resource in one RA process can be supported.

Proposal 5: When UE switches from RO type 1 to RO type 2, RAN2 to confirm which of the following is the desired UE behavior of set reselection:

- UE should perform a set re-selection on the RO type 2 thoroughly, using all the wanted features/feature combinations;

- UE should just check the set availability in RO type 2, using the final feature/feature combination associated with the already selected set of the RO type 1. If such set exists within RO type 2, UE can fallback to use that set; if does not exist, UE cannot fallback.

Proposal 6: Do NOT support Msg3 based early identification for SBFD-aware UE.

Proposal 7: Send LS to RAN1 to ask whether there is a need to solve the RA-RNTI collision issue for RRC SBFD RACH configuration option 2, and if yes, what the recommended solution is.

LTM in SBFD ROs

Observation 1: SBFD RO and LTM share same intention to reduce handover latency.

Observation 2: The RAN2 specification impact of supporting SBFD RO and LTM is small. There is no RAN3 spec impact on LTM+SBFD since current F1AP transfers RAN2 CFRA configuration signaling as a container.

Proposal 8: Support using SBFD RO in LTM cell switch and LTM early sync CFRA case.

Proposal 9: Include 1-bit RO type indication in the LTM cell switch MAC CE.

Proposal 10: For LTM early sync, include the SBFD time and frequency resource of each candidate cell in the RRC signaling EarlyUL-SyncConfig.

CFRA in SBFD ROs

Observation 3: Even if with the same RACH-ConfigGeneric and same TDD/SBFD time/frequency location, RRC SBFD RACH configuration option 1 and option 2 will derive different additional RO location for SBFD-aware UE.

Proposal 11: If a CFRA configuration indicates UE to use SBFD RO, UE should know the SBFD RACH configuration type (option 1 or option 2). FFS how UE knows SBFD RACH configuration type in each CFRA cases.

Proposal 12: Support using CSI-RS beam for SBFD RO in BFR and ReconfigurationWtihSync kind of CFRA, and there is no need to design separate SSB-RSRP or CSI-RSRP threshold than legacy.

Proposal 13: If a CFRA case indicates to use SBFD RO, when selecting set for the CFRA, UE should:

- If common RACH parameters of the same BWP provides SBFD ROs, UE selects set within the RACH partition of SBFD RO;

- If common RACH parameters of the same BWP does not provide SBFD ROs, UE selects set within the RACH partition of legacy RO.

Msg1 repetition in SBFD ROs

Proposal 14: Support fallback from lower Msg1 repetition number to higher Msg1 repetition number on SBFD ROs, and such fallback should be associated with the same feature/feature combination.

Proposal 15: Support UE firstly perform Msg1 repetition on one RO type for several times and fallback from low Msg1 repetition number to higher Msg1 repetition number, then fallback to another RO type to continue Msg1 repetitions.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06