3GPP TSG-RAN WG2 #130 meeting	R2-2503883

St Julian's, Malta, 19th May – 23rd May, 2025

Source:		NEC

Agenda item:		8.7.4.1	LCP enhancements

Title:		Remaining issues on LCP enhancements

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#129bis meeting, agreements [1] on LCP enhancements for XR were made as below:

Working assumption (to be revisited next meeting): No Bj enhancement is introduced. It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications. We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

In this contribution, we will provide our views on the remaining issues for LCP procedure.

# 2. Discussion

2.1 LCP enhancements for priority-adjusted data.

In the RAN2#129bis meeting, it was discussed whether an LCH with an additional priority is allowed to be transmitted even if Bj < 0, and the following agreements were reached.

Working assumption (to be revisited next meeting): No Bj enhancement is introduced. It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications.

Delay-sensitive LCP is introduced to ensure the transmission of priority-adjusted data, thereby preventing the data from becoming outdated. From this perspective, it is reasonable to allow an LCH with an additional priority to be transmitted even if Bj < 0, which is already supported by the current specification. However, to avoid starvation of other LCHs, it’s proposed that only data with remaining time below the configured threshold can be transmitted when Bj < 0. In other words, we only guarantee the transmission of priority-adjusted data under such conditions.

Proposal 1: If an additional priority is applied for an LCH, only priority-adjusted data of the LCH can be transmitted when Bj < 0.

The concept of PDU Set has been introduced in Rel-18, and all PDUs belonging to a PDU Set may be needed for the decoding of the corresponding information unit. In this situation, all PDUs belonging to a PDU Set shall be discarded as a whole if at least one PDU is discarded. Similarly, as all PDUs belonging to a PDU Set share the same PSDB, to avoid only part of a PDU Set being multiplexed in a MAC PDU but all PDUs are needed for decoding, which still brings extra delay for a PDU Set, RAN2 should consider maximizing the transmission of a PDU Set when the additional priority is applied for an LCH. For specification impact, maybe an additional note can be introduced.

Proposal 2: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when the additional priority is applied for an LCH.

The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

In the legacy LCP process, logical channels with equal priority should be served equally. However, with the enhanced delay-awareness LCP, there's a need to discuss whether an LCH with priority-adjusted data should be served before one without such data when both have equal priority. To ensure the timely transmission of delay-critical data, we prefer to serve the LCH with priority-adjusted data first in this scenario.

Proposal 3: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data during LCP when both have equal priority.

Regarding the configured UL grant originally for XR traffic, the delay-awareness LCP mechanism can be applied to multiplex the priority-adjusted data with a higher priority. However, there exist other types of periodic traffic (e.g., voice or URLLC service) that are configured with configured UL grant as well. For these services, if delay-awareness LCP is applied to the configured UL grants, priority-adjusted data of XR traffic may occupy the resources originally reserved for the periodic voice or URLLC traffic, which shall bring extra delay for these services and cause a bad user experience. For this, we propose that different configured UL grant configurations can apply different strategies of whether to apply the delay-awareness LCP mechanism.

Proposal 4: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

In the RAN2#125bis meeting, it was agreed to consider the potential impact of delay-awareness LCP on traffic from SRBs. From our understanding, we should discuss SRB 0 and SRBs other than SRB 0 separately.

RAN2 should consider potential impact on traffic from SRBs.

For SRB0 (i.e., data from UL-CCCH), it always has a higher priority than delay-critical data (i.e., data from any Logical Channel, except data from UL-CCCH), which has already been specified by the priority order of logical channels in current specification [2]. There is no need to break the rules.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; … - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; … NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

For SRBs other than SRB 0, a priority value is associated with the corresponding logical channel, either by applying the default configuration or an explicit configuration. From our understanding, by configuring a proper priority value for the logical channel with delay-critical data, the network can control the impact and the priority order between logical channels with delay-critical data and logical channels for SRBs other than SRB 0. So it can left to the network’s implementation and there should be no specification impact.

Proposal 5: It can be left to the network’s implementation to configure the priorities of LCHs with delay-critical data and LCHs for SRBs other than SRB 0.

2.2 Intra-UE prioritization

Regarding the specification impact of using additional LCP priority for SR priority determination during intra-UE prioritization. Firstly, we would like to advocate that the additional priority value should also be applied in the SR case. This allows SR associated with LCH with priority-adjusted data to be prioritized in case of overlapping UL grants and SRs.

For specification impact, based on the latest running CR, the following update may be captured.

The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR. The priority of the logical channel triggering an SR shall be the value of additionalPriority if the logical channel is configured with priorityAdjustmentThreshold and has priority-adjusted data; otherwise, the priority of the logical channel triggering an SR shall be the value of priority.

Proposal 6: [MAC-01] Additional priority is applied to SR priority determination during intra-UE prioritization.

# Summary

This contribution provides our analysis on the enhancements of LCP procedure, and has the following proposals:

Proposal 1: If an additional priority is applied for an LCH, only priority-adjusted data of the LCH can be transmitted when Bj < 0.

Proposal 2: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when the additional priority is applied for an LCH.

Proposal 3: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data during LCP when both have equal priority.

Proposal 4: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

Proposal 5: It can be left to the network’s implementation to configure the priorities of LCHs with delay-critical data and LCHs for SRBs other than SRB 0.

Proposal 6: [MAC-01] Additional priority is applied to SR priority determination during intra-UE prioritization.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7th Apr – 11th Apr, 2025

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.5.0