3GPP TSG-RAN WG2 #130 meeting	R2-2503885

# St Julian's, Malta, 19th May – 23rd May, 2025

Source:		NEC

Agenda item:		8.7.4.2	DSR enhancements

Title:		Remaining issues on DSR enhancements

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#129bis meeting, agreements [1] on DSR enhancements for XR were made as below:

To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

In this contribution, we will further discuss the remaining issues related to the enhanced DSR.

# 2. Discussion

Enhanced DSR

In the RAN2#129bis meeting, we discussed the “empty DSR” issue, where a DSR is triggered but there is no delay-reporting data, and reached the following agreement:

To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

And in the RAN2#128 meeting, we agreed to configure the triggering and reporting thresholds without constraints.

Let the network configure the triggering and reporting thresholds without constraints.

The first agreement imposes a constraint on the triggering and reporting thresholds, which conflicts with the second one. We believe the constraint is unnecessary. A better approach to addressing this issue is to ensure that delay-reporting UL data is available in the buffer when triggering the enhanced DSR. For example, regardless of whether the reporting threshold is higher or lower than the triggering threshold, if there is data with remaining time below the largest configured report threshold, we can ensure that delay-reporting UL data is present. Therefore, we propose that the enhanced DSR is triggered when there is data with remaining time below both the triggering threshold and the largest configured report threshold.

Proposal 1: To address the “empty DSR” issue, RAN2 agrees that the enhanced DSR is triggered when there is data with remaining time below both the trigger threshold and the largest configured report threshold.

In Rel-18, the definition of a PDCP SDU associated with a DSR was introduced. This definition is used when determining whether a pending DSR shall be cancelled. It is based on the concept of delay-critical PDCP SDU, which is only applicable to legacy DSR.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

With the introduction of the concept of delay-reporting UL data for enhanced DSR, we may need to either revise the definition of a PDCP SDU associated with a DSR, or introduce a new definition of a PDCP SDU associated with an enhanced DSR. For example,

The concept of a PDCP SDU associated with a DSR can be updated as below:

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU or delay-reporting PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

Or a PDCP SDU associated with an enhanced DSR can be defined as below:

A PDCP SDU is considered to be associated with an enhanced DSR if it has not been transmitted in any MAC PDU and is a PDCP SDU with remaining value of the running PDCP discardTimer below the largest configured reporting threshold and associated with the logical channel which triggered the DSR.

Proposal 2: RAN2 discusses to revise the definition of a PDCP SDU associated with a DSR, or introduce a new definition of a PDCP SDU associated with an enhanced DSR.

It was agreed in RAN2#129bis meeting that non-delay-reporting data ahead of delay-reporting data shall be counted in the buffer size calculation for the enhanced DSR.

Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review)

Given that buffer size of both non-delay-reporting data and delay-reporting data may be reported by enhanced DSR, there exists a scenario where the “delay-reporting buffer size plus non-delay-reporting buffer size” equals “total buffer size”. To avoid redundant buffer size reporting, for an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in enhanced DSR equals the total buffer size.

Proposal 3: For an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in enhanced DSR equals the total buffer size (i.e., “delay-reporting buffer size + non-delay-reporting buffer size” equals “total buffer size”).

It was agreed in the RAN2#129 meeting that truncated DSR and fallback to legacy DSR is not supported in case of limited PUSCH grant size. However, we still need to discuss how to report enhanced DSR if the UL-SCH resources cannot accommodate an enhanced DSR MAC CE plus its subheader, and the following options can be taken into consideration:

- Option 1: Trigger an SR. This behaves similarly to legacy DSR.

- Option 2: Merge pairs of delay information until available UL-SCH resources can accommodate the enhanced DSR MAC CE and its subheader.

- Option 3: Report at most one pair of delay information for an LCG, which may include all the delay information.

Proposal 4: RAN2 to discuss the following options if the UL-SCH resources cannot accommodate an enhanced DSR MAC CE plus its subheader:

- Option 1: Trigger an SR.

- Option 2: Merge pairs of delay information.

- Option 3: Report at most one pair of delay information for an LCG.

Enhanced DSR and legacy DSR

RAN2 has agreed that triggering and reporting thresholds for enhanced DSR can be configured without constraints. In other words, the largest configured reporting threshold for enhanced DSR may be greater than, equal to, or lower than the remainingTimeThreshold for the legacy DSR.

Figure 1: issues when legacy and enhanced DSR coexist

If both DSR types coexist, the following issues should be considered:

- If the largest configured reporting threshold is greater than or equal to remainingTimeThreshold, the reported enhanced DSR shall cover all the delay information of legacy DSR. In this case, the legacy DSR can be cancelled if enhanced DSR is triggered.

- If the largest configured reporting threshold (e.g., T2) is lower than the remainingTimeThreshold (e.g., T3), as illustrated in Figure 1,

  - If legacy DSR is cancelled or not triggered upon triggering the enhanced DSR, then the delay information of unreported residual delay-critical data with remaining time range [T2, T3) will be missed.

  - If legacy DSR is not cancelled upon triggering the enhanced DSR, and there is unreported residual delay-critical data with remaining time range [T2, T3) after reporting of enhanced DSR, then legacy DSR shall also be reported. In this case, legacy DSR should avoid reporting redundent delay information with the remaining time range [0, T2) that has been reported by enhanced DSR.

Observation 1: The coexistence of legacy and enhanced DSR may result in unreported residual delay-critical data and the reporting of redundant delay information.

As described above, the coexistence of legacy and enhanced DSR will pose further issues and increase complexity. Consequently, to simplify the implementation and avoid intertwining the procedures related to enhanced DSR and legacy DSR, we propose that only one DSR type be used at any given time. Specifically, only enhanced DSR MAC CE shall be used when at least one LCG is configured with multiple reporting thresholds.

Proposal 5: RAN2 confirms that only enhanced DSR MAC CE shall be used when at least one LCG is configured with multiple reporting thresholds.

# Summary

This contribution provides our analysis on the enhancement of DSR, and has the following proposals:

Proposal 1: To address the “empty DSR” issue, RAN2 agrees that the enhanced DSR is triggered when there is data with remaining time below both the trigger threshold and the largest configured report threshold.

Proposal 2: RAN2 discusses to revise the definition of a PDCP SDU associated with a DSR, or introduce a new definition of a PDCP SDU associated with an enhanced DSR.

Proposal 3: For an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in enhanced DSR equals the total buffer size (i.e., “delay-reporting buffer size + non-delay-reporting buffer size” equals “total buffer size”).

Proposal 4: RAN2 to discuss the following options if the UL-SCH resources cannot accommodate an enhanced DSR MAC CE plus its subheader:

- Option 1: Trigger an SR.

- Option 2: Merge pairs of delay information.

- Option 3: Report at most one pair of delay information for an LCG.

Proposal 5: RAN2 confirms that only enhanced DSR MAC CE shall be used when at least one LCG is configured with multiple reporting thresholds.

# References

# [1] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7th Apr – 11th Apr, 2025

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.4.0