3GPP TSG-RAN WG2 Meeting #130	R2-2503894

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

The following agreements were made:

Agreements on OD-SIB1: RAN2#129

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this paper we discuss further details of the OD-SIB1 for case 2 agreed in the RAN#105 plenary meeting.

# 2	Baseline operation for case 2 – For information only

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmission

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for case 2 but of course there are still various aspects that need to be handled.

# 3	Switching between legacy SIB1 transmission and on-demand SIB1 transmission

First, we provide a summary about related agreements and discussion for this aspect. These agreements are used as starting point and assumptions for our proposals and observations for this aspect.

RAN2 agreed about the following:

RAN2 #127bis agreements:

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

RAN2 #127bis agreements:

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

RAN2 #129bis agreements:

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

RAN1#118bis agreed about the following:

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

-	SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

-	UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Based on RAN2 #129bis agreement, legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. This means, when (during the SIB1 transmission window) the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), the cell can set the K_SSB of the SSB associated with this cell to:

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2: Legacy periodic SIB1 transmission

  - Legacy UE and Rel.19 UEs can camp on the cell as legacy UE can acquire broadcasted SIB1.

- K_SSB >23 for FR1 and K_SSB > 11 for FR2.

  - Legacy UE cannot camp on the cell as legacy UE identifies the cell as NCD-SSB.

  - If Rel.19 UE has SIB1 request configuration of a cell, Rel. UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Observation 1: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

For NES SIB1 transmission mode switch, the following points are discussed in this contribution:

- UE expectation to the value of K_SSB in NES, if SIB1 is currently being broadcasted or provided on-demand.

- How UE can check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- MIB TTI is 80ms. This means that MIB payload does not change every 80ms. However, with repetition MIB can be transmitted every 20ms.

For point 1) and 2) we summarize in the state diagram below, K_SSB value with respect to NES cell SIB1 transmission mode based on RAN1 and RAN2 agreements.

Observation 2: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2, is broadcasting SIB1 as legacy and NES UE will acquire SIB1 as in legacy.

- K_SSB > 23 for FR1 and K_SSB > 11 for FR2, is providing SIB1 on-demand. UE needs to monitor SIB1 to check if it is being broadcasted within a limited window before requesting SIB1 of that cell. If UE does not detect a SIB1, UE to request OD-SIB1 for that cell.

For 3), 38.212 specifies that:

“Data arrives to the coding unit in the form of a maximum of one transport block every 80ms.”

In 38.331, it specifies that SSB periodicity is at least 80 ms, with repetition within 80 ms. Basically from 38.331 and 38.212, UE is expecting to no change in PBCH payload. Based on this, many UEs apply combining for SSB detection/decoding, mainly cell-edge UEs.

Observation 3: Legacy and NES UEs assume MIB TTI is 80ms.

To avoid impact to legacy UE, NW should ensure MIB TTI of 80ms is respected. As K_SSB is a part of MIB, hence NW cannot switch between SIB1 broadcasting state or SIB1 provided on demand with a periodicity less than 80ms. This implies that when NES cell is operating with OD-SIB1 and K_SSB > 23 for FR1 and K_SSB > 11 for FR2, UE assumes that the same value of K_SSB within 80ms.

To avoid impact to legacy UEs and to guarantee MIB TTI of 80ms (8 radio frames), NW shall consider only specific timing for the switching from broadcasting SIB1 to on-demand SIB1. RAN2 can discuss to define this specific timing as function of SFN, e.g. SFN mod 8 = 0. Hence, Rel. 19 NES UE with SIB1 request configuration intended to transmit a request for OD-SIB1, should check at least every 80ms the SIB1 transmission status based on K_SSB carried by the MIB.

Based on above discussion, it is clear to avoid any misalignment between NES UE and NW, if NES UE has SIB1 request configuration of a cell, NES UE needs to acquire/re-acquire MIB.

Proposal 1: For switching of SIB1 transmission mode, RAN2 to consider that NES UE which has SIB1 request configuration of a cell, NES UE is expected to update MIB until SIB1 acquisition is completed.

# 6	RRC connected mode UEs

In RAN#106 following was endorsed in RP-243297:

RAN2 #129bis agreed about the following for RRC connected UEs:

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

The agreement 1 refers to rel.19 UEs that acquired SIBX during RRC connected state. However, it does not mention the mechanism to carry SIBX in RRC connected state. The working assumption excludes RRC release message that can carry SIBX , i.e. UL WUS configuration.

RAN2 should clarify which message (s) to carry SIBX to UE in RRC connected state to facilitate stage-3 work. NR RRC reconfiguration message includes the dedicatedSytemInformationDeliverySIBX that can be extended to carry SIBX:

From 38.331, v 18.1.0

Proposal 2: In case of NW operation in OD-SIB1 mode, UE in RRC_CONNECTED can obtain the SIBX from NWBy the NW dedicatedSytemInformationDeliverySIBX. And then UE can use OD-SIB1 request procedure to acquire SIB1.

# 7	UL WUS configuration contents

WUS configuration contents shall be designed considering that the size of WUS configuration is not large to avoid high resource utilization for transmitting WUS configuration and thus reducing NES gain.

The starting point of WUS configuration contents are the RAN1 proposed parameters. However, it is up to RAN2 to decide if any other parameters needed.

Regarding barring of specific UEs, for RedCap UEs, NTN UEs, Rel 18 Cell DTX UEs, they can be barred by SIB1 based on release 17 or release 18. Thus, to avoid requesting SIB1 by these UE while OD-SIB1 cell is barring these UEs, it is important to indicate the barring status for these UEs using WUS configuration.

Regarding CellAccessRelatedInfo in SIB1, it is used to indicate cell access related information for the cell. It is important to indicate to UE by WUS configuration about the CellAccessRelatedInfo to avoid that UE requesting SIB1 and then determines from SIB1, that it is not allowed to camp on  the cellIn RAN2 Wuhan meeting this was discussed offline and online somewhat and we concluded following:

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access.

[Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information.

Offline among interested companies. Comeback in CB session (Nokia).

[Comeback session]:

[Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia.

Revisit it next meeting.

So main motivation for adding information in the WUS config is to avoid UE reselecting to cell and requesting OD-SIB1 from network only to find out that it is impossible to camp on the cell i.e. suitable cell definition is not fulfilled..

Observation 4: It is useless to request OD-SIB1 from the cell where UE cannot anyway camp on

As can be seen from the minutes from Wuhan some companies considered also access related information i.e. UAC etc.. information which is only needed at the time when UE starts RACH to establish connection in the cell.

Observation 5: Access related information is used by UE only when starting to establish connection in the cell

As the UE uses access information only after it camps on the cell and starts connection establishment there is no need or benefit to provide that information in the WUS config.

Observation 6: There is no benefit for providing access related information as UE will only apply that information when starting to establish connection and the information may have changed while UE is camping in the cell.

What is relevant for UE to know whether it is possible to camp on cell are information related to suitable cell definition i.e. PLMN, TAC, cell reservation information as well as barring information resulting in somewhat following list.

- plmn-IdentityList

- trackingAreaCode

- cellReservedForOperatorUse

- cellReservedForOtherUse

- cellReservedForFutureUse

- cellBarredATG

- cellBarred-eRedCap1Rx

- Barred-eRedCap2Rx

- cellBarredNES

- cellBarredNTN

- cellBarred-RedCap1Rx

- cellBarred-RedCap2Rx

Additionally analogously to PLMN information also NPN information is needed assuming we support OD-SIB1 for NPN. In our view we should not outrule NPN from SIB1 usage.

Observation 7: For camping in the cell UE needs to know PLMN/NPN, Cell barring as well as Cell reservation information

As one notes cellBarred present in the MIB is not among the fields above – This is not needed as anyway UE needs to read MIB from the cell when making reselection decision to even know whether OD-SIB1 procedure is to be applied in the cell- thus not reducing amount unnecessary OD-SIB1 request. We are open to include cellBarred anyway if people consider that UEs could utilize it to avoid unnecessary reselection all together.

As indicated by earlier meeting discussions it seems that overhead maybe considered as a issue to provide extra information – CellBarring/Reservation information is quite small but one cannot say the same for PLMN/NPN information. Thus it might be better to have this assistance information as optional in the message. This way NW may provide the information for all, not a single or selected cells where NW has noticed unnecessary many requests without UEs actually camping in the cell.

Thus in a summary we propose_¨

Proposal 3: Allow in the SIB1 request configuration to provide following information:

  - CellAccessRelatedInfo (includes PLMN/NPN and cell reservations)

    - if desired we can remove RANAC/CellIdentity/iab-support/gnb-ID-Length/mobile-IAB-Support/ snpn-AccessInfoList as those are not needed for camping. Then it makes sense to have different version of the IE

  - cellBarredATG

  - cellBarred-eRedCap1Rx

  - Barred-eRedCap2Rx

  - cellBarredNES

  - cellBarredNTN and corresponding VSAT barring bits

  - cellBarred-RedCap1Rx

  - cellBarred-RedCap2Rx

In the Annex A we provide a TP to realize above

# 8	Discussion on other FFS points

We would like to discuss this one:

FFS: value for maxCells, maxSIB1-Message, maxPCI

FFS: if list of cells is ARFCN&PCI or only PCI

In fact naming of “maxCellODSIB1” is misleading as it currently is basically limitation how many different frequencies one can provide on the SIBXX and thenb maxPCI-r19 limits number of cells per frequency. Frequency limitation that we should be similar to amount of frequencies used for providing reselection information i.e. SIB4 maxFreqProposal; instead of using maxCellODSIB1 – use maxFreq to align with number of frequencies in SIB4

Also naming of frequency configuration seems to imply configuration for cells thus we propose:

Proposal 4: rename OD-SIB1-CellConfig to OD-SIB1_FrequencyConfig

Also now we seem to be missing possibility for a frequency to have multiple different od-SIB1-Configs but all the cells on a frequency need to share same configuration. We assume this is not the intention thus

Proposal 5: Add another layer of SEQUENCE in the OD-SIB1-CellConfig to allow a frequency have multiple different od-SIB1-Configs – One needs to assume that it is possible that no single cell shares same configuration.

Thus this would results in something along these lines:

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxFreq))  OF OD-SIB1-FreqConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-FreqConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

cellsSharingSameConfig  SEQUENCE (SIZE(1..maxPCI-r19))  OF CommonConfiguration     OPTIONAL,        -- Need R

}

CommongConfiguration ::= {

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId     OPTIONAL,        -- Need R

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

}

Then there is also following FFS in the RRC:

FFS how does UE check is SIB1 is already provided.

Related RAN2 agreements:

RAN2 #127bis agreements:

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Related RAN1 agreements:

RAN1#120bis Agreement

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS,

- If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=123 for FR2, select the following:

- Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

Intention of this is to avoid UE sending SIB1 request preambles if UE manages to receive SIB1 even after RRC triggering SIB1 request procedure. So practically what one would need is to have indication from RRC to MAC that UE received SIB1 even before SIB1 request procedure preamble transmission and then MAC would need to stop ongoing RACH procedure triggered due to SIB1 request.

Proposal 6: Add an indication from RRC to lower layers about successful SIB1 reception and in MAC a procedure to stop RACH procedure triggered due to SIB1 request. See Annex B for TP

# 8	Conclusions

Observation 1: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

Observation 2: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

Observation 3: Legacy and NES UEs assume MIB TTI is 80ms.

Proposal 1: For switching of SIB1 transmission mode, RAN2 to consider that NES UE which has SIB1 request configuration of a cell, NES UE is expected to update MIB until SIB1 acquisition is completed.

Proposal 2: In case of NW operation in OD-SIB1 mode, UE in RRC_CONNECTED can obtain the SIBX from NWBy the NW dedicatedSytemInformationDeliverySIBX. And then UE can use OD-SIB1 request procedure to acquire SIB1.

Observation 4: It is useless to request OD-SIB1 from the cell where UE cannot anyway camp on

Observation 5: Access related information is used by UE only when starting to establish connection in the cell

Observation 6: There is no benefit for providing access related information as UE will only apply that information when starting to establish connection and the information may have changed while UE is camping in the cell.

Observation 7: For camping in the cell UE needs to know PLMN/NPN, Cell barring as well as Cell reservation information

Proposal 3: Allow in the SIB1 request configuration to provide following information:

Proposal 4: rename OD-SIB1-CellConfig to OD-SIB1_FrequencyConfig

Proposal 5: Add another layer of SEQUENCE in the OD-SIB1-CellConfig to allow a frequency have multiple different od-SIB1-Configs – One needs to assume that it is possible that no single cell shares same configuration.

Proposal 6: Add an indication from RRC to lower layers about successful SIB1 reception and in MAC a procedure to stop RACH procedure triggered due to SIB1 request. See Annex B for TP

# Annex A: TP for camping information provision in SIB1 request configuration

NOTE: CHANGE MARKS ARE ON TOP OF VERSION R2-250xxxx Running RRC CR NES Post129bis_V03.docx

#### –	SIBxx

SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

od-SIB1-CellCampingInfo-r19               OD-SIB1-CellCampingInfo-r19                      OPTIONAL         -- Need R

...

}

OD-SIB1-CellCampingInfo-r19  ::= {

plmn-IdentityInfoList               PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}             OPTIONAL,   -- Need R

cellReservedForFutureUse-r16        ENUMERATED {true}             OPTIONAL,   -- Need R

npn-IdentityInfoList-r16            NPN-IdentityInfoList-r16      OPTIONAL,    -- Need R

cellBarredATG-r18                ENUMERATED {barred, notBarred}                                     OPTIONAL,  -- Need R

cellBarredNES-r18                ENUMERATED {notBarred}                                             OPTIONAL,  -- Need R

cellBarredNTN-r17            ENUMERATED {barred, notBarred}                                         OPTIONAL,  -- Need R

cellBarredRedCap-r17           SEQUENCE {

cellBarredRedCap1Rx-r17        ENUMERATED {barred, notBarred},

cellBarredRedCap2Rx-r17        ENUMERATED {barred, notBarred}

}                                                                                                   OPTIONAL,  -- Need R

cellBarred-eRedCap-r18           SEQUENCE {

cellBarred-eRedCap1Rx-r18        ENUMERATED {barred, notBarred},

cellBarred-eRedCap2Rx-r18        ENUMERATED {barred, notBarred}

}                                                                                                   OPTIONAL,  -- Need R

cellBarredFixedVSAT-r18          ENUMERATED {barred, notBarred}                                     OPTIONAL,  -- Need R

cellBarredMobileVSAT-r18         ENUMERATED {barred, notBarred}                                     OPTIONAL,  -- Need R

cellBarred2RxXR-r18              ENUMERATED {barred}                                                OPTIONAL  -- Need R

}

OD-SIB1-Config-r19  ::= {

sib1-rsrp-ThresholdSSB-r19                       RSRP-Range                                                              OPTIONAL,   -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                                   OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}  OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL, -- Need R

offsetToCarrier-r19                          INTEGER (0..2199)                                                          OPTIONAL, -- Need R

absoluteFrequencyPointA-r19                  ARFCN-ValueNR                                                              OPTIONAL, -- Need R

p-Max-r19                                    P-Max                                                                      OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                       INTEGER (-60..50)                                                          OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19                                                     OPTIONAL  -- Need R

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                    SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SEQUENCE (SIZE (1..maxSIB1-Message)) OF SIB1-RequestResources-r19,

preambleTransMax-r19                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-ra-PreambleStartIndex-r19               INTEGER (0..63),

sib1-ra-AssociationPeriodIndex-r19           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

sib1-ra-ssb-OccasionMaskIndex-r19            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

SIBxx field descriptions

od-SIB1-CellConfigList Provides a configuration to request SIB1 for serving cell and neighbor cells.

OD-SIB1-Config field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2).

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2) used for OD-SIB1 request transmission.

od-SIB1-CellCampingInfo If present UE uses this information to determine whether it is possible to camp on the cell for which OD-SIB1 request configuration is provided as defined and may bar the cell defined in 38.304 [20]. All the fields in this use exactly same definitions as defined when used in SIB1.

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1[57] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) to be used for OD-SIB1 request transmission.

sib1-rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt to transmit OD-SIB1 request, see TS 38.321 [3].

sib1-tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

sib1-restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1.

SIB1-RequestConfig field descriptions

preambleTransMax Max number of RA preamble transmission performed for OD-SIB1 request (see TS 38.321 [3]).

ssb-perRACH-Occasion Number of SSBs per RACH occasion for OD-SIB1

sib1-RequestPeriod Periodicity of the SIB1-Request configuration in number of association periods.

sib1-RequestResources Resources for SIB1 request

SIB1-RequestResources field descriptions

sib1-ra-AssociationPeriodIndex Index of the association period in the si-RequestPeriod in which the UE can send the SIB1 request for SIB corresponding to this si-RequestResources, using the preambles indicated by ra-PreambleStartIndex and rach occasions indicated by ra-ssb-OccasionMaskIndex.

sib1-ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = ra-PreambleStartIndex + i is used for SIB1 request; For N < 1, the preamble with preamble index = ra-PreambleStartIndex is used for SIB1 request.

sib1-ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3].

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources configured in the corresponding OD-SIB1-Config, excluding preambles used for other purposes (e.g. for SI request, or SIB1 request).

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |

Editor’s note: Only parameters in R1-2501645 that are in own rows are implemented and not all listed e.g. in cell 17P or 21P.

FFS to group some parameters under subIEs like frequencyInfoUL

FFS to separate IE OD-SIB1 as own IE, for review purposes it is here now.

FFS: value for maxCells, maxSIB1-Message, maxPCI

FFS: optionality of the parameters as there was no input on this

FFS: if list of cells is ARFCN&PCI or only PCI

# Annex B – on FFS how does UE check is SIB1 is already provided.

# ----------RRC----------------

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if UE indicated to trigger the lower layer to initiate the Random Access procedure for on demand SIB1 request without receiving the acknowledgement from lower layers:

2>	indicate lower layers of successful SIB1 reception:

1>	if the access is for NTN:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

3>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	if the UE is a fixed VSAT UE and the cellBarredFixedVSAT in the acquired SIB1 is set to barred or the cellBarredFixedVSAT is not included in the acquired SIB1, or

3>	if the UE is a mobile VSAT UE and the cellBarredMobileVSAT in the acquired SIB1 is set to barred or the cellBarredMobileVSAT is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

# ------------------ unchanged parts omitted from rest of the section ------------------------------

# ------------MAC-------------

### 5.1.6	Completion of the Random Access procedure

Upon completion of the Random Access procedure, the MAC entity shall:

1>	discard any explicitly signalled contention-free Random Access Resources for 2-step RA type and 4-step RA type except the 4-step RA type contention-free Random Access Resources for beam failure recovery request, if any;

1>	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer and the MSGA buffer.

Upon successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall:

1>	indicate the successful completion of the Random Access procedure to the upper layers.

Upon successful completion of the Random Access procedure initiated for LTM cell switch, the MAC entity shall:

1>	indicate the successful completion of the LTM cell switch to upper layers.

Upon reception of successful SIB1 reception indication from upper layer, the MAC entity shall stop, if any, ongoing Random Access procedure triggered by SIB1 request.