3GPP TSG-RAN WG2 Meeting #130	R2-2503902

St Julian, Malta, May.19th – 23rd, 2025

Agenda item:		8.4.3

Source:		Lenovo

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and agreed, the objectives related to RRM measurement for RRC IDLE/INACTIVE modes of the work items are given as following.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the previous RAN2 meetings [2]-[7], RRM measurement relaxation and offloading in RRC_INACTIVE/IDLE state has been discussed and following agreements have been reached:

RAN2#126 Agreements: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127 Agreements： - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Agreements: Working assumption: - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 Agreements: - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbor cell RRM measurement relaxation. RAN2#129 Agreements： - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds. RAN2#129bis Agreements: - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progresses, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR measuring SSB threshold or both. - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. - The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. - How to define LP-RSRP and LP-RSRQ is up to RAN1. - The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

This contribution aims to present our views on the RRM measurement relaxation and offloading from MR to LR for UEs in RRC_IDLE/INACTIVE mode.

- Discussion

According to the latest WID, both RAN4 and RAN2 are required to specify RRM measurement relaxation of UE MR for both serving cell and neighboring cell measurements, and UE serving cell RRM measurement offloading from MR to LR, including the necessary conditions. It has been approved by RAN4 that the ‘fully offloading case’ and ‘relaxed case b’ are considered as feasible while other cases are pending for further study, the details can be seen as below:

Approved cases on RRM measurement relaxation/offloading.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | OFF | OFF: FFS the conditions and details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

FFS cases on RRM measurement relaxation/offloading.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | OFF | ON |
| #4 Relaxed case c | OFF | ON, FFS the condition and the details | ON |
| #5 Extra Relaxed case | On with relaxation measurement | Off | OFF |
| #6 Extra Relaxed case | On with relaxation measurement | On with relaxation measurement | OFF |

RAN4 is required to define the RRM measurement relaxation/offloading requirements while RAN2 focuses on specifying the relaxation/offloading criterion for UEs supporting LP-WUS. Based on RAN4’ conclusion, it is suggested RAN2 gives priority to discuss the entry/exit conditions for the following offloading/relaxation cases:

- Case1-Fully offloading case：serving cell measurement is fully offloaded from MR to LR, while MR performs no measurement on serving cell, FFS on neighboring cell measurement.

- Case3-Relaxed case b: serving cell measurement and neighboring cell measurement are relaxed on MR, while LR also performs serving cell measurement, this case is also called as partial offloading case.

Observation 1: RAN4 has approved case1 on RRM measurement offloading and case 3 on RRM measurement relaxation as below:

- Case 1: serving cell measurement is fully offloaded from MR to LR while MR performs no measurement on serving cell.

- Case 3: serving cell measurement and neighboring cell measurement are relaxed on MR while LR also performs the serving cell measurement.

2.1 Serving cell measurement relaxation

As defined by RAN4, for case3 on serving cell measurement relaxation, MR performs relaxed measurement on serving cell and neighboring cell while LR is ON and performs the serving cell measurement, which can also be defined as the partial offloading case. Regarding to the entry conditions for the relaxation case, in RAN2 #129 meeting [6], it has been agreed that the entry conditions for serving cell RRM relaxation can include both MR and LR measurements, if LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

Compared to fully offloading serving cell measurement to the LR, both MR and LR can perform serving cell measurement for the relaxation case, the entry conditions can be defined to be less stringent than the fully offloading case, for example, the UE needs to at least stay in the coverage of LP-WUS, then UE may partially offload the measurement from MR to LR. But only in the case of the coverage performances are good enough, i.e., more stringent requirements than serving cell measurement relaxation, MR may fully offload the serving cell measurement to LR. Therefore, we proposed that for the serving cell measurement relaxation case, the MR and LR thresholds for entry conditions need to be configured lower than the serving cell measurement offloading case.

Proposal 1: The MR and LR thresholds on serving cell measurement relaxation entry conditions are configured lower than the thresholds on serving cell measurement offloading.

2.2 Low mobility criterion

As defined in RAN4#114 meeting, the scaling factor of MR RRM relaxation for serving cell/neighboring cell is 16, the requirement is quite different with Rel-16 neighboring cell measurement relaxation (e.g., the scaling factor is 2-4), which means that separate conditions for Rel-19 neighboring cell measurement relaxation for LP-WUS UE needs to be defined. In RAN2#127bis meeting [4], RAN2 assumed that for neighboring cell measurement relaxation for UEs capable of LP-WUS, MR based criterion on ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. In our understanding, ‘low mobility’ criterion guarantees that the UE has low possibility to perform the cell reselection, the neighboring cell measurement can then be relaxed. Same principle can still be applied in LP-WUS. Since for LP-WUS UE, potential cell switch due to high mobility may further lead to the UE to switch between MR and LR more frequently, which will reduce the power saving gain. It is suggested that ‘low mobility’ criterion can be used in Rel-19 neighboring cell measurement relaxation for UEs capable of LP-WUS in addition to ‘not at cell edge’ criterion. Network may configure the low mobility criterion optionally in addition to ‘not-at-cell-edge’ criterion.

Proposal 2: (38304-6) RAN2 to agree that ‘low mobility’ criterion can be used for Rel-19 neighboring cell measurement relaxation for UEs capable of LP-WUS.

Furthermore, in last RAN2#129bis meeting [7], RAN2 has discussed and agreed to merge the entry/exit condition for serving cell RRM measurement relaxation and Rel-19 neighboring cell RRM measurement relaxation (high-priority frequency is separate discussion). We understood that merging those two conditions which refers to UE can trigger both serving cell measurement relaxation and neighboring cell measurement relaxation simultaneously by using one unified condition, e.g., based on the same MR and LR threshold (if configured). As further clarified during online discussion, this principle currently applied to the criterion with ‘not at cell edge’, whether and how to apply to other criterions if supported is another issue. Based on our proposal 5, if ‘low mobility’ criterion has been applied to Rel-19 neighboring cell measurement relaxation, we understood than same criterion also needs to be applied to the serving cell measurement relaxation. It is reasonable since there is a relatively high possibility for UE with high mobility to perform cell reselection while needs normal measurement for both serving cell and neighboring cell.

Proposal 3: (38304-6) If ‘low mobility’ criterion has been agreed to apply for Rel-19 neighboring cell measurement relaxation, same criterion also needs to be applied to the serving cell measurement relaxation.

Additionally, for serving measurement offloading case, current agreements defined that the entry conditions for serving cell measurement offloading are at least MR result greater than a certain RSRP threshold, and LR result could also be considered. The agreed entry condition aims to guarantee the UE with a good coverage performance for both MR and LR. However, the serving measurement offloading aims to achieve UE power saving gain but also needs to limit the impacts on mobility performances as much as possible. For this purpose, the UE needs to at least stay in a relatively stable status and have a low possibility of performing cell (re)selection, then the serving measurement offloading from MR to LR could be considered, since LR cannot perform neighboring cell measurement. For example, UE with high-mobility camps on small cell may not be suitable to perform offloading even with large measured RSRP value. To achieve this, the low mobility criterion may also be applied to the serving cell measurement offloading case, which can be defined as the difference of RSRP value being lower than a (pre-)configured threshold within a time duration, and MR measurements are used for the entry condition evaluation for low mobility criterion.

Proposal 4: (38304-6) RAN2 to discuss additional entry condition for serving cell measurement offloading case, e.g., low mobility conditions evaluated by the MR.

2.3 Relationship between LP-WUS monitoring and RRM relaxation/offloading

In the last RAN2#129bis meeting [7], RAN2 has discussed the direction on merging the entry/exit condition for serving cell RRM measurement offloading or serving cell RRM measurement relaxation or neighboring cell RRM measurement relaxation and LP-WUS monitoring to reduce the number of total thresholds for LP-WUS as possible, which can be further divided as following sub-directions:

- Direction a): Merge LP-WUS monitoring and serving cell offloading.

- Direction b): Merge LP-WUS monitoring and Rel-19 RRM relaxation.

In our view, the LP-WUS monitoring targets to receive paging by detecting LP-WUS, while the RRM measurement relaxation/offloading targets to offload the RRM measurement to LR by measuring e.g., LP-SS, etc., which are two totally different procedures and may correspond to different measurement requirements. If RAN2 expects to merge those two conditions, then at least the measurement requirements should be identified as the same, which may need further confirmation. However, if one of above directions is expected from RAN2 to further reduce the number of thresholds, we prefer to merge the LP-WUS monitoring and Rel-19 RRM relaxation, since serving cell offloading case may have more stringent conditions than Rel-19 RRM relaxation. Additionally, merging the LP-WUS monitoring entry/exit conditions with serving cell offloading may limit the usage of LP-WUS to some extent since LP-WUS may be reachable even outside the fully offloading area in the cell. Therefore, we propose that RAN2 to consider merging the entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation. As RAN2 currently only defined ‘not-at-cell edge’ criteria as the entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation, we understood that the merge condition refers to ‘not-at-cell-edge’ criterial only in this stage. Since whether ‘low mobility criteria’ needs to be applied to Rel-19 RRM relaxation or not is another issue to be solved, if RAN2 agreed that the ‘low mobility criteria’ is needed for RRM, then it can be further discussed such criterial should also be applied to LP-WUS monitoring if those two conditions are merged.

Proposal 5: (38304-4) RAN2 to consider merge the entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation at least for ‘not-at-cell edge’ criteria.

2.4 Transition between case1 and case 3

We assumed that in previous RAN2 discussions, the entry conditions defined the criterion for UEs to start performing the serving cell measurement offloading or relaxation from the legacy RRM measurement mode. While the exit conditions defined the criterion for UEs to stop performing the serving cell measurement offloading or relaxation and fallback to legacy RRM measurement mode. Since RAN2 has agreed that it is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. In the case of both serving cell relaxation and offloading are configured in one cell, whether the UE can perform the transition between the offloading case and relaxation case directly can be further discussed. Specifically:

- From case1 to case3, in the case of UE is performing the serving cell measurement offloading, since it has been agreed that UE stops to perform the offloading based on LR measurement result, while the entry conditions for relaxation case should include both MR and LR measurement results. According to current agreements, UE needs to fallback to legacy RRM measurement mode and wakes up MR firstly, and then starts the relaxation based on MR and LR measurement results if configured, if no addition conditions are defined. Otherwise, it can also be assumed that only LR measurement result is considered as the entry condition for UEs to transit from serving cell measurement offloading to relaxation case directly, it is suggested RAN2 to have discussion on the issue.

- From case 3 to case1, in the case of UE is performing the serving cell measurement relaxation, both MR and LR measurement results are available, in this case, if MR threshold and LR threshold (if configured) for RRM measurement offloading are satisfied, UE can transit from the RRM measurement relaxation to offloading directly without additional defined conditions. NW may guarantee that thresholds for offloading is higher than the thresholds for relaxation.

Proposal 6: RAN2 to discuss whether UE can transit from serving cell measurement offloading (i.e., case1) to serving cell measurement relaxation (i.e., case 3) directly based on the LR measurement result only.

Proposal 7: No additional conditions is needed for UE to transit from serving cell measurement relaxation (i.e., case3) to serving cell measurement offloading (i.e., case1) directly.

- Conclusion

Observation 1: RAN4 has approved case1 on RRM measurement offloading and case3 on RRM measurement relaxation as below:

- Case 1: serving cell measurement is fully offloaded from MR to LR while MR performs no measurement on serving cell.

- Case 3: serving cell measurement and neighboring cell measurement are relaxed on MR while LR also performs the serving cell measurement.

Proposal 1: The MR and LR thresholds on serving cell measurement relaxation entry conditions are configured lower than the thresholds on serving cell measurement offloading.

Proposal 2: (38304-6) RAN2 to agree the ‘low mobility’ criterion can be used for Rel-19 neighboring cell measurement relaxation for UEs capable of LP-WUS.

Proposal 3: (38304-6) If ‘low mobility’ criterion has been agreed to apply for Rel-19 neighboring cell measurement relaxation, same criterion also needs to be applied to the serving cell measurement relaxation.

Proposal 4: (38304-6) RAN2 to discuss additional entry condition for serving cell measurement offloading case, e.g., low mobility conditions evaluated by the MR.

Proposal 5: (38304-4) RAN2 to consider merge the entry/exit conditions of LP-WUS monitoring and Rel-19 RRM relaxation at least for ‘not-at-cell edge’ criteria.

Proposal 6: RAN2 to discuss whether UE can transit from serving cell measurement offloading (i.e., case1) to serving cell measurement relaxation (i.e., case 3) directly based on the LR measurement result only.

Proposal 7: No additional conditions is needed for UE to transit from serving cell measurement relaxation (i.e., case3) to serving cell measurement offloading (i.e., case1) directly.

- References

3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

3GPP RAN2#126, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127bis, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#128, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#129, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#129bis, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.