3GPP TSG-RAN WG2 Meeting #130						R2-2503904

St Julian, Malta, May. 19th – 23rd, 2025

Agenda item:		8.2.3

Source:			Lenovo

Title:		Discussion on random access for Ambient IoT

Document for:	 	Discussion and Decision

Introduction

In RAN#107 meeting the work item for Ambient IoT was revised and approved in [1]. In last RAN2 meeting, A-IoT random access procedure was discussed and reached following agreements:

Agreements on msg1 - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. Agreements on new R2D message - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. Agreements on msg 2 - A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. - Same Msg2 format is used for initial transmission and retransmission of Msg2. Agreements on NACK - For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). - For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging - For CFRA, NACK feedback and re-access is not supported. FFS how to achieve - FFS on end of procedure

In this contribution, random access procedure related to msg content/format, and feedback mechanism are discussed, based on the output of TR 38.769 [2], WID scope and RAN2 agreements above.

Discussion

A-IoT Msg0

R2D trigger message

In last RAN1 meeting, it was agreed in the following that related to the R2D trigger message

Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact. - FFS: how to define the end of the R2D transmission in 9.4.1. Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

It was agreed in last RAN2 meeting that a new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. According to the RAN1 agreement, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. So RAN1 concludes not to use L1 signaling but using MAC layer signaling to indicate Msg1 resource.

Observation 1: RAN1 concludes not to use L1 signaling for any scheduling information related to resource allocation, but indicated by higher-layer signaling.

Proposal 1: Confirm a new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources.

According to above RAN1 agreement and RAN2 agreement in last meeting, the R2D trigger message is used to indicate the timing information of a set of triggered Msg1 resources, and may contain following information:

- 1 bit to indicate X value (X=1 or X=2) time domain resource(s) for Msg1 transmission(s)

- Toffset1: the starting time for the first Msg1 time domain resource, if explicitly indicated

- Toffset2: the starting time for the second Msg1 time domain resource, derived based on the starting time of the first Msg1 time domain resource plus Toffset2, if explicitly indicated

In RAN1, it is still FFS whether above timing offset (e.g. Toffset1 and Toffset2) is pre-defined or indicated implicitly or explicitly, and also the value of the timing offset is FFS. RAN2 cannot discuss the R2D trigger message format and size yet, and it is better to wait for RAN1 further progress on this aspect to design the R2D trigger message format and size.

Proposal 2: R2D trigger message may contain following information according to RAN1 agreement, the discussion on format and the size of the R2D trigger message can wait for RAN1 further progress.

- 1 bit to indicate X value (X=1 or X=2) time domain resource(s) for Msg1 transmission(s)

- Toffset1: the starting time for the first Msg1 time domain resource, if explicitly indicated

- Toffset2: the starting time for the second Msg1 time domain resource, derived based on the starting time of the first Msg1 time domain resource plus Toffset2, if explicitly indicated

Another unclear question is whether we support interleaved R2D trigger message with random access procedure. For example, following figures provided in [3] illustrate relationship between Msg1 and Msg2, we noticed that in the example 3, there are multiple R2D trigger messages (QueryRep in the figure 1) before the first Msg2, which means random access procedure is interleaved with R2D trigger message. We think this case should be firstly discussed and determine whether this case is supported or not. Since if this case is supported, it will impact random access design in many aspects e.g. contention resolution determination, how to determine NACK indication, end of the procedure etc.

Figure 1 Examples of the Msg1 and Msg2 relationship [3]

Proposal 3: RAN2 is suggested to discuss whether to support interleaved R2D trigger message transmission before Msg2 transmission (acc. to example 3 in figure 1).

A-IoT Msg1

As discussed in last section, RAN1 agreed X=1 and X=2, and 1 bit is used to indicate this value. But RAN1 has not decided to include this 1-bit indication in which message. If this 1-bit indication is included in paging message, then X value corresponding to all R2D trigger message in the same paging round is same. On the other hand, if this 1-bit indication is included in R2D trigger message, then X value corresponding to each R2D trigger message could be different, see examples in figure 2.

Figure 2 1 bit X value indication in paging message (a) or in R2D trigger message (b)

Observation 2: Depending on 1-bit X value is indicated in which message

- If in paging message, X value corresponding to all R2D trigger message in the same paging round is same

- If in R2D trigger message, X value corresponding to each R2D trigger message in the same paging round could be different.

Whether 1-bit indication for X value is in paging message or in R2D trigger message may wait for RAN1 further progress. Under these two options, device behavior for Msg1 transmission may have a little difference. Assume there are totally N Msg1 resources and A-IoT device randomly selects value M among 0~N-1. X is the time domain resource number and Y is the frequency domain resource number.

If 1-bit indication is in paging message, the device obtains X=X0 value from the paging message. Upon receiving each R2D trigger message, the device determines whether M value is less than X0*Y. If yes, the device will perform access and transmit Msg1 in the (M-value)th resource. If not, the device will update M value as M=M-X0*Y.

If 1-bit indication is in R2D trigger message, upon receiving each R2D trigger message, the device obtains X=X1 value from the R2D trigger message, and determines whether M value is less than X1*Y. If yes, the device will perform access and transmit Msg1 in the (M-value)th resource. If not, the device will update M value as M=M-X1*Y.

Proposal 4: The device behavior for Msg1 transmission has minor difference, consider 1-bit X value indication is in paging message or R2D trigger message:

- In paging message, the device obtains X value once, and determines access occasion and updates counter based on the obtained X value

- In R2D trigger message, the device obtains X value for each R2D trigger message, and determines access occasion and updates counter based on the obtained X value corresponding to each R2D trigger message.

A-IoT Msg2

Msg2 Multiplexing

In last RAN2 meeting, it was agreed that one Msg2 message contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices, i.e. Msg2 multiplexing is supported. So it is possible that the reader responses Msg2 in once for all Msg1 resources corresponding to one R2D trigger message which is so called common Msg2, or the reader responses Msg2 separately for each Msg1 resource corresponding to one R2D trigger message which is so called separate Msg2. On the other hand, since it was agreed in RAN1 that X value could be 1 or 2, when X=2, the reader can also respond Msg2 corresponding to each time domain resource, which so called partial Msg2. Above three cases are illustrated in the following figure 3.

Figure 3 Three Msg2 types: (a) Separate Msg2 (b) Partial Msg2 (c) Common Msg2

Whether to support all three cases in above figure will impact random access design e.g. contention resolution determination for the device. So, we suggest RAN2 to discuss above three cases before other Msg2 issues.

Proposal 5: RAN2 to discuss whether all following three Msg2 cases are supported

- Separate Msg2

- Partial Msg2

- Common Msg2

Contention resolution determination for A-IoT device

According to the discussion in above Msg2 multiplexing type, there may have three Msg2 response types. This may introduce complexity for A-IoT device to receive Msg2 and determine contention resolution success/failure. For example, for separate or partial Msg2 response case, the device may need to determine whether contention resolution is failure after receiving all Msg2 messages corresponding to this R2D trigger message. And for common Msg2, the device determines whether contention resolution is failure by one Msg2 message. It is better to have a unified solution to cover all possible Msg2 multiplexing cases to avoid the complexity for A-IoT device.

For separate Msg2 and partial Msg2

- Success: the device receives matched RN16 in one Msg2 among all Msg2(s) corresponding to the R2D trigger message

- Failure: the device does not receive matched RN16 in any Msg2 corresponding to the R2D trigger message, or does not receive any Msg2 corresponding to the R2D trigger message

- Miss: the device does not receive any Msg2 corresponding to the R2D trigger message

For common Msg2

- Success: the device receives matched RN16 in the Msg2 corresponding to the R2D trigger message

- Failure: the device does not receive matched RN16 in the Msg2 corresponding to the R2D trigger message, or does not receive the Msg2 corresponding to the R2D trigger message

- Miss: the device does not receive the Msg2 corresponding to the R2D trigger message

So, a possible unified solution is A-IoT device determines contention resolution success only when it receives matched RN16 in all Msg2 messages corresponding to the R2D trigger message. Otherwise, the device determines contention resolution is not success (failure or miss) if it does not receive matched RN16 in all Msg2 messages or does not receive any Msg2 messages corresponding to the R2D trigger message.

How to determine the R2D trigger message and corresponding Msg2(s) needs further discussion. One possible solution is to label the corresponding Msg2. For example, NDI-like bit can be included in both R2D trigger message and corresponding Msg2(s). If the bit value is same, then they are corresponded. Another possible solution is to determine all Msg2(s) before subsequent R2D transmission is corresponding to the R2D trigger message. Here the subsequent R2D transmission could be R2D trigger message or paging message or command message.

Proposal 6: A-IoT device determines contention resolution success only when it received matched RN16 in all Msg2(s) corresponding to the R2D trigger message.

Proposal 7: A-IoT device determines corresponding Msg2(s) in following two options

- Option 1: Msg2 includes same flag as in R2D trigger message

- Option 2: All Msg2(s) before subsequent R2D transmission. Here R2D transmission could be R2D trigger message or paging message or command message.

RN16 Collision Problem

In last RAN2 meeting, it was agreed that one Msg2 message contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. These echoed random ID(s) are for those A-IoT devices that has successful contention resolution in specific Msg1 resource. And it is possible that two or more echoed random ID(s) corresponding to different access occasions/Msg1 resources are same in one Msg2 message, which means RN16 collides in the Msg2.

In last RAN2 meeting, this issue was discussed and there has comments that the probability of RN16 collision in Msg2 is very low. We calculate the collision probability as in the below:

Assuming there have N RN16 that responded in one Msg2. Then there are totally (216)N different combinations since each RN16 has 216 possible values. For the first RN16, there totally have 216 possible values. For the second RN16, to different with the first RN16, there totally have 216-1 possible values. And for the Nth RN16, to different with all the other N-1 RN16, there totally have 216-(N-1) possible values. So, the probability P0 that all N RN16 are different in one Msg2 is as following

P0=216*216-1*216-2*…*(216-N+1)(216)N

Then in one Msg2, the probability P for at least two RN16 are same is as following

P=1-P0=1-216*216-1*216-2*…*(216-N+1)(216)N

As RAN1 has agreed maximally X=2 in time domain, and assume Y=8 in frequency domain, then possibly the maximum probability for P is when N=X*Y=16, then P=1.8‰. Consider such probability, RN16 collision in one Msg2 does not happen frequently and could rely on the reader implementation to solve this problem, e.g. the reader does not respond collided RN16(s) in Msg2, and A-IoT device(s) will regard this as contention resolution failure and perform re-access. This means whenever inventory 1000 A-IoT device(s), less than 2 A-IoT device(s) have to perform unnecessary re-access because of RN16 collision problem.

The collision probability is also related to the discussion in section 2.1, that whether support interleaved R2D trigger message during random access procedure. If supported, e.g. in the example 3 of Figure 1, the probability needs to consider all Msg1 resource among three R2D trigger message, i.e. N=X*Y*3=48, and in this case P=1.7%.

Observation 3: The maximum probability that at least two RN16 is same in one Msg2 is 1.8‰, assuming X=2 and Y=8. The maximum probability considering three R2D trigger messages is 1.7%.

Proposal 8: If there has consensus that the observed RN16 collision probability in one Msg2 (≤1 .8‰, or ≤1.7% considering three R2D trigger messages) is low enough, the RN16 collision in one Msg2 can be seen as a rare case and rely on the reader’s implementation and A-IoT device re-access to solve the problem.

On the other hand, during the discussion for AS ID assignment for CBRA, there had the following agreement

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID.

One motivation to assign a new AS ID by the reader is for the case two echoed random ID(s) are same and collide and the random ID(s) cannot be used as AS ID directly. On the other hand, RAN2 agreed to use Msg2 to assign new AS ID in last meeting. So, if there has no consensus that the RN16 collision probability in one Msg2 (≤1.8‰) is low enough i.e. if RAN2 assumes RN16 collision may happen in one Msg2, AS ID assignment in the Msg2 for collided RN16 needs to be identified by the device.

Observation 4: RAN2 agreed for CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. One motivation of the reader to assign a new AS ID is that RN16 collision problem may happen in one Msg2 message.

Proposal 9: If there assumes RN16 collision may happen in one A-IoT Msg2 message, AS ID assignment in the Msg2 for collided RN16(s) needs to be identified by the device.

How the A-IoT device identify its RN16 among multiple same RN16(s) could be further discussed, if above proposal is agreed. To our understanding, the simplest solution is to introduce additional parameter from Msg1 resource aspect, since anyway the collided RN16 are transmitted in the different Msg1 resource. One option is that the A-IoT device determines Msg1 resource index based on the order of the allocated Msg1 resource in paging message. And another option is that when allocating Msg1 resource in paging message, the reader also indicates a resource index explicitly that associated with each Msg1 resource. Then in Msg2 message, when the RN16 collision happens and needs to assign new AS ID, besides echoed RN16, a resource index is also indicated for A-IoT device to determine which AS ID is assigned for itself.

Proposal 10: Introduce Msg1 resource index indication in Msg2 when assigning new AS ID for A-IoT device when RN16 collision happens in one A-IoT Msg2. Two options to determine the Msg1 resource index are as following:

- Option 1: A-IoT device determines Msg1 resource index based on the order of the allocated Msg1 resource in paging message.

- Option 2: A-IoT device determines Msg1 resource index based on the indicated resource index by the reader in paging message.

AS ID field in Msg2 for multiplexing case

In last meeting, it was agreed that A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices i.e. multiplexing case is supported in Msg2. On the other hand, it was agreed that for CBRA, Msg 2 is used for AS ID assignment. In most cases, RN16 can be used as AS ID directly, this could be done implicitly when Msg2 contains echoed RN16 only. For the case when RN16 collides with other RN16 in the Msg2, or collides with existing AS ID, the reader will include a new AS ID, and the reader needs to also indicate there includes a new AS ID to A-IoT device for decoding.

Proposal 11: A-IoT Msg2 contains one or multiple Msg1 response(s), each Msg1 response may contain following information

- 1 bit to indicate the presence of new AS ID (mandatory)

- Echoed RN16 (mandatory)

- New AS ID (optional)

Proposal 12: A-IoT device determines to use RN16 as AS ID if 1 bit indicates no new AS ID is included in matched Msg1 response in Msg2.

Proposal 13: A-IoT device determines to use new AS ID as AS ID if 1 bit indicates new AS ID is included in matched Msg1 response in Msg2.

Msg3 scheduling

A-IoT Msg2 may indicate the time-frequency resource information that shall be used for Msg3. But how this information is exactly indicated depends on RAN1 progress. During last two meetings, RAN1 has reached following agreement for Msg3 resource scheduling:

Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device Agreement Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

However, the Msg3 scheduling is still under discussion and not finalized yet. For example, the resource relationship between Msg2 and Msg3 is not concluded yet in RAN1. The discussion on time-frequency resource information included in A-IoT Msg2 for A-IoT Msg3 still needs to wait for RAN1 further progress.

Proposal 14: The discussion on time-frequency resource information included in A-IoT Msg2 for A-IoT Msg3 waits for RAN1 further progress.

Msg2 Monitoring Window

After sending Msg1, A-IoT device will monitor Msg2. The discussion on how the device monitoring Msg2 e.g. by the timer or in a specific time window could be deferred until RAN1 has finalized corresponding discussion. In last RAN1 meeting, there only had following agreement for R2D transmission monitoring and has not finalized on this aspect.

Agreement A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device.

Proposal 15: The discussion on Msg2 monitoring window waits for RAN1’s further progress.

NACK Indication

NACK indication for CBRA

In last meeting it was agreed for CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. And for msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer.  FFS whether subsequent R2D message is R2D trigger message or paging. So before the subsequent R2D message, if device receives NACK, the device will perform re-access. Otherwise, the device determines Msg3 transmission is successful.

If the subsequent R2D message is R2D trigger message, this implies the reader cannot send R2D trigger message before the device finishes Msg3 transmission. So, if the following example procedure is supported,  it is not work that subsequent R2D transmission is R2D trigger message. On the other hand, since the re-access anyway happens after the subsequent paging message, it seems no advantage to use subsequent R2D trigger message for NACK indication reception.

Figure 4 Example of the Msg1 and Msg2 relationship [3]

Observation 5: If subsequent R2D message for NACK indication reception is R2D trigger message, Msg1/2 scheduling flexibility is restricted.

On the other hand, if the subsequent R2D message is paging message, all possible Msg1/2 scheduling cases are allowed. And considering the re-access anyway happens after the paging message, it is natural to use subsequent paging message for NACK indication reception.

Proposal 16: For A-IoT Msg3, rely on whether the device receives NACK indication before subsequent paging message to determine re-access.

Addressing for NACK indication in CBRA

NACK indication is used to indicate A-IoT device to perform re-access, when Msg3 transmission cannot succeed by retransmitting Msg2. It is possible that NACK indication for specific device is sent after multiple Msg3 transmissions from multiple A-IoT devices. On the other hand, it is also possible there have multiplexed NACK indications for multiple A-IoT devices. So, it is necessary to address the NACK indication is for which A-IoT device.

It was agreed during last two meetings that from RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”. And AS ID is applied for Inventory + command case. So, for inventory + command case, i.e. the reader is indicated by CN that the service type is inventory + command, AS ID will be applied for each A-IoT device, and AS ID can be used for NACK indication for addressing the A-IoT device.

Proposal 17: NACK indication contains one or multiple NACKs for one or multiple A-IoT devices.

Proposal 18: When the reader is indicated by CN that the service type is inventory + command, AS ID is used for NACK indication for addressing the A-IoT device(s).

On the other hand, for inventory only case, i.e. the reader is indicated by CN that the service type is inventory only, AS ID will not be applied, and RN16 can be used. It is possible that there has RN16 collision among multiple A-IoT devices. We could rely on reader implementation to avoid this case e.g. the reader will not respond the RN16 if it detects there has collision. Or we could rely on device re-access to solve the problem although the re-access may be unnecessary. For example, if NACK addressed for specific RN16 is target for multiple A-IoT devices, then all these devices will perform re-access. Among them, only re-access from one device is necessary but from system point of view, it still works.

Proposal 19: When the reader is indicated by CN that the service type is inventory only, RN16 is used for NACK indication for addressing the A-IoT device(s).

NACK indication for CFRA

In last meeting, for CFRA, it was agreed NACK feedback and re-access is not supported.  FFS how to achieve. Compared with CBRA, CFRA has no random access phase and has only two phases: 1) inventory response; 2) command response if there has followed command. CFRA procedure is illustrated in the following figure 5.

Figure 5 CFRA procedure

One point is not clear yet for CFRA is that whether R2D trigger message is still applied for indicating the start timing of inventory response. In R19, since there has only one device, the timing indication of the inventory response could be indicated in the paging message, and then R2D trigger message seems not so necessary. But on the other hand, if there has no R2D trigger message, the device processing will be different as for CBRA, where the device needs to process R2D trigger message to determine the start of the Msg1 resource. Further, to consider multiple devices scenario in future release, using R2D trigger message(s) to indicate the start of the inventory response from multiple devices is more suitable and more flexible. On the other hand, support R2D trigger message is beneficial for the unified MAC PDU format design for paging message. So, we slightly prefer to also support R2D trigger message for CFRA.

Proposal 20: R2D trigger message is also supported for CFRA procedure, which is used to indicate the start of inventory response resource.

If the procedure fails during inventory response phase, i.e. the reader does not receive inventory response from the device correctly, the reader has two choices: 1. The reader can retransmit paging and let the device to send inventory response again; or 2. The reader can indicate to CN the inventory failure and rely on CN implementation. In the first case, same paging will be transmitted again and in the second case, CN may trigger a new paging. No matter for which case, A-IoT device will receive the paging and respond to the paging message.

Figure 6 Inventory response failure for CFRA

On the other hand, if the procedure fails during command response phase, the (segment) message can be retransmitted by retransmitting command or retransmission scheduling with offset by the reader. This is exactly same as command response phase of CBRA.

Proposal 21: The reader may transmit same paging message again to schedule retransmission of inventory response from A-IoT device.

Proposal 22: The A-IoT device always responds to the paging which target for itself and indicate CFRA, ignores transaction id if transaction id is included.

If transaction id is anyway ignored by the device for CFRA case, it could be excluded to further save the signaling overhead. And the message format difference is not a problem, since there will be 1 bit to indicate CBRA or CFRA and different format could be defined.

Proposal 23: Transaction id is not included in paging message for CFRA.

Conclusion

In this contribution we have discussed A-IoT random access procedure and made the following observations and proposals:

A-IoT Msg0

R2D trigger message

Observation 1: RAN1 concludes not to use L1 signaling for any scheduling information related to resource allocation, but indicated by higher-layer signaling.

Proposal 1: Confirm a new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources.

Proposal 2: R2D trigger message may contain following information according to RAN1 agreement, the discussion on format and the size of the R2D trigger message can wait for RAN1 further progress.

- 1 bit to indicate X value (X=1 or X=2) time domain resource(s) for Msg1 transmission(s)

- Toffset1: the starting time for the first Msg1 time domain resource, if explicitly indicated

- Toffset2: the starting time for the second Msg1 time domain resource, derived based on the starting time of the first Msg1 time domain resource plus Toffset2, if explicitly indicated

Proposal 3: RAN2 is suggested to discuss whether to support interleaved R2D trigger message transmission before Msg2 transmission (acc. to example 3 in figure 1).

A-IoT Msg1

Observation 2: Depending on 1-bit X value is indicated in which message

- If in paging message, X value corresponding to all R2D trigger message in the same paging round is same

- If in R2D trigger message, X value corresponding to each R2D trigger message in the same paging round could be different.

Proposal 4: The device behavior for Msg1 transmission has minor difference, consider 1-bit X value indication is in paging message or R2D trigger message:

- In paging message, the device obtains X value once, and determines access and updates counter based on the obtained X value

- In R2D trigger message, the device obtains X value for each R2D trigger message, and determines access occasion and updates counter based on the obtained X value corresponding to each R2D trigger message.

A-IoT Msg2

Msg2 Multiplexing

Proposal 5: RAN2 to discuss whether all following three Msg2 cases are supported

- Separate Msg2

- Partial Msg2

- Common Msg2

Contention resolution determination for A-IoT device

Proposal 6: A-IoT device determines contention resolution success only when it received matched RN16 in all Msg2(s) corresponding to the R2D trigger message.

Proposal 7: A-IoT device determines corresponding Msg2(s) in following two options

- Option 1: Msg2 includes same flag as in R2D trigger message

- Option 2: All Msg2(s) before subsequent R2D transmission. Here R2D transmission could be R2D trigger message or paging message.

RN16 Collision Problem

Observation 3: The maximum probability that at least two RN16 is same in one Msg2 is 1.8‰, assuming X=2 and Y=8. The maximum probability considering three R2D trigger messages is 1.7%.

Proposal 8: If there has consensus that the observed RN16 collision probability in one Msg2 (≤1.8‰, or ≤1.7% considering three R2D trigger messages) is low enough, the RN16 collision in one Msg2 can be seen as a rare case and rely on the reader’s implementation and A-IoT device re-access to solve the problem.

Observation 4: RAN2 agreed for CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. One motivation of the reader to assign a new AS ID is when RN16 collision problem may happen in one Msg2 message.

Proposal 9: If there assumes RN16 collision may happen in one A-IoT Msg2 message, AS ID assignment in the Msg2 for collided RN16(s) needs to be identified by the device.

Proposal 10: Introduce Msg1 resource index indication in Msg2 when assigning new AS ID for A-IoT device when RN16 collision happens in one A-IoT Msg2. Two options to determine the Msg1 resource index are as following:

- Option 1: A-IoT device determines Msg1 resource index based on the order of the allocated Msg1 resource in paging message.

- Option 2: A-IoT device determines Msg1 resource index based on the indicated resource index by the reader in paging message.

AS ID field in Msg2 for multiplexing case

Proposal 11: A-IoT Msg2 contains one or multiple Msg1 response(s), each Msg1 response may contain following information

- 1 bit to indicate the presence of new AS ID (mandatory)

- Echoed RN16 (mandatory)

- New AS ID (optional)

Proposal 12: A-IoT device determines to use RN16 as AS ID if 1 bit indicates no new AS ID is included in matched Msg1 response in Msg2.

Proposal 13: A-IoT device determines to use new AS ID as AS ID if 1 bit indicates new AS ID is included in matched Msg1 response in Msg2.

Msg3 scheduling

Proposal 14: The discussion on time-frequency resource information included in A-IoT Msg2 for A-IoT Msg3 waits for RAN1 further progress.

Msg2 Monitoring Window

Proposal 15: The discussion on Msg2 monitoring window waits for RAN1’s further progress.

NACK Indication

NACK indication for CBRA

Observation 5: If subsequent R2D message for NACK indication reception is R2D trigger message, Msg1/2 scheduling flexibility is restricted.

Proposal 16: For A-IoT Msg3, rely on whether the device receives NACK indication before subsequent paging message to determine re-access.

Addressing for NACK indication in CBRA

Proposal 17: NACK indication contains one or multiple NACKs for one or multiple A-IoT devices.

Proposal 18: When the reader is indicated by CN that the service type is inventory + command, AS ID is used for NACK indication for address the A-IoT device(s).

Proposal 19: When the reader is indicated by CN that the service type is inventory only, RN16 is used for NACK indication for address the A-IoT device(s).

NACK indication for CFRA

Proposal 20: R2D trigger message is also supported for CFRA procedure, which is used to indicate the start of inventory response resource

Proposal 21: The reader may transmit same paging message again to schedule retransmission of inventory response from A-IoT device.

Proposal 22: The A-IoT device always responds to the paging which target for itself and indicate CFRA, ignore transaction id if transaction id is included.

Proposal 23: Transaction id is not included in paging message for CFRA.

Reference

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19)

- R2-2502262, Huawei, HiSilicon, China Southern Power Grid, Illustrative figures for a deeper understanding of A-IoT MAC operations