3GPP TSG-RAN WG2 Meeting #130	R2-2503906

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.8.4

Source:		Lenovo

Title:		Some remaining issues for MBS broadcast in NR NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding MBS broadcast services in NTN:

RAN2#128 Agreements: - The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. - The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. - The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. - Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) - The legacy SIB modification procedure is applied to update the intended service area information in the new SIB. RAN2#129 Agreements: - In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list - It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) RAN2#129bis Agreements: - RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) - If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH - If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. - UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. FFS whether and how the association between intended service area and frequencies is acquired.

In this contribution we discuss some remaining issues including MBS broadcast service continuity.

- Discussion

As per RAN2 discussions throughout the latest meetings, some enhancements for MBS broadcast service continuity in NR NTN were agreed in principle while other details are still FFS. As the Rel-19 function freezing is approaching we list some remaining issues to be resolved as well as other enhancements in potential.

## MBS broadcast service continuity for RRC_IDLE/INACTIVE

The legacy MBS broadcast service continuity for RRC_IDLE/INACTIVE in TN is guaranteed by cell reselection frequency priority handling based on the association between cells and services, as the MBS broadcast service area is mapped to one or more cells. The UE can simply determine the frequency priority based on the receiving or interested MBS broadcast service and the frequency it associates to, and reselects to a cell that supports that service within its whole coverage.

A major difference in NTN is that an NTN cell coverage could be larger than an MBS broadcast service area, which cannot be mapped to the cell’s whole coverage, UE cannot simply determine the frequency priority only based on the association between service and frequency. For this reason, in the latest RAN2#129bis it has been agreed that the corresponding frequency priority handling also considers the MBS broadcast service area information indicated, i.e. UE may prioritize the frequency for an interested service when UE can only receive the service on the frequency and the UE is in intended service area associated with the service provided in the frequency. Otherwise, UE may de-prioritize the frequency.

A further remaining issue is acquiring of the association between intended service area and frequency, and two options were on the table: adding service area information/ID in SIB21, or rely on USD information combined with FSAI/frequency info in SIB21.

Firstly, we check the legacy SIB21 contents, and there is only the association between frequency and FSAI:

#### – SIB21 SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). SIB21 information element -- ASN1START -- TAG-SIB21-START SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 } MBS-FSAI-r17 ::= OCTET STRING (SIZE (3)) -- TAG-SIB21-STOP -- ASN1STOP SIB21 field descriptions. mbs-FSAI-InterFreqList Contains a list of neighboring frequencies including additional bands, if any, that provide MBS services and the corresponding MBS FSAIs.. mbs-FSAI-IntraFreq Contains the list of MBS FSAIs for the current frequency. For MBS service continuity, the UE shall use all MBS FSAIs listed in mbs-FSAI-IntraFreq to derive the MBS frequencies of interest..

And the legacy USD regarding service area is described in TS26.346:

… A User Service Description may include an availabilityInfo element. If present, it shall extend the list of child elements of the MBMS userServiceDescription element by indicating the presence of additional data pertaining to the availability of the service. The availabilityInfo element shall include one or more infoBinding elements. The infoBinding element shall contain the child elements serviceArea and radiofrequency. A UE shall be capable of processing an infoBinding element that does not contain the child element serviceArea. NOTE 0a: For backwards compatibility reasons, serviceArea needs to be indicated as optional in the USD schema (i.e. ‘minOccurs="0"’). - The serviceArea element declares the one or more service areas over which this MBMS User Service is provided. This element is designated by the MBMS Service Area Identity (SAI) as defined in 3GPP TS 36.443 [104] and 3GPP TS 23.003 [77]. According to 3GPP TS 36.443 [104], MBMS Service Area Identity is frequency agnostic and can be mapped onto one or more cells. The specific usage of the MBMS Service Area Identity, or its correlation to other network identification information, is not defined in the present document. - The radioFrequency element indicates the one or more RF frequencies in the E-UTRAN downlink which transmit this MBMS User Service over the service area(s) identified by the serviceArea element. The frequency parameter is coded as EARFCN in TS 36.101 [105]. …

It is observed that, the legacy USD does not contain any actual geographic information, wherein the elements within USD are actually mapped to one or more cells, but cannot be mapped to part of an NTN cell.

Observation 1: The service area information in USD is at cell level and thus cannot describe the MBS broadcast area as the new SIB does in NTN.

As a result, relying on USD information combined with FSAI/frequency info in SIB21 cannot derive the association between MBS broadcast service area and frequency.

Observation 2: UE cannot derive the association between MBS broadcast service area and frequency in NTN only based on USD information combined with FSAI/frequency info in SIB21.

Therefore, the only option left is adding service area information/ID in SIB21.

Proposal 1: MBS broadcast service area information or ID is included in SIB21 for service continuity.

During the previous discussion there is further concern about the overhead of including the geographic information of MBS broadcast service area in SIB21. In our understanding it is unnecessary to include all the geographic information in SIB21, while only the area ID would be sufficient. It is obvious that a neighbour cell MBS broadcast area for service continuity is likely to be an area across both serving and neighbour cell, and therefore it should have been included in the new SIB for the serving cell, as per RAN2#128 agreement: “A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s)”.

Proposal 2: If the geographic information of a neighbour cell MBS broadcast area is included in the new SIB, its associated area ID can be included in SIB21 for service continuity (i.e. there is no need to duplicate the geographic information in SIB21).

MBS broadcast service continuity for RRC_CONNECTED

For CONNECTED MBS broadcast service continuity in TN, the UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, including the list of MBS frequencies and interested services, as well as frequency priorities. The gNB may use this indication when providing handover configuration to allow the UE to receive the MBS services the UE is interested in. Due to the same assumption that an MBS broadcast service area is mapped to one or more TN cells, UE does not need to provide its current or expected MBS broadcast service area information in the MBS Interest Indication.

When considering that an NTN cell coverage could be larger than an MBS broadcast service area, UE’s current/expected MBS broadcast service area information is necessary at gNB for further handover decisions e.g., to judge whether UE is staying/moving into or moving out of an MBS broadcast service area and configure an appropriate target cell.

Proposal 3: UE may report its current broadcast service area information or area ID to gNB for service continuity in RRC_CONNECTED.

- Conclusion

In this contribution we discuss some remaining issues of MBS broadcast service continuity in NTN. It is observed that:

Observation 1: The service area information in USD is at cell level and thus cannot describe the MBS broadcast area as the new SIB does in NTN.

Observation 2: UE cannot derive the association between MBS broadcast service area and frequency in NTN only based on USD information combined with FSAI/frequency info in SIB21.

And we propose:

Proposal 1: MBS broadcast service area information or ID is included in SIB21 for service continuity.

Proposal 2: If the geographic information of a neighbour cell MBS broadcast area is included in the new SIB, its associated area ID can be included in SIB21 for service continuity (i.e. there is no need to duplicate the geographic information in SIB21).

Proposal 3: UE may report its current broadcast service area information or area ID to gNB for service continuity in RRC_CONNECTED.