3GPP TSG-RAN WG2 #130	R2-2503918

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.1.2.2

Source:		Lenovo

Title:	Left issues related to applicability report for AIML based BM

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss left issues related to LCM for AIML based beam management.

# Discussion

## Open issue RRC-8: Coexistence between option A and option B

Open issue RRC-8: Coexistence between option A and option B Issue description: Considering that both Option A and B are to be specified, we need to discuss how the 2 options co-exist, whether there is any interaction between them etc. For instance, it is not clear whether both option A and option B can be configured or not. This can affect signaling design of ApplicabilityReportList. If further procedures for option B are discussed, it is suggested to taken into account whether these procedures may depend on the list of inference related parameters from RAN1. Proposed resolution: Companies may provide contributions to the following meeting to resolve the issue.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

As replied in RAN1 LS, RAN1 considers the following two approaches for gNB to provide the applicable inference configuration, which can be summarized as following in our understanding

Option A): in step 3, via RRCReconfiguration, gNB provides one or more of CSI-ReportConfig for inference configuration; in step 4, via RRCReconfigurationComplete, UE indicates the applicable CSI-ReportConfig; after step 4,

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW.

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Option B): in step 3, via RRCReconfigurationàOtherConfig, gNB provides candidate sets of inference related parameters; in step 4, via UAI, UE indicates the applicable ones among the candidate sets of parameters; in step 5, accordingly, via RRCReconfiguration, gNB will generate and provide the CSI-ReportConfig for inference configuration.

One thing that worth clarification/discussion is that in Option B, after step 5 the full inference configuration is provided to UE, will UE also indicate the applicability/inapplicability to gNB as in Option A, e.g.,

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW.

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

As reflected in the Note from RAN1 agreements below, the UE is expected to be only configured with applicable full inference configurations based on applicable set of inference parameters. In that case, the full inference configurations in RRCReconfiguration (step 5) can be assumed to be already applicable. Thus, the UE may not be required to report any further inapplicability in RRCReconfigurationComplete (step 6). While this can be a simple approach and similar to legacy procedure, we think this is an idealistic scenario for AI/ML functionality. As per RAN1 discussion and agreements, in Option B, applicability is determined solely based on the set of inference parameters such that the applicable status is not subject to change (until RRCReconfiguration) and the UE can be configured with full inference configuration for the reported applicable functionalities in Step 4.

In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig.

On the other hand, RAN2 has formed the following understanding that the applicability is determined by UE based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. In that sense, in Option B even if gNB provides the full inference configuration according to the applicable parameters, there is still a chance that the full inference configuration in Option B becomes inapplicable and thus the indication/update of applicability/inapplicability after receiving full inference configuration is needed in Option B similar as in Option A (as illustrated in the figure below).

RAN2#127: UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

RAN2 is suggested to clarify that in Option B, after UE receives the full inference configuration, UE will indicate/update the applicability/inapplicability of configured inference configuration as in Option A (i.e., in RRCReconfigurationComplete and UAI).

If the answer to Proposal 1 is yes, comparing Option A) and Option B), the fundamental difference seems to be in Option B) gNB will receive the applicable parameters (in Option B step 3-4) before providing the inference configuration to UE. The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Option A) and B). Even though in Option B) UE is not expected to be configured with a non-applicable CSI-ReportConfig for inference, it may not be reflected in the specification as it is eventually upon gNB implementation how CSI-ReportConfig for inference is generated.

Comparing Option A) and Option B), the fundamental difference seems to be in Option B) gNB will receive the applicable parameters (in Option B step 3-4) before providing the inference configuration to UE.

The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Option A) and B).

Considering above analysis, a unified procedure (combining approach A and B) can be supported from specification point of view:

Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

Step 4: (Optional) UE reports in RRCReconfigurationComplete/UAI the applicability of the sets of parameters for inference configuration

Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

Step 8: For the subsequent change of applicability related to either inference configuration, or parameters for inference configuration, UE will indicate to gNB in UAI.

- Note that since the applicability/inapplicability of inference configuration and applicability/inapplicability of parameters for inference configuration serve for different purposes, it is possible that in the same UAI message UE will indicate both

  - Applicability/inapplicability of parameters for inference configuration, which was previously provided in OtherConfig, and

  - Applicability/inapplicability of full inference configuration, which was provided in RRCReconfiugration message.

It’s worth noting that step 3 and step 4 can be beneficial for gNB to reduce the possibility of providing a non-applicable inference configuration.

For applicability report, Option A and B are not mutual exclusive. Option B can be regarded as a superset of Option A.

If Proposal 1 is agreed, for applicability report, RAN2 is suggested to support a unified solution with Option B as superset of Option A:

  - Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

  - Step 4: (Optional) UE reports in RRCReconfigurationComplete/UAI the applicability of the sets of parameters for inference configuration

  - Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

  - Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

  - Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

  - Step 8: For the subsequent change of applicability related to either inference configuration or parameters for inference configuration, UE will indicate to gNB in UAI.

## Open issue RRC-1: Cause related to inapplicability

Open issue RRC-1: Cause of inapplicability Issue description: It is FFS how to define the simple cause value of inapplicability related to model availability and how to capture it in the spec. This issue refers to the RAN2#129bis agreement: Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec. The issue is captured as an editor’s note in the running CR, clause 5.3.5.3. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

In the previous meeting, RAN2 agreed that when UE indicates the inapplicability, a simple cause value can be added as well.

First of all, it would be beneficial for RAN2 to confirm that the cause value associated with the inapplicability can be added in both Option A (for a full inference configuration) and Option B (for a set of inference parameters) cases.

RAN2 confirms that the cause value associated with the inapplicability reporting can be added in both Option A (for a full inference configuration) and Option B (for a set of inference parameters) cases.

Then, with respect to the exact cause value, from gNB’s implementation point of view, it could be beneficial and enough for gNB to distinguish

- The inapplicability is temporary (e.g., due to model availability, UE power, UE computing resources) and may become applicable in short time.

  - In this case, gNB may keep the inference configuration and expecting UE’s further update of applicability in near future via UAI.

- The inapplicability is lasting (e.g., in-proper gNB configuration) and probably will remain inapplicable for a while.

  - In this case, gNB could simply de-configure the corresponding inference configuration.

For an inference configuration/parameter-set that is reported to be inapplicable, it is beneficial for gNB to distinguish if 1) gNB should remove the relevant configuration/parameter-set or 2) keep the relevant configuration/parameter-set and wait for further update.

In the last meeting online discussion, some companies proposed to indicate “model unavailability” explicitly to gNB. However, it may not be really helpful to gNB considering:

- gNB will not know when the model will be available (it is totally upon UE implementation if/when the model downloading will start), thus cannot decide properly whether should keep or remove the current configuration which is not applicable.

- “model unavailability” is not the only reason that gNB may decide to keep the current configuration. Other reasons could be “UE low power” or “UE limited computing resource”. We don’t need to list all possible reasons.

Indication of “model unavailability” along does not really help gNB’s further decision.

When UE indicates the inapplicability of (Option A) a full inference configuration or (Option B) a set of inference parameters, UE can further indicate a cause value if the inapplicability is temporary.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Comparing Option A) and Option B), the fundamental difference seems to be in Option B) gNB will receive the applicable parameters (in Option B step 3-4) before providing the inference configuration to UE.

Observation 2	The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Option A) and B).

Observation 3	For applicability report, Option A and B are not mutual exclusive. Option B can be regarded as a superset of Option A.

Observation 4	Beam prediction with Set A and Set B beams of the same frequency is supported.

Observation 5	For a inference configuration/parameter-set that is reported to be inapplicable, it is beneficial for gNB to distinguish if 1) gNB should remove the relevant configuration/parameter-set or 2) keep the relevant configuration/parameter-set and wait for further update.

Observation 6	Indication of “model unavailability” along does not really help gNB’s further decision.

Based on the discussion above, we propose:

Open issue RRC-8: Coexistence between option A and option B

Proposal 1	RAN2 is suggested to clarify that in Option B, after UE receives the full inference configuration, UE will indicate/update the applicability/inapplicability of configured inference configuration as in Option A (i.e., in RRCReconfigurationComplete and UAI).

Proposal 2	If Proposal 1 is agreed, for applicability report, RAN2 is suggested to support a unified solution with Option B as superset of Option A:

a.	Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

b.	Step 4: (Optional) UE reports in RRCReconfigurationComplete/UAI the applicability of the sets of parameters for inference configuration

c.	Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

d.	Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

e.	Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

f.	Step 8: For the subsequent change of applicability related to either inference configuration or parameters for inference configuration, UE will indicate to gNB in UAI.

Open issue RRC-1: Cause related to inapplicability

Proposal 3	RAN2 confirms that the cause value associated with the inapplicability reporting can be added in both Option A (for a full inference configuration) and Option B (for a set of inference parameters) cases.

Proposal 4	When UE indicates the inapplicability of (Option A) a full inference configuration or (Option B) a set of inference parameters, UE can further indicate a cause value if the inapplicability is temporary.

# Reference