3GPP TSG-RAN WG2 #130	R2-2503920

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.1.3

Source:		Lenovo

Title:	NW side data collection for AI based BM

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management and positioning use cases.

# Discussion

## 2.1	Open issue RRC-18: NW control on retaining logged data at HO, Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Open issue RRC-18: NW control on retaining logged data at HO Issue description: The signaling details of the network control on how data should be retained at handover are FFS, based on the RAN2#129 bis agreement: Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. The issue is captured as an editor’s note in the running CR, in clause 5.3.5.3 and 6.2.2. It the rapporteur’s view, it should be clarified Whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. From the rapporteur perspective, it is not possible for the source gNB to add the 1-bit indication during HO, in the same RRCReconfiguration that encapsulates the RRCReconfiguration from the target gNB. Thus, the rapporteur sees the following two possible solutions for this issue: - The source gNB sends the 1-bit indication to the UE before HO. - The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO. No RAN3 impact is expected if the transmission of the 1-bit indication is limited to the case in which the target gNB is from the same vendor as the source gNB. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue. Open issue RRC-36: How data is forwarded to OAM or source gNB after HO Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed. Proposed resolution: It is suggested that companies provide contributions to resolve the issue later.

For the issue captured by the rapporteur as described above, we believe it is enough for the source gNB to send the indication of “retain logged data” before HO execution as in the first option, since it is upon source gNB’s decision and interest at the end. Besides, this approach is also considered future proof, if RRC idle/inactive state transition or RLF is considered.

Comparatively, in the second option, the “retain logged data” would be conveyed as part of the target cell configuration, which UE will only apply after the HO execution. That is strictly speaking target gNB’s decision. Besides, the second option is believed to be limited to the case of same gNB vendor, however, in OAM-centric data collection it is possible that two gNBs are not of the same vendor.

The source gNB sends the 1-bit indication of “retain logged data during HO” to the UE before HO.

Another issue is that since both gNB-centric and OAM-centric data collection are supported, the remaining logged data at UE during HO shall be forwarded by the new serving gNB to either old serving gNB or OAM depending on the case. Note that in legacy logged MDT, the logged data is always transferred to OAM, which is a bit simpler case.

Therefore, from RRC configuration point of view, when the serving gNB configures the UE to retain the logged data during HO, serving gNB should also provide information to distinguish the consumer of the logged data source (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

After HO and UE is connected to a new serving gNB, and upon the request from the new serving gNB, when UE transfers the logged data to the network, the same consumer information shall be reported to the new serving gNB, so the serving gNB can determine and forward the logged data to the correct destination.

The consumer of the logged data can be old serving gNB (for gNB-centric data collection) or OAM (for OAM-centric data collection).

When configuring UE to retain the logged data during HO, the serving gNB should also provide information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

When transferring the logged data to the new serving cell after HO, UE also includes the information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

Send an LS to RAN3 informing the progress related to the retrieve of logged training data in handover scenario. A draft is provided in Annex.

## 2.2	Open issue RRC-20: Further procedures for UE assistance information related to logging

Open issue RRC-20: Further procedures for UE assistance information related to logging Issue description: It has not yet been discussed whether further procedures for UE reporting assistance information related to logging is need. Such further procedures may be, e.g. prohibit timers, indication that battery state is not low any longer, indication that the memory is not full any longer, etc. The issue is captured as an editor’s note in the running CR, in clause 5.7.4.2 and 5.7.4.3. Proposed resolution: It is suggested that companies provide contributions to the following meeting to resolve the issue.

Considering the data availability indication can be triggered due to many facts:

  - Full buffer being reached (if configured)

  - Buffer threshold being reached (if configured).

  - Low power (if configured)

Therefore, a prohibit timer is a straightforward design to avoid the too frequent data availability indication.

RAN2 supports prohibit timer to avoid too frequent data availability indication.

## 2.3	Trigger of data availability indicator

In the previous RAN2 meeting, it was agreed that UE will indicate the data availability to gNB in the following scenarios

Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured)

For event-triggered data logging, for which gNB is not aware of the start of logging, and the logged data size may not be large enough to trigger the data availability indication in Case#1. In this case, additional trigger of data availability could be beneficial, e.g., after the instance of event-triggered data logging starts or finishes (i.e., the L3 serving cell measurement becomes worse/better than a threshold for TTT time).

UE can trigger the data availability report when the event-triggered data logging starts or finishes.

## 2.4	Open issue RRC-31: The release of logged AIML data in UE’s buffer

Open issue RRC-31: The release of logged AIML data in UE’s buffer Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB. Proposed resolution: It is suggested that companies provide contributions to resolve this issue later.

Another aspect that has not been discussed yet is the release of the logged data in the UE buffer. Since it could be possible that the logged data is retained in UE buffer, while the gNB does not fetch for a long time.

#### 5.5a.1.3 Reception of the LoggedMeasurementConfiguration by the UE Upon receiving the LoggedMeasurementConfiguration message the UE shall: 1> discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2; 1> store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig; #### 5.5a.1.4 T330 expiry Upon expiry of T330 the UE shall: 1> release VarLogMeasConfig; The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry. ### 5.5a.2 Release of Logged Measurement Configuration #### 5.5a.2.1 General The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information. #### 5.5a.2.2 Initiation The UE shall initiate the procedure upon receiving a logged measurement configuration in same or another RAT. The UE shall also initiate the procedure upon power off or upon deregistration. The UE shall: 1> stop timer T330, if running; 1> if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

In legacy logged MDT, the logged data will be released upon

- New logged measurement configuration is received

- Release of logged measurement configuration

- Power off or deregistration

- 48 hrs after logging duration is reached

For the logged data in RRC connected state, since multiple data logging configurations (e.g., for different use cases) can be configured at UE, the receiving or release of logged measurement configuration is no longer a proper trigger of logged data release.

RAN2#128 Agreement: Duration is not supported

Also, since RAN2 agreed that the logging duration is not supported, then it is not a relevant factor in this case neither.

On the other hand, considering the handover scenario, where the UE connects to a new serving gNB with logged data retained before handover. The new serving gNB has to always fetch the retained logged data (imagining UE buffer is full) before configuring UE with any new data logging configuration. Considering the consumer of the logged data could be the old gNB, and it is after all the new serving gNB’s decision whether to fetch the logged data retained in UE’s buffer, it could be beneficial and a clean approach to introduce dedicated indication for the serving gNB to release the logged data in UE’s buffer.

The logged data in UE’s buffer will be released upon

  - Power off or deregistration

  - 48 hrs after the release of logged measurement configuration

  - Explicit indication from the serving gNB

## 2.5	Reporting related aspects based on availability indication

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- On-demand request from the network is supported.   FFS details on signalling

According to the last time RAN2 meeting, after logging the data, UE can report it to network based on the on-demand request, and FFS if event-based reporting and if data availability indication should be sent beforehand. In the following figure, we describe the overall data reporting procedure that would work efficiently in our view.

- Step 1: UE can be configured with event that would trigger the data report after UE starts logging the required measurement results. For example, some buffer size threshold (e.g., percentage of maximum buffer size) can be configured to trigger the report before the buffer is full.

- Step 2: UE will first send an availability indication to gNB, such that gNB can be prepared to reserve radio resources for scheduling the UL transmission of logged data. Comparing to let UE transmit the logged data straight away once triggered, this is considered a safer option to not interfere with other regular UL transmissions from the same or other UEs, gNB has the flexibility to fetch the logged data when the UL traffic in the cell is low. Since UE may have logged data for different data collection configurations, e.g. data logged for different UE/NW-side conditions, it is beneficial to provide gNB information on the data collection configurations for which UE has some logged data available for transmission, e.g. by signalling an ID identifying the data collection configuration. The availability indication should - similar to a buffer status report - indicate the amount of logged data (e.g. in bytes) for each data collection configuration (identified by an ID) in order to enable gNB to request some corresponding logged data and allocate UE resources accordingly.

- Step 3: Upon reception of the availability indication, gNB can request the UE to transmit some specific, e.g. for some specific data collection configuration(s), or all the logged data stored in its buffer via UEInformationRequest message

- Step 4: UE will generate and transmit UEInformationResponse message that contains the requested logged data stored in its buffer. If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively.

If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	The consumer of the logged data can be old serving gNB (for gNB-centric data collection) or OAM (for OAM-centric data collection).

Based on the discussion above, we propose:

Open issue RRC-18:NW control on retaining logged data at HO

Proposal 1	The source gNB sends the 1-bit indication of “retain logged data during HO” to the UE before HO.

Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Proposal 2	When configuring UE to retain the logged data during HO, the serving gNB should also provide information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

Proposal 3	When transferring the logged data to the new serving cell after HO, UE also includes the information related to the consumer of the logged data (e.g., cell ID for gNB-centric data collection, or trace reference for OAM-centric data collection).

Proposal 4	Send an LS to RAN3 informing the progress related to the retrieve of logged training data in handover scenario. A draft is provided in Annex.

Open issue RRC-20: Further procedures for UE assistance information related to logging

Proposal 5	RAN2 supports prohibit timer to avoid too frequent data availability indication.

Proposal 6	UE can trigger the data availability report when the event-triggered data logging starts or finishes.

Open issue RRC-31:The release of logged AIML data in UE’s buffer

Proposal 7	The logged data in UE’s buffer will be released upon

a.	Power off or deregistration

b.	48 hrs after the release of logged measurement configuration

c.	Explicit indication from the serving gNB

Proposal 8	If one UEInformationResponse is not enough to convey all logged data, UE should be able to generate and transmit multiple UEInformationResponse messages consecutively without waiting for another UEInformationRequest message.

# 4	Draft LS to RAN3

# Title:	LS on NW side model training data collection for AI based beam management and CSI prediction

# Response to:

# Release:	Rel-19

# Work Item:	NR_AIML_Air

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

#### Name:

#### E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

For the use case of AI based beam management or AI based CSI prediction, an AI model could be deployed at gNB side.

In order to collect required data for the training of such gNB-sided model, RAN2 agreed to support gNB-centric and OAM-centric data collection. In case of handover, UE can be configured by the source gNB to retain the logged L1 measurements that has not been reported to network yet during handover. After the handover, UE can indicate the data availability to the target gNB, and the target gNB can later retrieve the logged L1 measurements from UE via the UE information procedure.

2. Actions:

To WG RAN3

ACTION:

RAN2 kindly asks RAN3 to take the above progress into account in their future work and provide further feedback if any.

3. Date of Next RAN2 Meetings:

RAN2#131		25th – 29st Aug 2025	TBC, IN

RAN2#131bis		13th – 17th Oct 2025	Malta, MT