3GPP TSG-RAN WG2 #130	R2-2503921

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.1.3

Source:		Lenovo

Title:	NW side data collection for AI based positioning

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management and positioning use cases.

# Discussion

## 2.2	Data collection for Positioning 3a and 3b

RAN1#116bis Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. RAN1#120 - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications.

(1st priority) case 3a) NG-RAN node assisted positioning with gNB-side model:

RAN1 has agreed that for case 3a, the label and its related data can be generated by at least LMF. Since it’s an AIML model deployed at gNB-side, gNB can retrieve the training data from LMF as discussed in RAN3.

From RAN2 point of view, RAN2 can assume LPP procedure is used by LMF to collect the training data from UE/PRU. The possible LPP impact could be upon RAN1 discussion. For example, if label comprises LOS/NLOS indication, there may be no further LPP impacts, however if the label also includes PRU SRS Tx Location, then possible LPP impacts are expected to retrieve/update the PRU location. RAN2 is thus suggested to wait for RAN1 progress before discussing the possible LPP impact.

For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 assumes LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation) without specification impact.

(1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning. It is only for UL based direct positioning.

During the RAN1#116bis and RAN1#117 meetings, RAN1 assumes the training data (e.g., label and time stamp) for sub use case 3b can be generated by PRU or non-PRU UEs, and how the training data is transferred from PRU to NW is out of RAN1 scope.

From RAN2 point of view, since case 3b is related to LMF-sided AIML model, it seems natural for LMF to collect the required training data (e.g., label and time stamp) from PRU via LPP protocol.

RAN1#117 Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved.

For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 assumes LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation) without specification impact..

# 3	Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	For (case 3a) NG-RAN node assisted positioning with gNB-side model, RAN2 assumes LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation) without specification impact.

Proposal 2	For (case 3b) NG-RAN node assisted positioning with LMF-side model, RAN2 assumes LMF collecting training data from PRU or non-PRU UE via existing LPP procedures (e.g., RequestLocationInformation and ProvideLocationInformation) without specification impact..