3GPP TSG-RAN WG2 Meeting #130	R2-2503943

St. Julian’s, Malta, 19 - 23 May, 2025

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on DL coverage enhancements

Agenda Item:	8.8.2

Document for:	Discussion and decision

- Introduction

In RAN2#129b, the following agreements on downlink coverage enhancement have been achieved [1]:

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution, we will provide further analysis on the remaining issues.

- Discussion

  - SMTC selection for RRC_IDLE/RRC_INACTIVE

RAN2 has agreed that the network can provide some location information to help UE in RRC_IDLE/RRC_INACTIVE mode to perform SMTC selection, FFS on the details of location information, e.g. serving cell SSB index, reference location, etc.

From our perspective, Option 2 is preferred because it saves the UE complexity of calculating the distance between the UE location and the respective refence locations. UE in RRC_IDLE/RRC_INACTIVE anyway needs to perform serving cell measurements to make cell reselection decisions. Besides, one reference location takes up about 48bits, meaning that Option 1 will need at least 288bits for selecting SMTCs on a single frequency. In Option 2, a 6-bit bitmap for SMTCs can be indicated for each SSB index, and the overall overhead is around 24bits considering a typical FR1 scenario.

Proposal 1: For UEs in RRC_IDLE/RRC_INACTIVE, the SMTCs are associated with serving cell SSB indexes. It is up to UE to select the corresponding SMTC based on measured SSB.

  - SMTC selection for RRC_CONNECTED

During RAN2#129bis, there is no time for online/offline discussion on RRC_CONNECTED mode, but the RAN2#129bis summary has narrowed down the solutions into 3 options [2]:

Option 1: UE provides assistance information to network, such as information related to the distance between UE and reference location of neighbour cells.

Option 2: Measurement report-based solution, e.g. Event D1/D2 triggered measurement report.

Option 3: UE coarse location report.

On coarse location reporting, we think it depends on user consent, and it cannot be guaranteed that the user consent is obtained for all UEs. In R17, SA3 has confirmed that user consent is needed for location reporting regardless of whether the reported location is coarse location or fine location [3]. Besides, SA3 has not standardized the user consent for location reporting in NTN, it is up to local regulations and proprietary mechanisms. That is also why RAN2 introduced PDD (propagation delay difference) reporting to solve SMTC issues in the last meeting before R17 freezing (i.e. RAN2#118 meeting).

Observation 1: UE coarse location report cannot be relied on to solve SMTC issue, as it depends on user consent while user consent is up to location regulations and proprietary mechanisms rather than standardized by SA3.

Option 1 is quite similar to the R17 PDD reporting that is included in the UE assistance information. PDD cannot be used to solve the SMTC configured in beam hopping scenarios, because the serving cell and neighbour cells come from the satellite and the PDD is zero.

Option 2 utilizes the existing event D1/D2, but we think there are two major drawbacks:

1) In the entering/leaving conditions of event D1/D2, there is one condition on UE distance to serving cell reference location, take event D1 as an example:

Inequality D1-1 (Entering condition 1) Ml1 – Hys > Thresh1 Inequality D1-2 (Entering condition 2) Ml2 + Hys < Thresh2 Inequality D1-3 (Leaving condition 1) Ml1 + Hys < Thresh1 Inequality D1-4 (Leaving condition 2) Ml2 – Hys > Thresh2

If the serving cell condition is not satisfied, event D1/D2 will never be triggered. It is also noted that the distance to serving cell cannot be configured as 0 (it is 50m at least), therefore if a UE is within 50m radius of serving cell coverage centre, D1/D2 cannot be triggered.

2) The second drawback is that Option 2 requires the network to configure too many ReportConfigs. Considering 6 neighbour cells on a single frequency, the network needs to configure 6 ReportConfigs, each including an event D1/D2 associated to a neighbour cell. It is also uncertain which one will be triggered first (this depends on UE implementation to determine which measurement task to be performed first), if one event D1/D2 is triggered and reported first, the network has no idea whether there are other events to be triggered subsequently, thus unclear whether the network should configure SMTC/gap immediately or wait for a while for other triggered reports.

Observation 2: Event D1/D2 requires a serving cell reference location condition to be configured, and multiple events to be configured (each corresponds to one neighbour cell).

Based on the above analysis, we think Option 1 is the cleanest solution.

Proposal 2: UEs in RRC_CONNECTED reports assistance information (e.g. closest reference location) to assist SMTC/gap configuration.

  - SIB1 transmission period for extended SSB periodicity

It is also noted that in the existing 38.331, SIB1 periodicity is either 20ms (multiplexing pattern 1) or the same as SSB periodicity (multiplexing pattern 2/3):

--- cited from TS 38.331 clause 5.2.1 --- - the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

Since SSB periodicity is extended to 160ms due to beam hopping, SIB1 periodicity should also be extended. No agreement has been made in RAN1 yet, as RAN1 is now focusing on SIB1 repetition under link level enhancements. We think it can be discussed in RAN2 whether to change the corresponding spec regarding SIB1 periodicity for initial access.

Proposal 3: For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 160ms if SSB periodicity is 160ms.

- Conclusion

This contribution discussed the DL coverage enhancements in Rel-19 NR NTN. Proposals are summarized as follows:

Proposal 1: For UEs in RRC_IDLE/RRC_INACTIVE, the SMTCs are associated with serving cell SSB indexes. It is up to UE to select the corresponding SMTC based on measured SSB.

Observation 1: UE coarse location report cannot be relied on to solve SMTC issue, as it depends on user consent while user consent is up to location regulations and proprietary mechanisms rather than standardized by SA3.

Observation 2: Event D1/D2 requires a serving cell reference location condition to be configured, and multiple events to be configured (each corresponds to one neighbour cell).

Proposal 2: UEs in RRC_CONNECTED reports assistance information (e.g. closest reference location) to assist SMTC/gap configuration.

Proposal 3: For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 160ms if SSB periodicity is 160ms.

- References

- R2-2502983, Report from session on NR NTN and IoT NTN, Session chair (ZTE)

- R2-2503055, Report of [AT129bis][303][R19 NR NTN] SMTC enhancements, Xiaomi

- S3-221268, LS reply on Reply LS on NTN specific User Consent and UE location in connected mode in NTN (contact company: Ericsson), SA3