3GPP TSG-RAN WG2 Meeting #130	R2-2503952

St.Julians, Malta, 19th –23rd May, 2025

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon

Title:	A-IoT random access procedure

Document for: Discussion and Decision

1	Introduction

In this contribution, the details of A-IoT random access procedure are further discussed.

2	Discussion

2.1	R2D trigger message and Msg1 related

2.1.1	QueryRep-like/Access occasion Trigger message (2-2, 2-3)

In RAN2#129bis meeting, a new R2D message (i.e., QueryRep-like message or Access Occasion Trigger message) was agreed to indicate the start of a set of access occasion(s). The Msg1 transmission related configurations are provided in paging message rather than the new R2D message.

Agreements on new R2D message - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

Observation 1:	RAN2 has assumed that the Msg1 transmission related configurations are provided in paging message, while R2D trigger message does not configure the parameters.

Thus, the “QueryRep-like” message is just one trigger. There is no other information needed in this MAC message, except for its message type. Considering that this message is the most frequently R2D message, it is beneficial for the efficiency to minimize the overhead. Thus, it can be one MAC message including only the message type field, i.e., 3 or 4-bit MAC PDU. Noted that this R2D trigger message is not suitable to be byte aligned. For example, in order to trigger 1000 devices for inventory, almost 1000 “QueryRep-like” messages are needed when one time-domain resource is triggered by a “QueryRep-like” message. It requires an additional 4000 bits, if this MAC message is 8bits which is byte aligned. In RFID [1], such design (see Figure 2.1.1-1) has already been adopted which works efficiently.

Figure 2.1.1-1	QueryRep command in RFID

Proposal 1:	(Issue 2-3) The R2D trigger message (i.e., Access Occasion Trigger message) only contains the message type field, i.e., just 3 or 4-bit MAC PDU.

The start of the first set of Msg1 resource(s) can also be indicated by the paging message. The “QueryRep-like” message provides no further parameter in addition to the start indication, which can be done by any R2D message. Therefore, the paging message can trigger the first set of Msg1 resources. Otherwise, the devices would have to receive 2 messages continuously (i.e. paging message and QueryRep-like message) before sending Msg1. It would mean that an additional QueryRep-like message would have to be introduced, which is not desirable.

Proposal 2:	(Issue 2-2) The start of the first set Msg1 resource(s) is indicated by the paging message, rather than by the Access Occasion Trigger message, for both CBRA and CFRA, i.e., combine the first R2D trigger message with paging message.

2.1.2	Msg1 resource selection (2-1)

For Msg1 resource selection, we had the following agreements for time and frequency domain resource selection.

6 From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA. FFS if this is applicable to FDMA. Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA. 2. RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

Thus, the reader indicates the total number of access occasions in time-frequency domain by paging message, the device directly randomly selects one access occasion from the total number of access occasions.

In RFID [1], a slot counter decrement mechanism is supported to determine the slot for access.

Upon receiving a Query, Tags with matching Sel and Target shall pick a random value in the range (0, 2Q–1), inclusive, and shall load this value into their slot counter. If a Tag, in response to the Query, loads its slot counter with zero, then its reply to a Query shall be as shown in Table 6-33 using the immediate reply type specified in 6.3.1.6.1; otherwise the Tag shall remain silent. If a Tag, in response to the QueryRep, decrements its slot counter and the decremented slot value is zero, then its reply to a QueryRep shall be as shown in Table 6-37 using the immediate reply type specified in 6.3.1.6.1; otherwise the Tag shall remain silent. A Tag shall respond to a QueryRep only if it received a prior Query.

A similar device behaviour can be specified for A-IoT, based on the random number generated for access occasion. The example of text proposal for this kind of Msg1 resource selection procedure is as below. As an alternative, the Msg1 resource selection is totally up to device implementation and nothing needs to be capture in the specification.

=================================Text Proposal============================= 5.3.1.1 Selection of access occasion for D2R transmission of Random ID message If Contention-Based Random Access procedure is initiated according to clause 5.2, the A-IoT MAC entity shall: 1> draw a random integer number 'i' in the range: 0 ≤ i ≤ n-1, where n is the number of access occasions configured in A-IoT Paging message; 1> set the ACCESS_OCCASION_COUNTER to 'i'; Upon reception of the Access Occasion Trigger message, the A-IoT MAC entity shall: 1> if the Access Occasion Trigger message triggering 'm' access occasion(s) is received: 2> if ACCESS_OCCASION_COUNTER < m: 3> select the 'ACCESS_OCCASION_COUNTER+1' th access occasion from the m access occasion(s) triggered by this Access Occasion Trigger message for transmission of the Random ID message; 3> apply the D2R Scheduling Info received from the A-IoT Paging message for the selected access occasion and indicate it to the physical layer; 3> initiate the transmission of Random ID message, as specified in clause 5.3.1.2. 2> else: 3> decrement ACCESS_OCCASION_COUNTER by m. =================================Text Proposal=============================

Proposal 3:	(Issue 2-1) RAN2 to discuss the following options for the TP about Msg1 resource selection:

Option 1: Msg1 resource selection details are left to device implementation, since it is not testable, i.e., no need to capture the details in the MAC specification;

Option 2: consider the above TP for Msg1 resource selection procedure (e.g., capture the countdown behaviour in the MAC specification).

2.2	CBRA procedure related

2.2.1	Contention resolution failure (2-4)

To handle the device contention resolution failure in order to correctly perform the re-access, the condition for device to determine the missing of its random ID should be defined.

Figure 2.2.1-1	Contention resolution failure examples

Observation 2:	A-IoT device does not support autonomously re-access, which can only be triggered by subsequent paging from reader. Therefore, it is necessary to have early determination on contention resolution failure before subsequent paging.

Then, the discussion point should be, after one device sends its Msg1, whether the following Msg2 for other devices will cause misunderstanding for this device’s contention resolution. Namely whether the early determination (e.g., based on some Msg2 timer) is needed to avoid such kind of collision among Msg2s.

This random ID collision among Msg2 happens after device-1 successfully sends its random ID, if any of the other N devices, whose random ID has been successful received by reader and included in a Msg2, use the same value of random ID as the device-1. In this situation, the device has not yet received its own Msg2 and the subsequent devices whose random ID has been successful received by reader will respond the Msg2 simultaneously, resulting in Msg3 transmission conflicts.

To assess the impact of random ID collisions, we can consider a simplified worst-case scenario. The worst case occurs when the "unlucky" device is the last one among all the devices, and it could have resolved contention successfully. All devices whose random IDs have not been received by the reader may impact this "unlucky" device. It means that this "unlucky" device has the highest probability of experiencing a random ID collision.

For example, let’s assume that 1000 devices try the access in one paging round, and among those, about 370 devices (i.e., N) successfully send the random ID, while, the remaining 630 devices continue to try to receive Msg2 for contention resolution. The “unlucky” device is the last device among the 370 devices and all 630 devices perform random access before it. The random IDs in Msg2 of those 630 devices may happen to be same/collide with the “unlucky” device, with a probability of 1-(1-1216630) = 0.96%. Even if the random ID collision happens, the consequence is that the device needs to perform re-access again.

Observation 3:	The random ID collision among Msg2 due to no Msg2 reception window is as low as 1%.

Observation 4:	If the Msg2 timer is not supported, the low complexity of device is more attractive, compared to the performance degradation caused by random ID collision among Msg2s.

Proposal 4:	(Issue 2-4) Similarly to the NACK feedback discussion, the device detects contention resolution failure by checking whether its random ID is received before paging, i.e., no need of timer.

2.3	Msg2 content

2.3.1	Random ID collision in Msg2 (2-5)

In RAN1#120 meeting, RAN1 had the following agreement for Msg2.

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

The Msg2 needs to support multiplexing of random ID to multiple devices. The probability that devices in different access occasions select the same random ID is estimated as follows. N represents the number of random ID successfully received by reader across different access occasions. To respond to Msg1s in access occasions assigned by one R2D message, either one (as shown in Figure 2.2.1-1) or multiple (as shown in Figure 2.2.1-2) Msg2(s) can be used, and these N random IDs are to be included in the Msg2(s). Note that no differentiation between different Msg2s is assumed. The probability under consideration is that at least two of these N random IDs are identical.

If the first random ID is random ID*, the probability that the second random ID is different from the first random ID is (216-1)/216. By analogy, the probability that the N-th random ID is different from the other N-1 random IDs is (216-(N-1))/216. Hence, the probability that N random IDs are totally different from each other is:

P0=(216)!216-N!216N=216×216-1…216-N-1216N.

The probability that at least two of these N random IDs are the same is:

P=1-P0=1-216×216-1…216-N-1216N.

Table 2.3.1-1	Calculation results of random ID collision probability P with different N

| N | P | N | P | N | P |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.00000% | 5 | 0.01526% | 9 | 0.05492% |
| 2 | 0.00153% | 6 | 0.02289% | 10 | 0.06864% |
| 3 | 0.00458% | 7 | 0.03204% | 11 | 0.08389% |
| 4 | 0.00916% | 8 | 0.04272% | 12 | 0.10066% |

According to RAN1 discussion, the total number of access occasions triggered by one R2D message is limited. For example, the access occasions assigned by one R2D message are 2 for time domain resources (X=2) and 8 for frequency domain resources (Y=8). Considering the optimal successful access probability in ALOHA [2] as 36.8%, then N is 6 and the probability P is just 0.02289%. Even for 2 consecutive QueryRep-like messages, i.e., 4 and 8 time- and frequency-domain resources for Msg1 transmission, the N is 12 and the probability P is about 0.1%.

Observation 5:	The probability of random ID collision in Msg2 is below 0.1%, which can be considered as sufficiently low.

Even if there is a collision of random IDs among devices in different access occasions, the reader can identify it and avoid echoing the ambiguous random ID(s) in Msg2 by implementation. This will prevent further ambiguity for the devices when performing the subsequent RA procedure.

Observation 6:	In case there is a collision of random IDs among devices in different access occasions, the reader can identify it and avoid echoing the ambiguous random ID in Msg2 by implementation.

Observation 7:	From RAN2 perspective, Msg2 does not require the information referring to the time-frequency resource of Msg1, i.e., the contained random ID alone is sufficient for the device to determine the response for its Msg1.

Thus, only echoed random ID in Msg2 is enough to determine the contention resolution without referring any Msg1 resource. If a strong concern on random ID collision in Msg2 is raised, the frequency index of Msg1 can be considered to refer to each echoed random ID. In case of Msg1 time domain resource X=1, only frequency index is needed; in case of Msg1 time domain resource X=2, the random ID collision between only two different Msg1 resources is negligible.

Proposal 5:	(Issue 2-5) RAN2 to discuss the following options for devices to determine the corresponding response in Msg2 for its Msg1:

Option 1: only use the random ID itself;

Option 2: use frequency index of Msg1 to reference each random ID.

2.3.2	AS ID assignment in Msg2 (2-7)

In RAN2#129bis meeting, it was agreed that Msg2 can be used for AS ID assignment for CBRA.

8 For CBRA, Msg 2 is used for AS ID assignment

The reader can decide whether to assign a new AS ID for a device, or allow the random ID upgrading to the AS ID. Thus, in a Msg2 including multiple random IDs, there is the case that only some of the random IDs have associated AS ID. The device has to know whether an AS ID is assigned for the echoed random ID. To achieve it, a 1-bit Extension field can be used following each echoed random ID to indicate whether an AS ID is assigned or not. For example, as showed in Figure 2.3.4-1, if the E is set to “1”, the 16-bits following the random ID is interpreted as the associated AS ID; otherwise, no AS ID is assigned.

Figure 2.3.2-1:	An example for Extension field to assign AS ID

Proposal 6a:	(Issue 2-7) Use 1-bit field, following each echoed random ID, to indicate whether an associated AS ID is assigned or not.

2.3.3	Number of random IDs (2-6)

In addition to the message type filed, the Msg2 includes a list of entries, each of which includes one random ID and one optional AS ID. In the MAC format design, there is no need to explicitly indicate the number of entries in one Msg2. The device knows the Msg2 TBS and the message type field size. Then, for the rest of Msg2 will be just a list. If RAN2 agrees to use 1-bit field, following each echoed random ID, to indicate whether an associated AS ID is assigned or not, then, the size of each entry (either 16+1 bits or 16+1+16 bits) is known by the device. Without the explicit information about the number of entries, device can decode the entry one by one till the end of Msg2 (see Figure 2.3.2-1).

Observation 8:	Assuming all the other parts in Msg2 have fixed length and are put in the beginning of the MAC PDU, the device can decode the random ID (and the associated AS ID if present) one by one till the end of Msg2.

Proposal 6b:	(Issue 2-6) No need to indicate the number of the included random IDs in Msg2, if proposal 6a is agreed.

Note that RAN1 agreed the Msg3 resources scheduled by one Msg2 are in the same time domain. Then, it means that the maximum Msg3 resources can be scheduled by one Msg2 is the max number of frequency resource, i.e. Y value.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

Proposal 7:	(Issue 2-6) The maximum number of random IDs in one Msg2 is Y, based on RAN1 conclusion, i.e., max value of frequency resources.

2.3.4	Device behaviour to handle Msg2 retransmission

In RAN2#129bis meeting, the following agreement was reached.

6 The device only keeps one AS ID at a time.

The intention of the above agreement is that if a new AS ID is assigned, it will overwrite the previous AS ID stored in the device. This rule should also be applicable for random ID. If a new AS ID is assigned, it will overwrite the random ID and only one ID is stored by the device.

Proposal 8a:	RAN2 confirms that the device is allowed not to store the random ID anymore once an AS ID is stored, i.e., only one ID is stored.

To handle Msg2 retransmission, if the device missed the first (or previous) Msg2, there is no specific device behaviour, since the device considers the retransmitted Msg2 as the initial Msg2. If the device received the first (or previous) Msg2 and failed to transmit the Msg3, the device needs to continue monitoring all R2D message, e.g., Msg2 or Msg4. If the retransmitted Msg2 is received, the device should be able to identify its own Msg2 and sends Msg3. There are following two cases:

-	Case 1: a new AS ID is assigned in the first (or previous) Msg2, the stored AS ID is the assigned AS ID;

-	Case 2: new AS ID is not assigned, the stored AS ID is the random ID sent in Msg1.

To simplify the handling complexity, a consistent device behaviour can be supported for both Case 1 and Case 2 above. For each entry of echoed random ID in Msg2, the device compares the stored AS ID with assigned AS ID if present; otherwise (AS ID is not present for this echoed random ID), the device compares the stored AS ID with echoed random ID. If the matched ID is found, the device will use the associated D2R scheduling information for Msg3 transmission.

Proposal 8b:	To determine the associated D2R scheduling information for Msg3 retransmission, if the device has the stored AS ID (i.e., Msg2 was successfully received previously) and upon receiving the retransmitted Msg2, the device compares the stored AS ID with for each entry of the echoed random ID(s) in Msg2:

Use the assigned AS ID field, if present;

Use the echoed random ID field, otherwise.

2.4	CFRA procedure specific

2.4.1	No re-access for CFRA (2-8)

As to the remaining issue for how to deal with a transmission (i.e. device ID) failure, since there is no re-access for CFRA, the following options can be considered:

-	Option 1: the reader always uses new transaction ID (regardless of the same service or a new service) in paging;

-	Option 2a: the device always responds to paging in case of CFRA, regardless of the same transaction ID or a new transaction ID (i.e. ignoring the transaction ID);

-	Option 2b: the device always responds to paging in case of CFRA, no transaction ID included in paging.

Option 1 provides the same device behaviour for CBRA and CFRA, as the device will always respond to paging with new transaction ID. The device behaviour for Option 2a is different, but the format of paging message for CBRA and CFRA is the same. However, transaction ID is actually useless, no matter which of the above options is chosen. Option 2b gives the simplest device behaviour and the format of paging message. Since the device behaviour/message handling will be different anyway between CBRA and CFRA, there is not much need to keep the device behaviour and the format of paging message aligned, therefore Option 2b is preferred for CFRA.

Proposal 9:	(Issue 2-8) Paging message for CFRA does not include the transaction ID. The device always responds to the paging for CFRA, as long as the paging identifier is matched.

2.4.2	Multiple devices CFRA (2-9)

The following agreement was reached for CFRA.

- RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario.

Regarding the need of supporting multiple devices for CFRA, one possible benefit is that it provides a tighter timeline for multiple devices’ service. However, if more information is required other than AS ID in R2D command message, this actually reduces the efficiency due to more overhead. There is no obvious gain, yet the complexity is increased. Thus, there is no need to consider the multiple devices scenario in CFRA.

Proposal 10:	(Issue 2-9) For CFRA, there is no need to further include any other information other than the AS ID in R2D command message, assuming the reader will not trigger multiple devices in CFRA.

2.5	NACK feedback

2.5.1	Re-access determination (2-10)

For NACK feedback indication, there was an FFS whether subsequent R2D message is trigger message or paging.

2 For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging

From device perspective, the device behaviours are the same no matter whether the subsequent R2D message is “QueryRep-like” message or paging message. The device needs to continue monitoring R2D message until it receives the subsequent paging triggering the re-access. Thus, it is pointless to detect failure earlier. Re-access determination before the subsequent paging message is sufficient.

Observation 9:	It makes no difference for device behaviour whether the subsequent R2D message is “QueryRep-like” message or the paging message.

Proposal 11:	(Issue 2-10) The device determines whether to perform re-access based on the reception of the NACK feedback indication before subsequent paging message.

2.5.2	NACK multiplexing (2-11, 2-12)

Within one paging procedure, some devices can finish the procedure successfully and some may not, thus the feedback indication should be per device feedback. The AS ID of the devices that failed can be included. The feedback can be sent to device in unicast way but it requires multiple R2D messages, each of which increases the signalling overhead, e.g., with more message type headers, preambles and CRCs in PHY layer. So, the feedbacks to multiple devices can be aggregated/carried into one broadcast R2D message. How many NACKs are aggregated and when to send the NACK feedback indication it up to reader implementation. Thus, a separate R2D message can be defined to carry NACK feedback indication(s).

Proposal 12:	(Issue 2-11) The "NACK feedback indication" is carried by one separate R2D message, using the AS ID to indicate the failed device.

Proposal 13:	(Issue 2-12) The "NACK feedback indication" can be one broadcast R2D message, which can include multiple AS IDs of the devices that failed.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	RAN2 has assumed that the Msg1 transmission related configurations are provided in paging message, while R2D trigger message does not configure the parameters.

Observation 2:	A-IoT device does not support autonomously re-access, which can only be triggered by subsequent paging from reader. Therefore, it is necessary to have early determination on contention resolution failure before subsequent paging.

Observation 3:	The random ID collision among Msg2 due to no Msg2 reception window is as low as 1%.

Observation 4:	If the Msg2 timer is not supported, the low complexity of device is more attractive, compared to the performance degradation caused by random ID collision among Msg2s.

Observation 5:	The probability of random ID collision in Msg2 is below 0.1%, which can be considered as sufficiently low.

Observation 6:	In case there is a collision of random IDs among devices in different access occasions, the reader can identify it and avoid echoing the ambiguous random ID in Msg2 by implementation.

Observation 7:	From RAN2 perspective, Msg2 does not require the information referring to the time-frequency resource of Msg1, i.e., the contained random ID alone is sufficient for the device to determine the response for its Msg1.

Observation 8:	Assuming all the other parts in Msg2 have fixed length and are put in the beginning of the MAC PDU, the device can decode the random ID (and the associated AS ID if present) one by one till the end of Msg2.

Observation 9:	It makes no difference for device behaviour whether the subsequent R2D message is “QueryRep-like” message or the paging message.

R2D trigger message and Msg1 related (2-1, 2-2, 2-3)

Proposal 1:	(Issue 2-3) The R2D trigger message (i.e., Access Occasion Trigger message) only contains the message type field, i.e., just 3 or 4-bit MAC PDU.

Proposal 2:	(Issue 2-2) The start of the first set Msg1 resource(s) is indicated by the paging message, rather than by the Access Occasion Trigger message, for both CBRA and CFRA, i.e., combine the first R2D trigger message with paging message.

Proposal 3:	(Issue 2-1) RAN2 to discuss the following options for the TP about Msg1 resource selection:

Option 1: Msg1 resource selection details are left to device implementation, since it is not testable, i.e., no need to capture the details in the MAC specification;

Option 2: consider the above TP for Msg1 resource selection procedure (e.g., capture the countdown behaviour in the MAC specification).

CBRA procedure related (2-4)

Proposal 4:	(Issue 2-4) Similarly to the NACK feedback discussion, the device detects contention resolution failure by checking whether its random ID is received before paging, i.e., no need of timer.

Msg2 content (2-5, 2-6, 2-7)

Proposal 5:	(Issue 2-5) RAN2 to discuss the following options for devices to determine the corresponding response in Msg2 for its Msg1:

Option 1: only use the random ID itself;

Option 2: use frequency index of Msg1 to reference each random ID.

Proposal 6a:	(Issue 2-7) Use 1-bit field, following each echoed random ID, to indicate whether an associated AS ID is assigned or not.

Proposal 6b:	(Issue 2-6) No need to indicate the number of the included random IDs in Msg2, if proposal 6a is agreed.

Proposal 7:	(Issue 2-6) The maximum number of random IDs in one Msg2 is Y, based on RAN1 conclusion, i.e., max value of frequency resources.

Proposal 8a:	RAN2 confirms that the device is allowed not to store the random ID anymore once an AS ID is stored, i.e., only one ID is stored.

Proposal 8b:	To determine the associated D2R scheduling information for Msg3 retransmission, if the device has the stored AS ID (i.e., Msg2 was successfully received previously) and upon receiving the retransmitted Msg2, the device compares the stored AS ID with for each entry of the echoed random ID(s) in Msg2:

Use the assigned AS ID field, if present;

Use the echoed random ID field, otherwise.

CFRA procedure specific (2-8, 2-9)

Proposal 9:	(Issue 2-8) Paging message for CFRA does not include the transaction ID. The device always responds to the paging for CFRA, as long as the paging identifier is matched.

Proposal 10:	(Issue 2-9) For CFRA, there is no need to further include any other information other than the AS ID in R2D command message, assuming the reader will not trigger multiple devices in CFRA.

NACK feedback (2-10, 2-11, 2-12)

Proposal 11:	(Issue 2-10) The device determines whether to perform re-access based on the reception of the NACK feedback indication before subsequent paging message.

Proposal 12:	(Issue 2-11) The "NACK feedback indication" is carried by one separate R2D message, using the AS ID to indicate the failed device.

Proposal 13:	(Issue 2-12) The "NACK feedback indication" can be one broadcast R2D message, which can include multiple AS IDs of the devices that failed.

# 4	References

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard.

- Y. Maguire and R. Pappu, “An Optimal Q-Algorithm for the ISO 18000-6C RFID Protocol,” in IEEE Transactions on Automation Science and Engineering, 2009.