3GPP TSG-RAN WG2 Meeting #130	R2-2503961

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.2.4

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1 Introduction

In RAN2#129bis meeting, RAN2 discussed the A-IoT data transmission and achieved the following agreements [1]:

AS ID AS ID is applied for Inventory + command case; AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed) For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits. Do not specify the reader behaviour on how exactly the ASID is generated. The device releases the AS ID upon power off (no stage 3 specification impact); The device only keeps one AS ID at a time. For CFRA, command message is used for AS ID assignment For CBRA, Msg 2 is used for AS ID assignment The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely. Segmentation For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. FFS whether the reader always includes the command for retransmission of segments. 1-bit indication is sufficient to indicate whether more D2R data will be sent. For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption. MAC PDU Format The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1. The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS. FFS whether we introduce D2R message type. Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits

In this contribution, we will discuss the left issues on the A-IoT data transmission.

2 Discussion

## 2.1 MAC PDU format of A-IoT

RAN2 discussed the MAC PDU format for A-IoT and concluded that R2D MAC header at least includes message type, length for SDU and variable part(s). For D2R, whether we need message type or reserved bits is FFS.

In R2D, we defined message type in MAC PDU for differentiating the MAC PDUs for different functionalities. For example, R2D MAC PDU message type may include paging, R2D trigger, Msg2 and data. Similarly, in D2R, message type should also include in MAC PDU for differentiating the MAC PDUs for different functionalities. For example, D2R MAC PDU for Msg1, Msg3 and data. It can make the A-IoT procedure steps clearer. Some company think the D2R message type can be determined by reader according to R2D context, it is not essential to have D2R message type in R19. But we should consider the compatibility of subsequent versions. Moreover, the message type values for D2R and R2D MAC PDU are separate.

Proposal 1: Apply message type to D2R MAC header as in R2D direction for forward compatibility.

Another issue is that in case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. How this is provided (i.e. SDU length field or padding length field) is FFS. We prefer that D2R MAC header includes a length field for D2R MAC SDU as in R2D MAC header design.   According to TR 38.848, the design target of maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device, based on the maximum application layer packet size. So the maximum MAC SDU is 1000 bits, the SDU length field in MAC header can be 7 bits. We can also reserve some reserved bits to facilitate the design of subsequent versions.

Proposal 2: A length field is supported for D2R MAC SDU in MAC header, in case where MAC PDU includes both MAC SDU and padding.

Proposal 3: The size of length field is at least 7 bits.

## 2.2 segmentation

RAN2 agreed that for the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. Purely “received offset=0” triggered first segmentation re-transmission or whole D2R message re-transmission may not work, since the D2R failure may be due previous R2D upper layer command failure. But whether offset zero is always included is FFS.

In order to unify the design, we prefer offset zero is always included in the retransmission scheduling message (e.g. R2D message includes the upper layer command) for the first segment/unsegmented D2R message. There only need one R2D message format to trigger D2R (re-)transmission.

Proposal 4: Offset zero is always included in the retransmission scheduling R2D message for the first segment/unsegmented D2R message.

Moreover, whether the reader always includes the command for (re)transmission of segments need to discussion. In RAN2#127bis meeting, RAN2 agreed that for the device we will not support AS layer buffering for A-IoT segmentation functionalities. We prefer reader always includes the upper layer command each time in the R2D message triggering the A-IoT device to (re)transmit the segment. This is beneficial in terms of reducing the complexity of the device, since device does not need to store the upper layer command and start/end location of the generated D2R message for subsequent segment. The cost is that the reader needs to repeatedly transmit the same upper layer service request to device in each scheduling message for the segments of the D2R message.

Proposal 5: Reader always includes the upper layer command each time in the R2D message triggering the A-IoT device to (re)transmit the segment.

## 2.3 Command type information

In SI phase, we discussed that command type (e.g. read/write/disable) should be visible to reader for effective resource scheduling, but no agreement was reached, and the conclusion was that we need to wait for SA2 LS response on expected D2R message size.

SA2 responded that AIOTF can provides the reader with the approximate D2R message size if provided by AF. If not provided by the AF, the D2R message size provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator, described as follows in SA2 TS 23.369.

5.4 Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command).

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

But the D2R message size information is optionally provided by AIOTF to reader. If AIOTF does not provide D2R message size to reader, the command type is useful information to reader for resource allocation. So AIOTF can provide command type to reader if it does not provide D2R message size.

Proposal 6: The command type (e.g., read, write, disable) can be indicated to reader by AIOTF, if D2R message size is not provided by AIOTF.

3 Conclusion

In this contribution we discuss the functionalities required for A-IoT, with the following proposals:

Proposal 1: Apply message type to D2R MAC header as in R2D direction for forward compatibility.

Proposal 2: A length field is supported for D2R MAC SDU in MAC header, in case where MAC PDU includes both MAC SDU and padding.

Proposal 3: The size of length field is at least 7 bits.

Proposal 4: Offset zero is always included in the retransmission scheduling R2D message for the first segment/unsegmented D2R message.

Proposal 5: Reader always includes the upper layer command each time in the R2D message triggering the A-IoT device to (re)transmit the segment.

Proposal 6: The command type (e.g., read, write, disable) can be indicated to reader by AIOTF, if D2R message size is not provided by AIOTF.

- References

- Draft report of 3GPP TSG RAN WG2 #129bis, RAN2 chair (interDigtal), Wuhan, China, 7 - 11 April, 2025.