3GPP TSG-RAN WG2 Meeting #130		R2-2503971

St Julians, Malta, 19th – 23rd May, 2025

Title: 	Measurement gap enhancements for XR

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.7.3

Document for:	Discussion and decision

# Introduction

At the RAN2#127bis meeting the following agreements were made for measurement gap skipping related enhancements for XR:

Agreements for RRM measurement gap skipping

- RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements.

- No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features

- RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear.

- RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4

- RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution.

In the last meeting, RAN4 has agreed on the usage of UAI for measurement occasion skipping and an LS R4-2504972 has been sent to R2:

Issue 5-1: General on UAI <Agreement>: • Agree to define UAI based on the following assumptions, and further discuss the content of UAI • RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) • Not define UE behaviour and the corresponding requirements based on UAI • Supporting of UAI is optional for UE with optional UE capability. And the following was agreed in RAN4 #114bis (R4-2504911): • The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. o R ∈ {0, 20, 40, 60}%

In this contribution, we look at further RAN2 aspects and propose a way forward.

# Discussion

Based on the RAN4 LS, a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s with value range of {0, 20, 40, 60}% can be reported by UAI, but when measurement Gap(s) are configured, the ratio (R) of gap occasions applies to all active measurement gap(s) or per measurement gap is not clear. Considering that the ratio (R) of gap occasions recommended for cancellation is used for network reference, e.g. to decide whether/when to skip the measurement Gap, and whether/when to skip the measurement Gap is based on the indication in DCI, the ratio (R) of gap occasions recommended for cancellation can be per UE and applies to all active measurement gap(s).

Proposal 1: RAN2 confirms the RAN4 LS that a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s with value range of {0, 20, 40, 60}% can be reported by UAI.

Proposal 2: the ratio (R) of gap occasions applies to applies to all active measurement gap(s).

Considering that the ratio (R) of gap occasions recommended for cancellation is mainly used to avoid collision between measurement Gap and delay critical data transmission and/or reception, it is only necessary in some use case. Hence, whether UE needs to report ratio (R) of gap occasions recommended for cancellation in UAI should be controlled by NW (e.g. an indication of reporting ratio (R) of gap occasions recommended for cancellation). Only if NW activates reporting ratio (R) of gap occasions recommended for cancellation, ratio (R) of gap occasions recommended for cancellation can be reported.

Proposal 3: Dedicated signalling is used to enable UAI for reporting the ratio of gap occasions that is recommended for cancellation – i.e. it is off by default.

In the RAN4 LS, RAN4 has agreed that supporting of UAI is optional for UE with optional UE capability. Only UE supports reporting ratio (R) of gap occasions recommended for cancellation, NW can an indication of reporting ratio (R) of gap occasions recommended for cancellation is included in DL dedicated signalling.

Proposal 4: RAN2 confirms that per UE capability is defined for reporting ratio (R) of gap occasions recommended for cancellation.

Considering that value range of the ratio (R) of gap occasions recommended for cancellation is {0, 20, 40, 60}%, the reporting triggering should consider the value changes, e.g. the ratio (R) of gap occasions recommended for cancellation is reported when:

- the indication of reporting ratio (R) of gap occasions recommended for cancellation is set, and

- the ratio (R) of gap occasions recommended for cancellation is available in the UE, and

- the ratio (R) of gap occasions value has not been provided to network, or the ratio (R) of gap occasions recommended for cancellation has changed after it is most recently sent to network and

- the prohibit timer for the reporting of UAI for this purpose is not running

Proposal 5: the ratio (R) of gap occasions recommended for cancellation may be reported by UAI when:

- the indication of reporting ratio (R) of gap occasions recommended for cancellation is enabled, and

- the ratio (R) of gap occasions recommended for cancellation is available in the UE, and

- the ratio (R) of gap occasions value has not been provided to network, or the ratio (R) of gap occasions recommended for cancellation has changed after it is most recently sent to network and

- the prohibit timer for the reporting of UAI for this purpose is not running

# Conclusion

The following proposals are made in this:

Proposal 1(RRC-6): RAN2 confirms the RAN4 LS that a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s with value range of {0, 20, 40, 60}% can be reported by UAI.

Proposal 2(RRC-6): the ratio (R) of gap occasions applies to applies to all active measurement gap(s).

Proposal 3(RRC-6): Dedicated signalling is used to enable UAI for reporting the ratio of gap occasions that is recommended for cancellation – i.e. it is off by default.

Proposal 4(RRC-6): RAN2 confirms that per UE capability is defined for reporting ratio (R) of gap occasions recommended for cancellation.

Proposal 5(RRC-6): the ratio (R) of gap occasions recommended for cancellation may be reported by UAI when:

- the indication of reporting ratio (R) of gap occasions recommended for cancellation is enabled, and

- the ratio (R) of gap occasions recommended for cancellation is available in the UE, and

- the ratio (R) of gap occasions value has not been provided to network, or the ratio (R) of gap occasions recommended for cancellation has changed after it is most recently sent to network and

- the prohibit timer for the reporting of UAI for this purpose is not running

# References

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#105

- 3GPP TS 38.321, V18.2.0 (2024-06), NR; Medium Access Control (MAC) protocol specification

- R4-2504972, Reply LS on UE assistance information, RAN4#114bis, April, 2025