3GPP TSG-RAN2#130									          R2-2503972

St Julian, Malta, 19th – 23rd May, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	LCP enhancements for XR

Agenda item:	8.7.4.1

Document for: 	Discussion and Decision

# Introduction

Rel-19 XR WI[1] has an objective for specify additional Logical Channel priority handling using delay/deadline information of packets to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

In this contribution we discuss the remaining issues related to LCP enhancements.

# Discussion

It has been agreed to introduce an independent per-LCH remaining time threshold for applying delay-critical priority in RAN2, and LCP prioritization within a logical channel will not be considered in RAN2 discussions, based on the threshold, “LCH priority-adjusted data” can be determined per LCH, e.g. if there is data in one logical channel with remaining time less than or equal to the threshold, all the data in the logical channel will be prioritized for transmission, and the data in the logical channel will be called “LCH priority-adjusted data”.

Proposal 1: RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel having data with remaining time less than or equal to the remaining time threshold configured for LCP.

In RAN2#129bis meeting[2], there is the following FFS:

Agreements on LCP enhancements - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

Although the WI objective is to “Specify additional Logical Channel priority handling using delay/deadline information of packets”, considering SR priority is also considered in LCP procedure for Intra-UE prioritization, the additional LCP priority for “LCH priority-adjusted data” can also used for SR priority determination. Only one Note to clarify this is enough.

Proposal 2: For “LCH priority-adjusted data”, the additional LCP priority is also used for SR priority determination. Add a note for this in the MAC specification.

In RAN2#129bis meeting[2], there is the following working assumption:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

Considering that unfairness will occur for non LCH priority-adjusted data if allowing an LCH with an upgraded priority to be transmitted even if Bj is negative. e.g. the LCH with small discardTimer will always be prioritized, and all the LCH will be scheduled with large delay when the discardTimer is same(e.g. always prioritize the data with remaining time less than the threshold). So, in the first LCP round, an LCH with an upgraded priority should not be allowed to be transmitted when Bj is negative.

Proposal 3: Confirm the working assumption that No Bj enhancement is introduced.

# Conclusion and proposals

The following proposals are made:

Proposal 1 (MAC-01): RAN2 confirms that “LCH priority-adjusted data” is all data in a logical channel having data with remaining time less than or equal to the remaining time threshold configured for LCP.

Proposal 2 (MAC-01): For “LCH priority-adjusted data”, the additional LCP priority is also used for SR priority determination. Add a note for this in the MAC specification.

Proposal 3: Confirm the working assumption that No Bj enhancement is introduced.

# References

- RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#106

- Draft_RAN2_129bis_Meeting_Report_v2