3GPP TSG-RAN WG2 Meeting #130                                  R2-2503985

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.1.2.3

Source:				Samsung

Title:					Discussion on LCM for POS use case

Document for:		Discussion & Decision

1	Introduction

This contribution provides our view on LCM for POS use case according to the objective in WID [1]. Please, note that the WID was updated in RAN#107 to delete the objectives for 2nd priority as below.

Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: o Direct AI/ML positioning:  (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning  (2nd priority) Case 2b: UE assited/LMF-based positioining with LMP side model, dirct AI/ML positioining  (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning o AI/ML assisted positioning  (2nd priority) Case 2s: UE-assisted/LMF-based positioining with UE-side model, AI/ML assited positioning  (1st priority)Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

2	Discussion

According to WID [1], there are three positioning sub use cases, which can be summarized as below.

| Case | Positioning type | Model entity | AI/ML for Pos type |
| --- | --- | --- | --- |
| 1 | UE-based positioning | UE-side model | Direct |
| 3a | NG-RAN node assisted positioning | gNB-side model | assisted |
| 3b | NG-RAN node assisted positioning | LMF-side model | Direct |

Until now, the following agreements were made for the use case 3a/b with gNB/LMF-side model.

For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and wait for any further inputs from other WGs For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

According to the agreement above, we do not see any potential RAN2 spec. impact for the case 3a/b (with gNB/LMF-side model). Thus, from RAN2 perspective, the case 3a/b can be deprioritized compared to the case 1 (with UE-side model). Also, since the case 2a/b were already deleted in the revised WID [1], RAN2 can focus on the use case 1 for now and further discuss other cases later if needed. Therefore, we discuss the LCM aspect mainly considering the case 1 with UE-side model in the following part.

2.1 Baseline LCM procedures for Case 1

In RAN2#127bis, the following baseline procedure is agreed for LCM for Case 1.

1: 	Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message.

Step 5: The LMF requests the inferred location information using the LPP request location information message.

Step 6: UE reports the inferred location using LPP provide location information message.

2: 	Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

3: 	Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress).

4: 	FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively).  FFS the signalling details.

5:   RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Among the open issues with FFS above, the yellow ones were resolved by the following agreements in RAN2#129 and RAN2#129bis.

Agreements (in RAN2#129)

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

Agreements (in RAN2#129bis)

- UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

However, there are the remaining green-colored issues marked as FFS and we would like to further discuss them one by one.

Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress):

According to the latest FL summary on POS case in RAN1 [2], “network-side additional condition” can be understood as below.

Understanding of “network-side additional condition”: Info about network side implementation details which the network side is not willing to send to UE explicitly, but such info is needed for UE-side model to ensure consistency between training and inference.

With the understanding above, RAN1 has discussed 
1) What information can be explicitly provided to UE as assistance data,
2) What information can be implicitly provided to UE as NW-side additional condition
and the following agreement was made in RAN1#119.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs).

Based on the agreement, the Info #7 can be regarded as candidate NW-side additional condition, but RAN1 is still discussing whether/how Info #7 is provided from LMF to UE as NW-side additional condition. RAN2 can just wait for RAN1’s conclusion on this.

Proposal. 1: RAN2 can wait for RAN1’s conclusion on NW-side additional condition.

2.2 Consistency between training and inference

For Case 1 positioning, the UE can have the model trained within a certain area A for AI positioning. Specifically, the model can be trained based on the measurement results of DL-PRS from the set of TRPs (e.g., TRP 1, TRP 2, TRP 3, TRP 4, TRP 5) within the area A. Now let’s assume that the UE receives the assistance data for inference in step 3, but the assistance data (i.e., DL-PRS information) is for the TRPs (e.g., TRP 1’, TRP 2’ TRP 3’ TRP 4’ TRP 5’) different from the ones used in training phase. In this case, we can consider two different scenarios for the reason of different TRPs in the assistance data.

In the first scenario, we can assume that the UE moves to a new area B and thus LMF provides the assistance data for the new TRPs within that area. In this case, it is reasonable for the UE to train/download a new model trained within the new area for the consistency between training and inference.

For the second case, the LMF may dynamically turn on/off the DL-PRS transmission from TRPs while the UE is still located within the area A. For example, the LMF turns off the DL-PRS Tx from the TRPs used in training phase and turn on the DL-PRS Tx from the new TRPs with some reason and provide the assistance data for the new TRPs. In this case, the UE can request LMF to provide the assistance data for a certain set of TRPs used in training phase and the existing On-demand DL-PRS request can be reused for this.

Proposal. 2: The legacy On-demand DL-PRS request can be reused for UE to request the assistance data for inference that is consistent with the one in training phase.

However, regarding any potential enhancement on the existing On-demand PRS, we think it should be discussed in RAN2 after RAN1 make conclusion on whether/how to provide NW-side additional condition (e.g., Geographical coordinates of the TRPs) in step 3. As described in the section 2.1 above, now RAN1 is considering several options on how to provide the UE with the NW-side additional condition (e.g., whether to use the associated ID or not). If RAN1 concludes to adopt the associated ID for the assistance data in step 3, it can be also used in the on-demand DL-PRS request message. Otherwise, the list of preferred TRPs can be directly included instead. Therefore, we prefer to postpone the discussion on the enhancement on the existing On-demand DL-PRS request in RAN2.

Proposal. 3: For the consistency between training and inference, RAN2 discuss any potential enhancement on the legacy on-demand DL-PRS request after RAN1 make conclusion on whether/how to provide NW-side additional condition (e.g., associated ID) in step 3.

2.3 Performance monitoring

For UE-side model case, the UE who deploy the model can do performance monitoring as baseline, but it may not be the only entity for performance monitoring. From our understanding, RAN1 is still considering at least the LMF as another monitoring entity for UE-side model case.

In RAN1#116bis, it is agreed to study the following options for label-based (model output based) monitoring.

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Also, in RAN1#119, RAN1 agreed to support at least Option A as below.

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

Since RAN1 agreed to support at least Option A, RAN2 can start to discuss the signaling aspect to support the option A. For each sub-options (A-x) above, the expected impact on LPP signalling can be summarized as below.

- Option A-1: LMF provides the target UE with the information on ground truth label of the target UE. Considering that this is the assistance information for performance monitoring, it seems reasonable that the LMF provides the information on ground truth label via LPP ProvideAssistanceData message.

- Option A-2: LMF provides the target UE with the position calculation assistance data. Considering that the existing information for UE-based positioning method is provided to UE via LPP ProvideAssistanceData message, any new position calculation assistance data for performance monitoring can be included in the same message.

- Option A-3: LMF provides the target UE with the PRU measurement & location. For the PRU information delivery, the existing Rel-18 PRU information (i.e., nr-PRU-DL-Info) in LPP ProvideAssistanceData message can be reused as already described in the RAN1 agreement above.

- Option A-4: PRU provides the target UE with the PRU measurement & location. According to RAN1 agreement, it can be realized by implementation in a manner transparent to specification. No spec. impact is foreseen now.

Based on the observation above, for all the sub-options A-x, the assistance data for performance monitoring can be provided by LMF to the target UE via LPP ProvideAssistanceData.

Proposal. 4: For POS Case 1, LMF can provide UE with the assistance data for performance monitoring metric calculation via LPP ProvideAssistanceData message.

For the open issue LPP-17 (Signalling of "ground-truth label" information), we understand that the LMF can provide the ground-truth label of the target UE as the assistance data for performance monitoring for Option A-1 and the existing LPP ProvideAssistanceData message can be reused for this. However, since there is no field to include the ground-truth label for the target UE in LPP ProvideAssistanceData message now, we can introduce a new field if RAN1 agree to support the Option A-1.

Proposal. 5: [LPP-17] For performance monitoring option A-1, LMF can provide the ground-truth label of the target UE via LPP ProvideAssistanceData message. A new field can be introduced to include the ground-truth label of the target UE if RAN1 decide to support the option A-1.

Meanwhile, RAN2 can also discuss how the UE reports the outcome of performance monitoring to the LMF. Based on the RAN1 discussion until now, it is understood that the UE can calculate the performance metric by evaluating the difference between the location estimate based on AI/ML method and the ground truth label. For the ground truth label, the UE can secure the label after receiving the assistance data from LMF via LPP ProvideAssistanceData. By the way, for the location estimate based on AI/ML method, the UE can obtain the location estimate as an output of AI/ML model inference after receiving a request from LMF via LPP RequestLocationInformation. Therefore, it seems reasonable for UE to report the output of performance monitoring via LPP ProvideLocationInformation as a response to the inference request from the LMF.

Proposal. 6: [LPP-16] For POS Case 1, UE can provide LMF with the outcome of performance monitoring via LPP ProvideLocationInformation message.

Since the selection of which sub-options (A-x) to support is still under discussion in RAN1, we can await RAN1's conclusion regarding the detailed content of the assistance data and the outcomes to prevent any potential duplication of efforts. Once RAN1 agrees to support a specific option and provides a detailed list of parameters to support that option, RAN2 can proceed with implementing the signaling details based on the RAN1 parameter list.

Proposal. 7: For performance monitoring in POS Case 1, the content of assistance data for monitoring metric calculation and the content of monitoring outcome can be determined/implemented later based on RAN1 parameter list.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Baseline LCM procedure for Case 1:

Proposal. 1: RAN2 can wait for RAN1’s conclusion on NW-side additional condition.

Consistency between training and inference:

Proposal. 2: The legacy On-demand DL-PRS request can be reused for UE to request the assistance data for inference that is consistent with the one in training phase.

Proposal. 3: For the consistency between training and inference, RAN2 discuss any potential enhancement on the legacy on-demand DL-PRS request after RAN1 make conclusion on whether/how to provide NW-side additional condition (e.g., associated ID) in step 3.

Performance monitoring:

Proposal. 4: For POS Case 1, LMF can provide UE with the assistance data for performance monitoring metric calculation via LPP ProvideAssistanceData message.

Proposal. 5: [LPP-17] For performance monitoring option A-1, LMF can provide the ground-truth label of the target UE via LPP ProvideAssistanceData message. A new field can be introduced to include the ground-truth label of the target UE if RAN1 decide to support the option A-1.

Proposal. 6: [LPP-16] For POS Case 1, UE can provide LMF with the outcome of performance monitoring via LPP ProvideLocationInformation message.

Proposal. 7: For performance monitoring in POS Case 1, the content of assistance data for monitoring metric calculation and the content of monitoring outcome can be determined/implemented later based on RAN1 parameter list.

4	Reference

- RP-250792, ‘Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface’, Qualcomm

- R1-2410718, ‘Summary #5 of specification support for positioning accuracy enhancement’, Ericsson