3GPP TSG-RAN WG2 #130	R2-2503989

St. Julian’s, Malta, 19 – 23 May 2025

Agenda item:		8.6.3	L1 event triggered measurement reporting

Title:		Remaining issues on MR MAC CE

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#129bis, there were some important agreements for event-triggered L1 measurement report [1]. Running CRs start capturing the agreements including the MAC running CR [2].

Agreements (related to L1 event-triggered MR)

- No L1/L2 filtering issue will be discussed in RAN2.

- For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

- When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

- When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

- The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

- FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

In this contribution, we discuss some remaining issues on MR MAC CE and provide our views.

# 2. Discussion

2.1	Inclusion of LTM candidate measurement in MR triggered by LTM2

There is the FFS on the MR triggered by the event LTM2.

- FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

We assume that the network may configure the LTM2 to trigger other events (e.g. LTM3/5) upon receiving the MR triggered by the LTM2. At that point in time, there is no strong need of having beam measurement information for LTM candidates. In other case, the network may configure the LTM2 in addition to the LTM3/5, where the LTM3/5 may be intended to trigger the early sync rather than LTM cell switch. In this case, the network can get useful information from those other events. In both cases, we assume only current beam information is enough.

Proposal 1: For MR triggered by LTM2, the UE includes only the current beam information in the MR MAC CE.

2.2	Event-periodic reporting

As agreed in the RAN#129bis as the working assumption and reflected in the MAC running CR, there are four types of beams, i.e. Type1-4 in the agreements and Type00-11 in the running CR [2], respectively. For the event-periodic reporting, at the first triggered MR the triggered beam is considered as the Type1 (Type00 in MAC CE). One issue to be clarified is to which type the following measurement reporting triggered by expiry of periodical reporting timer is categorized. Technically, we assume the beam should be the Type1. In other cases (i.e. during periodical reporting timer is running), it is the Type3 (Type10).

- Typei: This field indicates the type of the RS i of LTM candidate cell included in the event triggered L1 measurement report. The field is set to 00 to indicate the RS(s) that has satisfied the entry condition of the event associated with the report ID for TTT and triggers this measurement report MAC CE; it is set to 01 to indicate the RS(s) that has satisfied the leaving condition of the event associated with the report ID for TTT and triggers this measurement report MAC CE; it is set to 10 to indicate the RS(s) in the BEAM_TRIGGERED_LIST associated with the report ID as specified in clause 5.x.3 other than the RS(s) with Type set to 00; it is set to 11 to indicate the RS(s) not satisfying the event for TTT, if configured by network by allowReportAnyBeam specified in TS 38.331 [5]. The RS(s) not satisfying the event for TTT is selected based on the decending order of measured quantity. The length of the field is 2 bits; NOTE 1: RS(s) with Type set to 00 include the RS that has satisfied the entry condition of the event associated with the report ID for TTT and firstly triggers this measurement report MAC CE, and the RS(s) that have satisfied the entry condition of the event associated with the report ID for TTT after this measurement report MAC CE is triggered (either triggered by entry condition or leaving condition of the event associated with the report ID) and before this measurement report MAC CE is multiplexed.

In the current definition, a beam satisfied the entry condition for TTT and triggers this MR MAC CE is the Type1 (Type00). It is obvious that the expiry of periodical reporting timer also triggers the MR MAC CE, i.e. latter part of the Type1 definition is met. For the former part of the Type1 definition, it just says the beam satisfied the entry condition for TTT and it does not strictly define when it did. Once a beam satisfied the entry condition for TTT, the former part of the Type1 definition is also met. Therefore, it can be confirmed that the beam  triggering the MR MAC CE due to expiry of periodical reporting timer is also the Type1. At first, we would like to confirm this.

Proposal 2: RAN2 to agree that the beam triggering the MR MAC CE due to expiry of periodical reporting timer is also the Type1.

Following the Proposal 2, it would be good to discuss whether it is necessary to do any rewording or addition to the Type1 definition or the NOTE 1 to avoid any misinterpretation. Given that the NOTE 1 is kept, it would be simpler to add one more case of event-periodic reporting there.

Proposal 3: RAN2 to discuss need of clarification for event-periodic reporting, where one example is to add the following text at the end of the NOTE 1: “and the RS that triggers this measurement report MAC CE due to expiry of periodical reporting timer.”

2.3	Normal MR MAC CE vs Truncated MR MAC CE

Currently normal L1 measurement report MAC CE and the truncated L1 measurement report MAC CE are defined with the uniform format and differentiated by the corresponding eLCIDs [2]. The previous agreement is “When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE”. However, it is not entirely clear in which case it is the normal MR MAC CE (or truncated MR MAC CE). We discuss the detail to make it clearer. Note that in this part, we assume the P2 for the Type1 definition.

Firstly, it should be confirmed which beam(s) is essential to be reported and which beam(s) is not. From the network perspective, the Type1 and Type2 beams are very important, while the Type3 beam is relatively less important and the Type4 beam is nice to have. Therefore, we can consider that at least triggered Type1,2 beams must be reported by either normal MR MAC CE or truncated MR MAC CE.

Observation 1. Type1, 2 beams are essential beams in event-triggered measurement report, while Type3, 4 beams would be less important from network perspective.

Secondly, this clarification is also related to cancelling the triggered measurement report. Currently it is assumed that when the normal MR MAC CE is reported, the UE cancels the triggered measurement report. This would mean that when the truncated MR MAC CE is reported, there are some remaining beams to be reported, e.g. essential beams. With the above observation, we assume these remaining beams include at least one of Type1, 2 beams. Otherwise, the truncated MR MAC may include only less important beams, which sounds less efficient or does not make much sense.

Observation 2. Given that only normal MR MAC CE reporting cancels the triggered measurement report, the truncated MR MAC CE is used only when there are some remaining essential beams (e.g. Type1,2). Otherwise, it results in undesirable consequence that a triggered measurement report always includes maximum number of beams (i.e. up to N in total) by one or more MAC CEs.

Having had the Observation 2, we see yet another option for discussion on normal vs truncated MR MAC CE. The point is to reconsider the definition as well as the associated UE’s cancelling behaviour. The normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams, serving beam and the Type3, 4 beams up to N in total. Otherwise, the truncated MR MAC CE is used. This means that only the first reporting after triggering may be done by the normal MAC CE, if the UL grant is sufficient.

In this option, we also need to define when to cancel the measurement report. In addition to the case of using normal MAC CE, the measurement report should be cancelled after including all the essential beams. Otherwise, the same issue is expected as discussed above, i.e. measurement report always includes maximum number of beams (i.e. up to N in total) by one or more MAC CEs.

Observation 3. As an alternative to the current approach in the running CR, following approach may be useful:

- Normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams, serving beam and the Type3, 4 beams up to N in total. Otherwise, truncated MR MAC CE is used.

- UE cancels the measurement report when the normal MR MAC CE is used, or if there is no remaining Type1, 2 beam when the truncated MR MAC CE is used.

Based on the observations above, we propose to discuss further details of normal or truncated MR MAC CE by referring to two alternative approaches and decide which way to go.

Proposal 4: RAN2 to select one of the following two approaches:

- Approach 1 (using current running CR as baseline)

  - Normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams and serving beam at least. Otherwise, the truncated MR MAC CE is used.

  - UE cancels the measurement report when the normal MR MAC CE is used.

- Approach 2 (alternative)

  - Normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams, serving beam and the Type3, 4 beams up to N in total. Otherwise, the truncated MR MAC CE is used.

  - UE cancels the measurement report when the normal MR MAC CE is used, or if there is no remaining Type1, 2 beam when the truncated MR MAC CE is used.

# from MAC running CR [2] For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId of the serving cell included in the LTM-CSI-ReportConfig, the MAC entity shall: 1> if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled: 2> if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.xx; 3> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId for the RS triggered the report by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; 3> cancel the the triggered L1 measurement report; 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.xx; 3> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId for the RS triggered the report by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; 2> else: 3> if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured: 4> trigger the SR using the dedicated SR configuration for L1 measurement report. NOTE X: After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE.

2.4	Handling of remaining MR caused by insufficient grant

In the running CR, there is the NOTE X to leave it to UE implementation which beam(s) is selected among the remaining beams after sending the truncated MR MAC CE. Following our discussions above, the situation is that the remaining beams also include at least one Type1 beam and/or Type2 beam. Otherwise, the UE should cancel the measurement report.

With the assumption above, we think at least the Type1, 2 beams should be prioritized over the Type3, 4 beams. Smart UE implementation will achieve this prioritization, while it is not sure if there is common understanding about the prioritization between the Type3 and Type4 beams. From the network perspective, the Type3 is relatively important than the Type4. Selection among the beams of the same type can be fully up to UE implementation. We would like to confirm that at least prioritization among the Types should be like Type1 > Type2 > Type3 > Type4 and discuss whether to capture this in the specification (e.g. NOTE) or only in the Chair notes.

Proposal 5: RAN2 to confirm that the UE prioritizes the beams in the order of Type1, Type2, Type3 and Type4 for L1 measurement report. Prioritization among beams of the same type is left to UE implementation.

Proposal 5a: RAN2 to discuss whether to capture the P5 in the specification (e.g. NOTE) or in the Chair notes.

# 3. Conclusion

In this contribution we discussed remaining issues for MR MAC CE and made the following proposals.

Event LTM2:

Proposal 1: For MR triggered by LTM2, the UE includes only the current beam information in the MR MAC CE.

Event-periodic reporting:

Proposal 2: RAN2 to agree that the beam triggering the MR MAC CE due to expiry of periodical reporting timer is also the Type1.

Proposal 3: RAN2 to discuss need of clarification for event-periodic reporting, where one example is to add the following text at the end of the NOTE 1: “and the RS that triggers this measurement report MAC CE due to expiry of periodical reporting timer.”

Normal MR MAC CE vs Truncated MR MAC CE:

Note: In this part, we assume the P2 for the Type1 definition.

Observation 1. Type1, 2 beams are essential beams in event-triggered measurement report, while Type3, 4 beams would be less important from network perspective.

Observation 2. Given that only normal MR MAC CE reporting cancels the triggered measurement report, the truncated MR MAC CE is used only when there are some remaining essential beams (e.g. Type1,2). Otherwise, it results in undesirable consequence that a triggered measurement report always includes maximum number of beams (i.e. up to N in total) by one or more MAC CEs.

Observation 3. As an alternative to the current approach in the running CR, following approach may be useful:

- Normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams, serving beam and the Type3, 4 beams up to N in total. Otherwise, truncated MR MAC CE is used.

- UE cancels the measurement report when the normal MR MAC CE is used, or if there is no remaining Type1, 2 beam when the truncated MR MAC CE is used.

Proposal 4: RAN2 to select one of the following two approaches:

- Approach 1 (using current running CR as baseline)

  - Normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams and serving beam at least. Otherwise, the truncated MR MAC CE is used.

  - UE cancels the measurement report when the normal MR MAC CE is used.

- Approach 2 (alternative)

  - Normal MR MAC CE is used only when the MAC CE can include all the Type1, 2 beams, serving beam and the Type3, 4 beams up to N in total. Otherwise, the truncated MR MAC CE is used.

  - UE cancels the measurement report when the normal MR MAC CE is used, or if there is no remaining Type1, 2 beam when the truncated MR MAC CE is used.

Handling of remaining MR caused by insufficient grant:

Proposal 5: RAN2 to confirm that the UE prioritizes the beams in the order of Type1, Type2, Type3 and Type4 for L1 measurement report. Prioritization among beams of the same type is left to UE implementation.

Proposal 5a: RAN2 to discuss whether to capture the P5 in the specification (e.g. NOTE) or in the Chair notes.

# References

[1] R2-2502981, Report from session on R18 SL, R18/19 MOB and R19 NES, Vice Chairman (Samsung)

[2] [POST129b][117][MOB] (Vivo)

# Annex for reference [2]

@Discussion summary

  - Part I – Other open issue list

Rapporteur provides the list of open issues as below, and the corresponding suggestions on how to address them. Some of them could be further discussed based on contributions or resoved based on further progress. Companies are invited to provide comments on whether it is open issue and whether the suggestions from reapporteur is accuracy enough.

< skip>

    - Event triggered L1 measurement

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-5 | The content of L1 MR MAC CE triggered by LTM2 Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. | Issue Type: Essential How to address it: based on companies’ contribution |
| MAC-6 | Truncated L1 MR MAC CE details. E.g. Except the triggered beam, whether any beam should be prioritised to be included in the truncated L1 MR MAC CE? In case the UL grant could include at least 2 beams in truncated L1 MR MAC CE, which beam should be included as the first beam in the truncated L1 MR MAC CE? Based on following agreement: - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| MAC-7 | Whether one UL TB could include more than one truncated MAC CE or only one? For the case more than 1 triggering events are pending, if UL grant is sufficient for one regular L1 MR MAC CE or for two truncated L1 MR MAC CEs but insufficient for two regular L1 MR MAC CEs, the UE behaviour should be discussed. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| MAC-8 | FFS the coexistence between event triggered report and mTRP, e.g. how UE determine the current beam or candidate beam in this case? | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |