3GPP TSG-RAN WG2 Meeting #130	R2-2503991

St Julian’s, Malta, 19th –23rd May, 2025

Agenda Item:	8.2.4

Source:	Huawei, HiSilicon

Title:	A-IoT data transmission

Document for: Discussion and Decision

# 1	Introduction

Based on the current RAN2 progress, and focusing on the issues without conclusions, this contribution discusses AS ID, segmentation, and MAC format design.

# 2	Discussion

## 2.1	AS ID (Issue 3-3)

### 2.1.1	AS ID release agreement modification (Issue 3-3)

In the RAN2#129bis meeting, the issue of releasing AS ID was discussed:

9 The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely.

According to the current description, “new transaction for that device” means that devices which are not selected by the paging with new service cannot release AS IDs, and they may keep AS IDs indefinitely.

However, since the reader cannot be aware of which devices are selected, it cannot know whether the overlapped device(s) is selected by the new paging. Besides, the reader cannot know when the devices run out of power. Hence, to ensure the uniqueness of the used AS ID, the reader will consider the set of AS ID in the previous service is still occupied. Then, during the subsequent procedures of the new services, the collision probability between random ID and the used AS ID (used in current and last services) will be higher. The reader needs to allocate new AS ID more frequently. The Msg2 overhead will be significantly increased. Almost 100% additional Msg2 overhead when allocating an AS ID to a device, compared with the case in which a Random ID is re-used as the AS ID.

Figure 2.1.1-1	AS ID management by device according to the current agreement

It is suggested to remove “for that device” from the agreement. In the modified description, the AS ID is considered to be kept only during one service, and the above issues can be avoided.

Observation 1:	The reader is not aware which devices will be selected by the paging message of new service. Then, AS ID management per service is not feasible.

Proposal 1:	(Issue 3-3) The RAN2 agreement needs to be revised/clarified, since reader has no idea on which devices will be selected by the paging and the corresponding occupied AS IDs:

“The device releases the AS ID at least: upon receiving Paging with new transaction id for that device, i.e. different session/service”

### 2.1.2	Other conditions for AS ID release (Issue 3-3)

Other conditions for AS ID release need to be discussed. On the one hand, after the end of the procedure, timely AS ID release can provide more available AS IDs for subsequent procedures. On the other hand, the reader has no idea on when the next service arrives. If the next service never arrives, the devices will keep AS ID indefinitely (unless running out of power). To solve such issues, an explicit R2D message may be needed for AS ID release.

One effective way is that a device releases AS ID after the end of its procedure. There are two reader implementations for triggering the end of the procedure, which is discussed as follows:

- The procedure is terminated when the subsequent paging arrives. The subsequent paging messages with same transaction ID can be considered as the end of the procedure. In this design, the subsequent paging message should be sent after the reader handles all on-going devices in the current paging round. For example, in the inventory + command case, the reader needs to finish command procedure before sending the subsequent paging message, as shown in Figure 2.1.2-1.

- The procedure is still on-going when the subsequent paging arrives. To reduce the probability of collision between random IDs and occupied AS IDs during the same service period, a separate R2D message for AS ID release before the new service arrives is needed, as shown in Figure 2.1.2-1.

Figure 2.1.2-1	An example of automatically releasing AS ID upon subsequent Paging/separate R2D messages

Observation 2a:	There are different understandings or reader implementation as to the end of the procedure:

Manner 1: reader should complete the device procedure before next paging, if possible.

  - AS ID can be released upon receiving paging with same transaction ID for those successful device.

  - This is beneficial when one paging round handles thousands of device.

Manner 2: reader may continue the data transmission after/across the subsequent paging.

  - AS ID should not be release autonomously by device upon paging with same transaction ID.

  - Then, in this case, many AS ID are occupied before new service, which requires a separate R2D message to release AS ID before the new service.

Moreover, if a reader does not want to be strongly restricted to send the subsequent paging message after handling all on-going procedures, one way is to flexibly indicate whether to release or not in a paging message (e.g., 1-bit indication). By reader implementation, some subsequent paging messages trigger AS ID release, while other paging messages have no impact on AS ID release. For example, in the inventory + command case, the reader can determine which subsequent paging message is used to release AS ID, as shown in Figure 2.1.2-2.

Observation 2b:	Paging message with 1-bit indication control can allow both reader implementation manners above (i.e. release AS ID either by subsequent paging or by new paging).

Figure 2.1.2-2	An example of flexibly indicating whether a subsequent Paging message to release AS ID or not

Proposal 2:	(Issue 3-3) RAN2 to discuss the need of one broadcast R2D message to releases all devices’ AS ID (e.g., the paging message includes 1-bit indication).

### 2.1.3	AS ID release in CFRA (Issue 3-3)

For CFRA, if the AS ID used in the previous service is not released timely, there will be confusion when a device receives a command which is sent to the newly selected device. The device cannot distinguish whether the command is sent to it or not if the command procedure is processed across two services. For example, as shown in Figure 2.1.3-1, after the second paging message, the reader receives two R2D command from CN. If the reader cannot associate the two Device ID with different AS ID, the R2D Command may be associated to the wrong AS ID. This will result in the following transmission collision. Besides, the device behaviour is not defined if receiving an AS ID different from the stored one.

Hence, for CFRA, a device should release the AS ID at least upon receiving a paging message for a new service, and the command procedure should not be processed across two services.

Figure 2.1.3-1	An example of a procedure across two services

Observation 3:	For CFRA, the device should release the AS ID upon receiving paging for new service; otherwise, it may not work since reader cannot associate the “device ID” across two service requests from CN.

As for CFRA cases, NACK-based re-access is not supported, so the transaction ID is unnecessary. When initializing CFRA, each access occasion is allocated for the dedicated device from the reader. So, the reader knows whether the selected device successfully responds to the current paging message. If the device successfully responds to the paging message, the reader will not initialize a subsequent paging procedure. Otherwise, when the D2R message transmission failure occurs due to bad link quality, the reader needs to send a subsequent paging message triggering device to resend D2R message (e.g. device ID). Hence, for CFRA, the device should always respond to the paging message because transaction ID is unnecessary. Combined with Observation 2, it can be concluded that a device should release the stored AS ID upon receiving a paging regardless of the same or a new transaction ID.

Proposal 3:	(Issue 3-3) In CFRA, RAN2 confirms that device releases its AS ID, if any, upon receiving paging message.

### 2.1.4	Others

In the RAN3#127bis meeting, about the device association between gNB and AIOTF the following conclusion was reached:

RAN3#127bis agreement: - Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

Observation 4:	RAN3 agreed to use RAN NGAP Device ID: “Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.”

Based on this Device Association, if the reader associates the AS ID with the RAN NGAP Device ID, it can address the R2D command to the target device even if the R2D commands are sent out of order. This depends on reader implementation.

Observation 5:	For command case, it is reader implementation to maintain the association between AS ID and RAN NGAP Device ID for one specific device.

## 2.2	Segmentation (Issue 3-1, 3-2)

### 2.2.1	Device skips segmentation

The segmentation function requires the device to buffer the starting position information of the D2R SDU data. However, supporting additional buffer may increase complexity or power consumption. To give more flexibility of implementation, some devices can be allowed to not implement segmentation to reduce device complexity.

RAN2#129 agreements: For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3

Observation 6:	To further reduce device complexity, some device should be able to not implement segmentation.

By this consideration, if a device do not support segmentation, it can skip the segmentation procedure. That is, if the allocated TBS is not sufficient, the device can implement to skip (segmentation) transmission. From reader perspective, if it does not receive D2R message, then it can re-allocate TBS or skip this device. This depends on the reader implementation, and no capability reporting is needed. In most scenarios, the reader can know the devices' capability by deployment implementation.

Proposal 5:	RAN2 agrees that devices should be allowed to skip the D2R transmission/segmentation, when the allocated TBS is not sufficient to accommodate the entire D2R message.

### 2.2.2	Command and offset (Issue 3-1, 3-2)

As achieved in the last RAN2#129 meeting, offset information can be used for segmentation retransmission. When scheduling D2R (re-)transmission, the non-segmented packet or first segment transmission may fail in the following two cases:

- Case 1: The device fails to receive or misses the R2D upper-layer data, leading to the situation that device does not transmit D2R data.

- Case 2: The device receives the R2D upper-layer data and transmits the D2R data, but the reader fails to receive the D2R data.

When scheduling non-segmented packet or the first segment re-transmission, the reader cannot distinguish which case has caused the failure of D2R transmission. It is reasonable for the reader to retransmit R2D upper-layer data to ensure that both cases can be solved. If the reader only indicates offset=0 to the device, the device in case 1 will still be unable to (re)-transmit D2R data. Hence, including R2D upper-layer data in the R2D message can totally solve the issues caused by cases 1 and 2, while "received size/offset" is unnecessary.

For non-first segment (re-)transmission, the reader will determine that the device successfully received the R2D upper-layer data and exclude case 1. Hence, an offset should be included to schedule the non-first-segment D2R (re-)transmission. The R2D upper-layer data is unnecessary if the device store the starting position of D2R data in the upper layer.

Hence, there can be two alternatives of R2D message format designs for D2R scheduling:

Table 2.2.2-1	Two alternatives of R2D message format designs

|  | Alt. 1 |  | Alt. 2 |
| --- | --- | --- | --- |
| Field | Non-segmented message and the first segment scheduling | Non-first segment scheduling | Non-segmented packet, the first segment scheduling, and non-first segment scheduling |
| R2D upper-layer data | Included | Not included | Included |
| Received size/offset | Not included | Included | Included |

By Alt. 1, the R2D message size can be changed according to the specific cases, and the signaling overhead can be saved. But two different formats of R2D PDU for scheduling D2R data transmission need to be defined. Besides, for non-first segment scheduling, the device needs to store the starting position of D2R data.

By Alt. 2, the R2D PDU format can be designed to be unified. Then, device perform the same procedure (i.e. spec text) regardless of segmentation or not. For non-first segment scheduling, the device does not need to store the starting position of D2R data due to R2D upper-layer data carried in R2D message. For each segmentation transmission, device just needs to find the “segment part” from the indicated upper layer data according to the indicated offset. It does not matter whether the upper layer can always provide response each time.

Observation 7a:	For Alt.1, the overhead of R2D upper-layer command or offset field can be saved in different cases.

Observation 7b:	For Alt.2, for each segmentation, device implementation just picks the “segment part” from the upper layer data. It does not matter whether upper layers always provide upper layer response each time.

Proposal 6:	(Issue 3-1, 3-2) RAN2 to discuss two alternatives and decide between them:

Alt.1: Two formats of R2D messages are used to trigger the D2R (re-)transmission:

    - For the scheduling of non-segmented message or first segment, “R2D upper-layer command” is included and “received size/offset=0” is NOT included;

    - For the scheduling of non-first segment, “received size/offset” is included and “R2D upper-layer command” is NOT included.

Alt.2:	Single format of R2D message is used to trigger the D2R (re-)transmission: “R2D upper-layer command” and “received size/offset” are always included.

### 2.2.3	R2D message size limitation due to no segmentation

It has been agreed in RAN2 to not support R2D segmentation in this release. The assumption is that the write command size will not be large, normally. Taking RFID as an example, the main information provided by Write Command includes:

- Command (8-bit identifier of command types).

- MemBank (2-bit memory bank pointer).

- WordPtr (variable-bit word address). WordPtr uses format EBV (extensible bit vectors), which can indicate the word (16 bits) address in the memory bank specified by MemBank. It can use less bits to map a large range of index. For example, 128 addresses need 8 bits, and 16383 addresses only needs 16 bits.

- Data (16-bit data to be written after being XORed by RN16). Because, for ultra-low power consumption devices, the write latency is even longer than other processing time (e.g. R2D/D2R message processing). RFID only considers supporting Write Command including 16-bit data to be written each time as a mandatory command.

- RN16 (16-bit random number).

In RFID, a Read Command includes 8-bit Command, 2-bit MemBank, WordPtr (16 bits as an example), 8-bit WordCont (number of words to read), 16-bit RN. The Read Command total size is also not more than 100 bits.

Observation 8:	The typical size for one R2D command is not large, e.g. below 100bits.

In case there could be the case of large R2D command size, the CN can also do segmentation by implementation. Therefore, the R2D message sent to reader will not be large.

Then, it is worthy to inform the RAN2 conclusion to CT1, which may define the format and size limitation for the R2D NAS message.

Proposal 7:	Inform CT1 about the RAN2 conclusion on not supporting R2D segmentation for their considerations, e.g., as to the maximum value of R2D command message size.

### 2.2.4	Message size information

For command use case, command type information can help a reader to estimate the D2R response size from device if the D2R message size is not provided by CN. The reader can allocate an approximately accurate TBS for this D2R response. For example, to respond to a read command, the device should transmit data indicated by the read command, and the reader should schedule a D2R TBS which can cover data to be read. When the reader schedules a write command response or disable command response, the scheduled D2R TBS may just need to include much shorter acknowledgment information. The D2R message sizes of command responses are significantly different for different command types. Hence, if reader knows the command type information from the CN, the reader can allocate a more accurate TBS for the command response.

Proposal 8:	RAN2 suggests to RAN3 that the CN should indicate the command type (e.g., read, write, disable) to the reader.

## 2.3	MAC PDU format and others (Issue 2-3, 3-4, 3-6, 4-1)

### 2.3.1	Byte-alignment (Issue 2-3)

In RAN2#129bis meeting, a new R2D message (i.e., QueryRep-like message or Access Occasion Trigger message) was agreed to indicate the start of a set of access occasion(s). For the A-IoT MAC format design, the Bit-alignment looks also attractive, to save signalling overhead.

The Msg1 transmission related configurations are provided in paging message rather than the new R2D message.

Agreements on new R2D message - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

There is no other information needed in the “QueryRep-like” message. So it is just one trigger, which would be with short size (e.g. just 3- or 4-bit message type indication) but also occurs very frequently. So it does not need to be byte-aligned, otherwise, padding/reserved bit is required, which is just a waste of radio resource. For example, in order to trigger 1000 devices for inventory, almost 1000 “QueryRep-like” messages are needed when one time-domain resource is triggered by a “QueryRep-like” message. It requires an additional 4000 bits, if this MAC message is 8 bits which is byte aligned. In RFID [1], such design (see Figure 2.3.1-1) has already been adopted and works efficiently.

Figure 2.3.1-1	QueryRep command in RFID

Proposal 9:	(Issue 2-3) The R2D trigger message need not be byte-aligned.

### 2.3.2	D2R padding and length (Issue 3-4)

First, the reader can be aware of how many resources are needed for R2D transmission, so Padding in a R2D message is not needed. Moreover, for R2D transmission, RAN1 has concluded that postamble or TBS (via L1 R2D control information) information can determine the end of PRDCH. From device perspective, with the design in RAN1, the device can directly decode the end of the R2D transmission. Besides, multiplexing for R2D is not supported, so the R2D MAC PDU will not include other MAC SDUs or MAC elements. There will not be a SDU with an uncertain length. Hence, there is no need to introduce a length field in MAC for R2D assuming R2D MAC padding is not included.

RAN1#119 Agreement For determining the end of PRDCH at the device, following two options are studied and captured in the TR 38.769: - Option 1: TBS information (via implicit/explicit L1 R2D control information) - Option 2: Postamble (at the end of PRDCH)

Proposal 10:	(Issue 3-4) The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed.

In RAN2#129bis agreements, there is FFS SDU length field or padding length field is included in MAC for D2R.

RAN2#129bis Agreement In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS.

Considering that the reader can get some information for D2R data size from CN, it can schedule the D2R TBS accurately. For example, in command use cases, the reader will not blindly schedule a much larger D2R TBS in case of no D2R data size indicated. Even though the D2R data size is not indicated, the reader can first try to schedule a relatively small and reasonable TBS. If the TBS is smaller than D2R data, the segmentation procedure will be initialized. Hence, the SDU length can be much larger than padding length, and the length field in MAC should indicate the size of MAC padding. Besides, the MAC SDU is byte aligned, so the MAC padding is also considered to be in the unit of byte. Since, the maximum D2R size is not larger than 1,000 bits (125 bytes), the size of padding length field is not larger than 7 bits, and 5 or 6 bits can also work if the reader can schedule TBS more accurately. For example, if the MAC padding can be limited to be less than 32 bytes by accurately scheduling, 5-bit padding length field can work.

Proposal 11:	(Issue 3-4) The length field in MAC indicates the size of D2R MAC padding, in the unit of byte, using 7 bits. This (7 bits in the unit of byte) also applies to the “Received Data Size” field used for segmentation.

### 2.3.3	Upper layer data not available case (Issue 3-6)

In the current MAC running CR, there is the checking when performing the construction of D2R MAC PDU.

### 5.4.1 D2R message transmission Once the D2R Scheduling Info for D2R upper layer data transmission is received in the A-IoT Paging message or the Random ID Response message or the R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall: 1> if upper layer data is available to be transmitted: 2> if the size of the resulting MAC PDU including the total UL data is smaller than or equal to the resource size given by the D2R Scheduling Info: 3> generate the D2R Upper Layer Data Transfer message, including: 4> include the Data SDU field; 4> set the More Data Indication field to value 0; 4> if the size of the resulting MAC PDU including the total UL data is expected to be smaller than the resource size given by the D2R Scheduling Info: 5> include the MAC Padding field; 3> instruct the physical layer to transmit the generated D2R Upper Layer Data Transfer message; 2> else (the size of the resulting MAC PDU including the total UL data is larger than the resource size given by the D2R Scheduling Info): 3> the upper layer data SDU is to be segmented according to clause 5.4.3;

This sub-clause is to investigate whether there is the case of “If the upper layer data is NOT available to be transmitted”.

As to the write command, the SA2 replied LS R2-2500063 indicates the intention, as below, for the D2R response:

SA2 answer: No, a D2R response is always expected for a R2D message for command service type (e.g., the service data response for 'Read' service operation, and the operation result indication response for 'Write' or 'Disable' service operation).

CT1 may define this D2R response for write operation as: the D2R response indicates that the device has successfully completed the write operation. In that case, RAN2 may need to consider to support various device implementations. Depending on the power consumption used for write operation (based on the device implementation design) and the different write command size, the device upper layer may deliver the D2R response into MAC layer for transmission relatively fast or slow. This is to some extent unpredictable.

When the reader schedules the D2R transmission for write command response, the D2R upper layer response may not be ready yet, i.e., no upper layer data is available for D2R transmission. In such case, it is natural to put MAC padding for such D2R transmission. Otherwise, we have to design the tight coordination between device and reader regarding when the D2R upper layer data is available, e.g., something like BSR before transmission. Therefore, the least standard efforts are allowing the device to send MAC padding.

Observation 9a:	Different write command size and different device write speed (depending on the power consumption used for write operation) may cause unpredictable time when the upper layer delivers the “successful write” response to A-IoT MAC.

Observation 9b:	When the reader schedules the D2R transmission for write command response, the D2R upper layer data may not be available/ready yet. In this case, device can just send MAC padding, i.e., no other enhancement is needed.

Proposal 12a:	(Issue 3-6) The D2R response of write operation may cause the case of “the upper layer data is NOT available” upon D2R scheduling.

Proposal 12b:	(Issue 3-6) Allow device to send MAC padding without SDU, in case “the upper layer data is NOT available” upon D2R scheduling.

### 2.3.4	RAN1 parameters for D2R scheduling (Issue 4-1)

There could be some RAN1 parameters for D2R scheduling, e.g. some examples in R1-2502236.

- TBS;

- Preamble length;

- Midamble details;

- Apply FEC or not;

- Block repetition;

- X value, Interval between two consecutive Msg1 resource in time domain;

- Y value, {R, T_chip}.

Based on the RAN2 CBRA and CFRA cases for data transmission procedure, the following table is provided for the possible way to carry those parameters.

Table 2.3.4-1	How the D2R scheduling information can be carried by R2D messages

| RA type | D2R message | Paging | Msg2 | R2D Upper Layer Data Transfer message |
| --- | --- | --- | --- | --- |
| CBRA | Msg1, i.e., Random ID message | Provided by paging message | N/A | N/A |
|  | Msg3 in CBRA, i.e., D2R Upper Layer Data Transfer message | Provided by paging (e.g., some very basic configurations) and Msg2 (e.g., any parameters possibly different from Msg1) |  | N/A |
|  | Following D2R message, i.e., D2R Upper Layer Data Transfer message | Provided by paging (e.g., some very basic configurations), and… | N/A | …additionally, by R2D command (e.g., any parameters possibly different from Msg1) |
| CFRA | First D2R message, i.e., D2R Upper Layer Data Transfer message | Provided by paging message | N/A | N/A |
|  | Following D2R message, i.e., D2R Upper Layer Data Transfer message | Provided by paging (e.g., some very basic configurations), and… | N/A | …additionally, by R2D command (e.g., any parameters possibly different from Msg1) |

In the end, the final RAN2 design will be discussed after RAN1 provides us the parameter list.

Proposal 13:	(Issue 4-1) RAN1 will provide the D2R scheduling information parameter list, based on which RAN2 can further determine the information to put into paging message, Msg2 and R2D upper layer data message.

# 3	Conclusion

This contribution makes the following proposals:

Observation 1:	The reader is not aware which devices will be selected by the paging message of new service. Then, AS ID management per service is not feasible.

Observation 2a:	There are different understandings or reader implementation as to the end of the procedure:

Manner 1: reader should complete the device procedure before next paging, if possible.

  - AS ID can be released upon receiving paging with same transaction ID for those successful device.

  - This is beneficial when one paging round handles thousands of device.

Manner 2: reader may continue the data transmission after/across the subsequent paging.

  - AS ID should not be release autonomously by device upon paging with same transaction ID.

  - Then, in this case, many AS ID are occupied before new service, which requires a separate R2D message to release AS ID before the new service.

Observation 2b:	Paging message with 1-bit indication control can allow both reader implementation manners above (i.e. release AS ID either by subsequent paging or by new paging).

Observation 3:	For CFRA, the device should release the AS ID upon receiving paging for new service; otherwise, it may not work since reader cannot associate the “device ID” across two service requests from CN.

Observation 4:	RAN3 agreed to use RAN NGAP Device ID: “Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.”

Observation 5:	For command case, it is reader implementation to maintain the association between AS ID and RAN NGAP Device ID for one specific device.

Observation 6:	To further reduce device complexity, some device should be able to not implement segmentation.

Observation 7a:	For Alt.1, the overhead of R2D upper-layer command or offset field can be saved in different cases.

Observation 7b:	For Alt.2, for each segmentation, device implementation just picks the “segment part” from the upper layer data. It does not matter whether upper layers always provide upper layer response each time.

Observation 8:	The typical size for one R2D command is not large, e.g. below 100bits.

Observation 9a:	Different write command size and different device write speed (depending on the power consumption used for write operation) may cause unpredictable time when the upper layer delivers the “successful write” response to A-IoT MAC.

Observation 9b:	When the reader schedules the D2R transmission for write command response, the D2R upper layer data may not be available/ready yet. In this case, device can just send MAC padding, i.e., no other enhancement is needed.

AS ID release (Issue 3-3)

Proposal 1:	(Issue 3-3) The RAN2 agreement needs to be revised/clarified, since reader has no idea on which devices will be selected by the paging and the corresponding occupied AS IDs:

“The device releases the AS ID at least: upon receiving Paging with new transaction id for that device, i.e. different session/service”

Proposal 2:	(Issue 3-3) RAN2 to discuss the need of one broadcast R2D message to release all devices’ AS ID (e.g., the paging message includes 1-bit indication).

Proposal 3:	(Issue 3-3) In CFRA, RAN2 confirms that device releases its AS ID, if any, upon receiving paging message.

Segmentation (Issue 3-1, 3-2)

Proposal 5:	RAN2 agrees that devices should be allowed to skip the D2R transmission/segmentation, when the allocated TBS is not sufficient to accommodate the entire D2R message.

Proposal 6:	(Issue 3-1, 3-2) RAN2 to discuss two alternatives and decide between them:

Alt.1: Two formats of R2D messages are used to trigger the D2R (re-)transmission:

    - For the scheduling of non-segmented message or first segment, “R2D upper-layer command” is included and “received size/offset=0” is NOT included;

    - For the scheduling of non-first segment, “received size/offset” is included and “R2D upper-layer command” is NOT included.

Alt.2:	Single format of R2D message is used to trigger the D2R (re-)transmission: “R2D upper-layer command” and “received size/offset” are always included.

Proposal 7:	Inform CT1 about the RAN2 conclusion on not supporting R2D segmentation for their considerations, e.g., as to the maximum value of R2D command message size.

Proposal 8:	RAN2 suggests to RAN3 that the CN should indicate the command type (e.g., read, write, disable) to the reader.

MAC PDU format and others (Issue 2-3, 3-4, 3-6, 4-1)

Proposal 9:	(Issue 2-3) The R2D trigger message need not be byte-aligned.

Proposal 10:	(Issue 3-4) The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed.

Proposal 11:	(Issue 3-4) The length field in MAC indicates the size of D2R MAC padding, in the unit of byte, using 7 bits. This (7 bits in the unit of byte) also applies to the “Received Data Size” field used for segmentation.

Proposal 12a:	(Issue 3-6) The D2R response of write operation may cause the case of “the upper layer data is NOT available” upon D2R scheduling.

Proposal 12b:	(Issue 3-6) Allow device to send MAC padding without SDU, in case “the upper layer data is NOT available” upon D2R scheduling.

Proposal 13:	(Issue 4-1) RAN1 will provide the D2R scheduling information parameter list, based on which RAN2 can further determine the information to put into paging message, Msg2 and R2D upper layer data message.

# 4	References

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard.