3GPP TSG-RAN WG2 Meeting #130	R2-2504000

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.6

Source:	OPPO

Title:	Discussion on XR Rate Control

Document for:	Discussion, Decision

Introduction

This paper addresses the open issues related to XR rate control.

- [MAC-04] Which type of ID should be used to identify a QoS flow, e.g. DRB ID + QFI ID or some other identifier.

- [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows.

- [RRC-Issue1] FFS how to indicate whether bit rate query is enabled based on which granularity (QoS flow level or DRB level).

- [MAC-08] Handling of triggered UL rate queries (e.g. multiplexing, transmission and cancelation, etc).

- [MAC-09] How UE should handle the Rate Control MAC CE in DC configuration.

- [MAC-10] Behavior of bitRateQueryProhibitTimer.

Discussion

  - [MAC-04] Which type of ID should be used to identify a QoS flow

With the introduction of QoS flow level bit rate signalling, RAN2 needs to discuss how to indicate a QoS flow.

- FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

The QFI is uniquely designed within a PDU session/DRB in the legacy NR system. We can utilise DRB ID + QFI or PDU Session ID + QFI for identifying a specific QoS flow, preferring the DRB ID + QFI approach for its signalling efficiency and resilience to future changes.

maxNrofPDU-Sessions-r17 INTEGER ::= 256 -- Maximum number of PDU Sessions maxDRB INTEGER ::= 29 -- Maximum number of DRBs (that can be added in DRB-ToAddModList). maxQFI INTEGER ::= 63

[MAC-04] In the UL Rate Control MAC CE, use DRB ID + QFI to identify a specific QoS flow.

  - [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows

Following discussions in several meetings, RAN2#129bis has reached consensus on including QoS flow level information in the UL Rate Control MAC CE.

- From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

Each XR application may be associated with multiple QoS flows (e.g. video, audio), each with distinct QoS requirements. Thus, further discussion is needed on whether the UL Rate Control MAC CE includes indications for multiple QoS flows.

As the uplink rate control mechanism is meant to alleviate the congestion, there may be a need for the gNB to simultaneously adjust the bit rates of multiple QoS flows associated with the XR application(s). To minimise the signalling overhead from sending multiple MAC CEs (assuming one MAC CE per QoS flow), it is recommended to consolidate indications for multiple QoS flows in a single MAC CE.

[MAC-05] The UL Rate Control MAC CE can include bit rate indications for multiple QoS flows.

  - [RRC-Issue1] Granularity on indicating whether bit rate query is enabled

RAN2#129bis has confirmed the working assumption concerning the bit rate inquiry outlined below.

RAN2#129 - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). RAN2#129bis - The working assumption is changed to agreements on rate query MAC CE.

Furthermore, RAN2 has made additional agreements, leaving open issues on the details of the prohibit timer (e.g. granularity).

Agreements related to RRC CR - 1. For the available data rate query MAC CE, confirm on the following: - Introduce a prohibit timer for the UL transmission of the data rate query MAC CE - Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration - FFS the details of prohibit timer, e.g. granularity

For precise and timely rate query/control, the granularity of the prohibit timer for the bit rate query should match that of the UL Rate Control MAC CE. Thus, we prefer a per-QoS flow prohibit timer for the rate query MAC CE.

[RRC-Issue1] RAN2 introduces a per-QoS flow prohibit timer for the rate query MAC CE.

  - [MAC-08] Handling of triggered UL rate queries

According to the current MAC specification, the MAC entity generates the Recommended bit rate MAC CE when it has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP.

5.18.10 Recommended Bit Rate [omitted] If the MAC entity has UL resources allocated for new transmission the MAC entity shall: 1> for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled: 2> if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and 2> if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query; 3> start the bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query; 3> cancel this Recommended bit rate query.

This multiplexing rule may also apply to the rate query MAC CE. In case the UL resource cannot accommodate such a MAC CE, there is no urgent requirement to contain truncated query information, as it is a kind of UE assistance information.

[MAC-08] The MAC entity can generate the rate query MAC CE to include the information associated with the triggered bit rate query(s), when the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate the rate query MAC CE plus its subheader. Otherwise, the MAC entity does not generate and transmit the rate query MAC CE.

Concerning the cancellation of a triggered Recommended bit rate query, the MAC entity cancels this query upon the generation of the Recommended bit rate MAC CE for this query.

5.18.10 Recommended Bit Rate [omitted] If the MAC entity has UL resources allocated for new transmission the MAC entity shall: 1> for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled: 2> if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and 2> if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query; 3> start the bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query; 3> cancel this Recommended bit rate query.

We propose that the cancellation of the bit rate query should follow the same rule as that of the recommended bit rate query. Specifically, the MAC entity cancels a triggered bit rate query when the rate query MAC CE, including the information associated with this bit rate query, is generated.

[MAC-08] The MAC entity cancels a triggered rate query when the rate query MAC CE, including the information associated with this rate query, is generated.

  - [MAC-09] Handle the Rate Control MAC CE in DC

In the DC scenario, data from one DRB/QoS flow is sent across multiple carriers linked to different cell groups. In the event of congestion, the UE may obtain different recommended bit rate values for the same QoS flow from MN and SN. The UE behaviour needs clarification in this situation. The previously discussed DC duplication had a similar deliberation, and no special UE handling is introduced eventually; i.e., the UE always follows the latest MAC CE from MN or SN. We do not see a significant difference between this case and DC duplication.

[MAC-09] No special UE handling is introduced for the Rate Control MAC CE in the DC scenario.

  - [MAC-10] Behavior of bitRateQueryProhibitTimer

In the current MAC specification, for a triggered Recommended bit rate query, the MAC entity generates the Recommended bit rate MAC CE for that query if the UL resource is available and if the bitRateQueryProhibitTimer for that query is configured but not running. Once the Recommended bit rate MAC CE for that query is generated, the MAC entity starts the bitRateQueryProhibitTimer for the associated logical channel and direction. This prohibition rule may also apply to the bitRateQueryProhibitTimer for XR. In cases where the prohibit timer is designed per QoS flow, whether to report the bit rate information of a certain QoS flow is controlled by its own prohibit timer.

5.18.10 Recommended Bit Rate [omitted] The MAC entity may request the gNB to indicate the recommended bit rate for a specific logical channel and a specific direction. If the MAC entity is requested by upper layers to query the gNB for the recommended bit rate for a logical channel and for a direction (i.e. for uplink or downlink), the MAC entity shall: 1> if a Recommended bit rate query for this logical channel and this direction has not been triggered: 2> trigger a Recommended bit rate query for this logical channel, direction, and desired bit rate. If the MAC entity has UL resources allocated for new transmission the MAC entity shall: 1> for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled: 2> if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and 2> if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query; 3> start the bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query; 3> cancel this Recommended bit rate query.

[MAC-10] If the bitRateQueryProhibitTimer for a QoS flow associated with a triggered bit rate query is running, the MAC entity does not generate the rate query MAC CE for that query.

[MAC-10] Upon generating the rate query MAC CE for a triggered bit rate query, the MAC entity starts the bitRateQueryProhibitTimer for the associated QoS flow.

Conclusion

We have the following proposals:

Proposal 1	[MAC-04] In the UL Rate Control MAC CE, use DRB ID + QFI to identify a specific QoS flow.

Proposal 2	[MAC-05] The UL Rate Control MAC CE can include bit rate indications for multiple QoS flows.

Proposal 3	[RRC-Issue1] RAN2 introduces a per-QoS flow prohibit timer for the rate query MAC CE.

Proposal 4	[MAC-08] The MAC entity can generate the rate query MAC CE to include the information associated with the triggered bit rate query(s), when the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate the rate query MAC CE plus its subheader. Otherwise, the MAC entity does not generate and transmit the rate query MAC CE.

Proposal 5	[MAC-08] The MAC entity cancels a triggered rate query when the rate query MAC CE, including the information associated with this rate query, is generated.

Proposal 6	[MAC-09] No special UE handling is introduced for the Rate Control MAC CE in the DC scenario.

Proposal 7	[MAC-10] If the bitRateQueryProhibitTimer for a QoS flow associated with a triggered bit rate query is running, the MAC entity does not generate the rate query MAC CE for that query.

Proposal 8	[MAC-10] Upon generating the rate query MAC CE for a triggered bit rate query, the MAC entity starts the bitRateQueryProhibitTimer for the associated QoS flow.

Reference

[1] RP-250107 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)