3GPP TSG-RAN WG2 Meeting #130	R2-2504023

# St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1909 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing UE capability for A4 A5 ReportOnLeave |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips, Qualcomm Incorporated, Samsung, MediaTek Inc, Xiaomi, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_Aerial-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In LTE RRC spec, A4/A5 ReportOnLeave has been supported since Rel-15, and new RRC parameter (i.e. a4-a5-ReportOnLeave-r15) was introduced in ReportConfigEUTRA. However, no explicit UE capability has been introduced for this, this causes problem in implementation at both network side and UE side. For example, it is unclear whether all Rel-15 version E-UTRAN UEs should by default support A4/A5 ReportOnLeave or not. In RAN2#129bis meeting, RAN2 discussed this issue and decided to introduce an optional capability signalling for this. This CR introduces the UE capability parameter that indicated above. |  |  |  |  |  |  |  |  |
|  |  | “ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce new UE capability “a4-a5-ReportOnLeaveSupport-r15” in UE-EUTRA-Capability, to indicate whether the UE supports the configuration of a4-a5-ReportOnLeave-r15 in report configuration and triggering measurement report when the leaving condition is met for A4 and A5 events. Impact analysis Impacted 5G architecture options: LTE SA, (NG)EN-DC, NE-DC Impacted functionality: A4/A5 based L3 measurement Inter-operability: - If the UE is implemented according to the CR and the network is not, the network is unclear in which case a4-a5-ReportOnLeave can be configured, and failure may happen if the network configures a4-a5-ReportOnLeave and the UE does not support the feature. - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Missing UE capability for A4/A5 ReportOnLeave configuration and reporting. From UE perspective, it is unclear whether all Rel-15 should mandatory support this; from network perspective, it is unclear in which cases the network can configure a4-a5-ReportOnLeave-r15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.xx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.331 CR 5119 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2502333 based on the conclusion made in RAN2#129bis |  |  |  |  |  |  |  |  |

Start of change

4.3.6	Measurement parameters

4.3.6.1	interFreqNeedForGaps and interRAT-NeedForGaps

These fields define for each supported E-UTRA band whether measurement gaps are required to perform inter-frequency measurements on each supported E-UTRA radio frequency band and inter-RAT measurements on each supported RAT/band combination. A UE also indicates for each band combination as in the supportedBandCombination whether measurement gaps are required to perform inter-frequency measurements on each supported E-UTRA radio frequency band and inter-RAT measurements on each supported RAT/band combination.

4.3.6.2	rsrqMeasWideband

This field defines whether the UE can perform RSRQ measurements in RRC_IDLE and RRC_CONNECTED with wider bandwidth as specified in TS 36.133 [16].

4.3.6.3	timerT312-r12

This field defines whether the UE supports T312 as specified in TS 36.331 [5].

4.3.6.4	alternativeTimeToTrigger-r12

This field defines whether the UE supports alternativeTimeToTrigger as specified in TS 36.331 [5].

4.3.6.5	benefitsFromInterruption-r11

This field indicates whether the UE power consumption could benefit from being allowed to cause interruptions to serving cells when performing measurements of deactivated SCell carriers for measCycleSCell of less than 640ms, as specified in TS 36.133 [16].

4.3.6.6	incMonEUTRA-r12

This field defines whether the UE supports increased number of E-UTRA carrier monitoring in RRC_IDLE and RRC_CONNECTED as specified in TS 36.133 [16], and whether the UE supports extended number of cell re-selection priorities for EUTRA frequencies in RRCConnectionRelease, as specified in TS 36.331 [5]. It is mandatory for UEs of this release of the specification, except for Category 0 and 1bis UEs.

A UE that supports increased number of E-UTRA carrier monitoring shall also support extended number of measurement identities.

4.3.6.7	incMonUTRA-r12

This field defines whether the UE supports increased number of UTRA carrier monitoring in RRC_IDLE and RRC_CONNECTED as specified in TS 36.133 [16].

A UE that supports increased number of UTRA carrier monitoring shall also support extended number of measurement identities.

4.3.6.8	extendedMaxMeasId-r12

This field defines whether the UE supports extended number of measurement identities as defined by maxMeasId-r12 in TS 36.331 [5].

It is mandatory for UEs of this release of the specification if incMonEUTRA-r12 or incMonUTRA-r12 or dc-Support-r12 or extendedMaxObjectId-r13 is supported.

4.3.6.9	crs-DiscoverySignalsMeas-r12

This field defines whether the UE supports CRS based discovery signals measurement as specified in TS 36.331 [5], and PDSCH/EPDCCH RE mapping with zero power CSI-RS configured for discovery signals.

4.3.6.10	csi-RS-DiscoverySignalsMeas-r12

This field defines whether the UE supports CSI-RS based discovery signals measurement as specified in TS 36.331 [5]. A UE that supports this feature shall also support crs-DiscoverySignalsMeas-r12.

4.3.6.11	extendedRSRQ-LowerRange-r12

This field defines whether the UE supports the extended RSRQ lower value range from -34dB to -19.5dB in measurement configuration and reporting as specified in TS 36.133 [16].

4.3.6.12	rsrq-OnAllSymbols-r12

This field defines whether the UE supports the RSRQ measurement on all OFDM symbols as specified in TS 36.214 [23] and also the extended RSRQ upper value range from -3dB to 2.5dB in measurement configuration and reporting as specified in TS 36.133 [16]. If the UE supports rsrq-OnAllSymbols-r12 and rsrqMeasWideband it shall also support the RSRQ measurement on all OFDM symbols with wider bandwidth.

4.3.6.13	rs-SINR-Meas-r13

This field defines whether the UE can perform RS-SINR measurements in RRC_CONNECTED as specified in TS 36.214 [23].

4.3.6.14	allowedCellList-r13

This field defines whether the UE supports configuration and use of allow-listed cells as specified in TS 36.331 [5].

4.3.6.15	extendedFreqPriorities-r13

This field defines whether the UE supports extended E-UTRA frequency priorities as specified in TS 36.331 [5] and indicated by cellReselectionSubPriority field.

A UE supporting NR SA operation shall support extended E-UTRA frequency priorities and NR frequency priorities as specified in TS 36.331 [9] and indicated by CellReselectionSubPriority field.

4.3.6.16	extendedMaxObjectId-r13

This field defines whether the UE supports extended number of measurement object identities as defined by maxObjectId-r13 in TS 36.331 [5]. The field is mandatory present for the UE supporting the configuration of sCellToAddModListExt. A UE indicating support of extendedMaxObjectId-r13 shall also indicate the support of extendedMaxMeasId-r12.

4.3.6.17	ul-PDCP-Delay-r13

This field defines whether the UE supports UL PDCP Packet Delay per QCI measurement as specified in TS 36.314 [25]. A UE that supports the UL PDCP Delay measurement shall also support the measurement configuration and reporting as specified in TS 36.331 [5].

4.3.6.18	Void

4.3.6.19	rssi-AndChannelOccupancyReporting-r13

This field defines whether the UE supports measurement and reporting for RSSI and channel occupancy. This field is only applicable if the UE supports downlink LAA operation.

4.3.6.20	multiBandInfoReport-r13

This field defines whether the UE supports the acquisition and reporting of multi band information for reportCGI as specified in TS 36.331 [5].

4.3.6.21	Void

4.3.6.22	Void

4.3.6.23	ceMeasurements-r14

This field defines whether the UE supports intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED, as specified in TS 36.133 [16], TS 36.304 [14] and TS 36.331 [5]. In this release of specification, it is mandatory for UEs of Category M1 and M2 and UEs that support coverage enhancements to support ceMeasurements-r14. A UE indicating support of ceMeasurements-r14 shall also indicate support of ce-ModeA-r13.

4.3.6.24	ncsg-r14

This field defines whether the UE supports NCSG gap as specified in TS 36.133 [16]. If the UE supports ncsg-r14 and asynchronous DC, the UE shall support NCSG Pattern Id 0, 1, 2 and 3. If the UE supports ncsg-r14 but the UE does not support asynchronous DC, only NCSG Pattern Id 0 and 1 shall be supported.

4.3.6.25	perServingCellMeasurementGap-r14

This field defines whether the UE supports per CC measurement gap as specified in TS 36.331 [5].

4.3.6.26	shortMeasurementGap-r14

This field defines whether the UE supports shorter measurement gap length (i.e. gp2 and gp3) in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [37].

4.3.6.27	nonUniformGap-r14

This field defines whether the UE supports measurement non uniform Pattern Id 1, 2, 3 and 4 in LTE standalone as specified in TS 36.133 [16].

4.3.6.28	rlm-ReportSupport-r14

This field defines whether the UE supports RLM event and information reporting as specified in TS 36.133 [16].

4.3.6.29	Void

4.3.6.30	qoe-MeasReport-r15

This field defines whether the UE supports QoE Measurement Collection for streaming services.

4.3.6.31	ca-IdleModeMeasurements-r15

This field defines whether the UE supports performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode, including reporting them when requested by eNB while in RRC_CONNECTED, as specified in TS 36.331 [5].

4.3.6.32	ca-IdleModeValidityArea-r15

This field defines whether the UE supports configuration of validityArea for performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE mode, as specified in TS 36.331 [5]. A UE that supports this feature shall also indicate support of ca-IdleModeMeasurements-r15.

4.3.6.33	qoe-MTSI-MeasReport-r15

This field defines whether the UE supports QoE Measurement Collection for MTSI services.

4.3.6.34	multipleCellsMeasExtension-r15

This field defines whether the UE supports measurement reporting triggered based on a number of cells.It is mandatory to support this feature for UEs which have Aerial UE subscription as defined in TS 23.401 [18].

4.3.6.35	heightMeas-r15

This field defines whether the UE supports height-based measurement reporting as specified in TS 36.331 [5]. It is mandatory to support this feature for UEs which have Aerial UE subscription as defined in TS 23.401 [18].

4.3.6.36	measGapPatterns-r15

This field defines whether the UE that supports NR supports gap patterns 4 to 11 in LTE standalone as specified in TS 36.133 [16], and for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC as specified in TS38.133 [37].

4.3.6.37	dl-ChannelQualityReporting-r16

This field indicates whether the UE supports DL channel quality reporting of the configured carrier for FDD in RRC_CONNECTED as specified in TS 36.321 [4]. This feature is only applicable if the UE supports any ue-Category-NB.

4.3.6.37a	ce-DL-ChannelQualityReporting-r16

This field indicates whether the UE supports DL channel quality reporting of the serving cell when the UE is operating in coverage enhancement mode A or B in RRC_CONNECTED as specified in TS 36.321 [4]. A UE indicating support of ce-DL-ChannelQualityReporting-r16 shall also indicate support of ce-ModeA-r13.

4.3.6.38	interRAT-NeedForGapsNR-r16

This field defines for each supported E-UTRA band or band combination whether measurement gaps are required to perform SSB based inter-RAT measurements on each supported NR band.

4.3.6.39	ce-MeasRSS-Dedicated-r16

This field indicates whether the UE supports improved DL RSRP measurement accuracy through use of RSS in RRC_CONNECTED, and whether the UE supports measurement of neighbour cell RSS in the same narrowband as the MPDCCH, when the UE is operating in coverage enhancement mode A or B as specified in 36.133 [16]. A UE indicating support of ce-MeasRSS-Dedicated-r16 shall also support resynchronization signals as defined in 6.8.8.

4.3.6.39a	ce-MeasRSS-DedicatedSameRBs-r16

This field indicates whether the UE supports improved DL RSRP measurement accuracy through use of RSS in RRC_CONNECTED, and whether the UE supports measurement of neighbour cell RSS in the same 2-RBs as the serving cell RSS 2-RBs, when the UE is operating in coverage enhancement mode A or B as specified in 36.133 [16]. A UE indicating support of ce-MeasRSS-Dedicated-r16 shall also support resynchronization signals as defined in 6.8.8. A UE indicating support of ce-MeasRSS-DedicatedSameRBs-r16 shall not indicate support of ce-MeasRSS-Dedicated-r16.

4.3.6.40	eutra-IdleInactiveMeasurements-r16

This field defines whether the UE supports:

-	(if the UE also indicates support of inactiveState-r15), performing eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_INACTIVE, including reporting them when requested by the eNB while resuming from RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5];

-	(if the UE also indicates support of RRC connection suspension), reporting eNB-configured CRS-based RRM measurements for configured carrier(s) in RRC_IDLE while resuming the RRC connection from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5];

A UE that indicates support of this feature shall also indicate support of ca-IdleModeMeasurements-r15.

4.3.6.41	nr-IdleInactiveMeasFR1-r16

This field defines whether the UE supports performing eNB-configured SSB-based RRM measurements for configured NR FR1 carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE also indicates support of inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5].

4.3.6.42	nr-IdleInactiveMeasFR2-r16

This field defines whether the UE supports performing eNB-configured SSB-based RRM measurements for configured NR FR2 carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE also indicates support of inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5].

4.3.6.43	idleInactiveValidityAreaList-r16

This field defines whether the UE supports configuration of validityAreaList-r16 for performing eNB-configured measurements for configured carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE supports inactiveState-r15), as specified in TS 36.331 [5].

A UE that indicates support of this feature shall also indicate support of eutra-IdleInactiveMeasurements-r16 or nr-IdleInactiveMeasFR1-r16 or nr-IdleInactiveMeasFR2-r16.

4.3.6.44	measGapPatterns-NRonly-r16

This field indicates whether the UE supports gap patterns 2, 3 and 11 in LTE standalone when the frequencies to be measured within this measurement gap are all NR frequencies.

4.3.6.45	measGapPatterns-NRonly-ENDC-r16

This field indicates whether the UE supports gap patterns 2, 3 and 11 in (NG)EN-DC when the frequencies to be measured within this measurement gap are all NR frequencies.

4.3.6.46	nr-IdleInactiveBeamMeasFR1-r16

This field defines whether the UE supports performing eNB-configured SSB-based beam level RRM measurements for configured NR FR1 carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE also indicates support of inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5].

A UE that supports this feature shall also support nr-IdleInactiveMeasFR1-r16.

4.3.6.47	nr-IdleInactiveBeamMeasFR2-r16

This field defines whether the UE supports performing eNB-configured SSB-based beam level RRM measurements for configured NR FR2 carrier(s) in RRC_IDLE and in RRC_INACTIVE (if the UE also indicates support of inactiveState-r15), including reporting them when requested by the eNB while resuming from RRC_IDLE/RRC_INACTIVE or in RRC_CONNECTED, as specified in TS 36.331 [5].

A UE that supports this feature shall also support nr-IdleInactiveMeasFR2-r16.

4.3.6.48	nr-RSSI-ChannelOccupancyReporting-r17

This field indicates whether the UE supports performing measurements and reporting of RSSI and channel occupancy on the corresponding NR band. If both sharedSpectrumMeasNR-EN-DC-r17 and sharedSpectrumMeasNR-SA-r17 are included, the UE shall set the value of nr-RSSI-ChannelOccupancyReporting-r17 consistently for the same NR band.

4.3.6.49	connModeMeasIntraFreq-r17

This field defines whether the UE supports intra-frequency neighbour cell measurements in RRC_CONNECTED, as specified in TS 36.133 [16] and TS 36.331 [5]. This field is only applicable for UEs of any ue-Category-NB.

4.3.6.50	connModeMeasInterFreq-r17

This field defines whether the UE supports inter-frequency neighbour cell measurements in RRC_CONNECTED, as specified in TS 36.133 [16] and TS 36.331 [5]. This field is only applicable for UEs of any ue-Category-NB.

4.3.6.51	nr-CellIndividualOffset-r16

This parameter defines whether the UE supports use of cell specific offset for NR inter-RAT measurements in LTE for reporting of NR neighbours as specified in TS 36.331 [5].

#### 4.3.6.52	gaplessMeas-FR2-maxCC-r17

This field defines whether the UE supports inter-RAT NR FR2 measurement without measurement gap as specified in clause 9.1.2 of TS 38.133 [37] while the number of configured serving cells is less than or equal to the indicated number. This field is applicable when only E-UTRA serving cells are configured. The UE reporting this field and supporting (NG)EN-DC shall not indicate support of independentGapConfig in MeasAndMobParametersMRDC (defined in TS 38.306 [32]).

4.3.6.53	Void

4.3.6.54	Void

4.3.6.xx	a4-a5-ReportOnLeaveSupport-r15

This field defines whether the UE supports measurement reporting when the leaving condition is met for A4 and/or A5 event as specified in TS 36.331 [5].

End of change