3GPP TSG-RAN WG2 Meeting #130                                    R2-2504040

St.Julians, Malta, May 19th – 23rd , 2025

Agenda Item:	8.11.3

Source:	vivo

Title:	SBFD other aspects

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss other aspects of SBFD.

Discussion

Whether to support SBFD operation on SUL

A serving cell can be configured with supplementary uplink (SUL) for the purpose of coverage enhancement. In the Rel-18 WI Coverage Enhancement (CovEnh), it was discussed and determined that:

- Msg1 repetition can be configured on SUL;

- It is up to the NW whether to configure both NUL with Msg1/3 repetition and SUL for the purpose of CovEnh;

According to the WID [1], SBFD operation is only supported on the TDD carrier, where UL sub-bands are configured on downlink or flexible symbols. Considering SUL is usually deployed with a paired downlink carrier, it seems SBFD RACH operation does not share the intention to be deployed on SUL. However, according to the draft running CR [2], SBFD related RACH configuration can be configured in SUL IE. Therefore, we propose RAN2 to confirm that SBFD RACH operation is not supported on SUL and revise the running CR accordingly.

Proposal 1: SBFD operation is not supported on SUL.

Conclusion

Based on the discussion, we have the following observation and proposals:

Proposal 1: SBFD operation is not supported on SUL.

References

- RP-241614 Revised WID - Evolution of NR duplex operation Sub-band full duplex (SBFD)

- [Post129bis][212][SBFD] Running CR for 38.331 (Huawei)