3GPP TSG-RAN WG2 Meeting #130		R2-2504051

St Julian’s, Malta, May 19th – 23rd 2025

Agenda Item: 		8.1.2.2

Source:			Sony

Title:	Discussion on LCM for UE side model for Beam Management

Document for:		Discussion & Decision

# Introduction

In this contribution we provide our views on model monitoring aspects for UE side model for beam management, including what UE should report and how to report to NW for performance monitoring.

# Discussions

## 2.1 Model monitoring resource optimization

For the BM-Case1 and BM-Case2 with a UE-side AI/ML model, Type 1 performance monitoring including the following two options are supported in RAN1 [1] as below:

Option 1 (NW-side performance monitoring): · UE sends a report to NW (for the calculation of performance metric at NW) o Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report o FFS on other contents · The report is at least configured/triggered by NW · Note: this may or may not have additional spec impact Option 2 (UE-assisted performance monitoring): · UE calculates performance metric(s) o FFS how to report and what to report

Regarding option 2, UE-assisted performance monitoring, it is necessary to define specific events that trigger reporting for model monitoring. Considering the scenario, in BM-Case 1, when predicted candidate beams are also measured in Set B, a direct comparison between the predicted and measured results can be performed to determine whether the inference is successful or not. For example, this involves checking if the predicted RSRP differs from the measured RSRP. When such a failure is detected, the UE can report it to the network to initiate further monitoring procedures. This process can be defined as an event.

For BM-Case 2, performing model monitoring needs to measure the full resource set configured by NW which results in significant measurement overhead and latency. Especially, to avoid the "ping-pong" effect, the performance metric must be calculated across multiple time instances rather than relying on a single measurement. Hence, to reduce this overhead, we propose reusing part of the input data from the previous inference instance for each new inference instance. This approach can be illustrated as following picture: for each inference instance, the model's input is based on measurements from five consecutive slots and predict for future four consecutive slots. In the first inference instance, the input data is derived from the measurements of slots one through five. In the second inference instance, the input data is collected from two parts: the new measurement from slot six and the reused measurement data from slots two through five, which were measured in the first inference instance.

This approach doesn’t need NW to configure the full set of resources according to the model's input requirements. Instead, only a portion of the resources needs to be configured, while the other part of the input data is derived from the measurements obtained in the previous instance. This approach can significantly reduce the measurement overhead.

Figure1. schematic diagram of multiple inferences instances with data reuse in BM-Cases 2

In this figure, the candidate beam in slot eight has been predicted in multiple inference instances. If the predicted results in each instance are the same or similar, this indicates that the model's performance is stable and suitable for the current scenario. Conversely, if the predicted results vary significantly, it may suggest that the model is failing to infer accurately.

Proposal 1: Support for defining event(s) to trigger reporting for monitoring,

  - when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1.

  - when the multiple prediction results for the same future time instance vary significantly in BM-Case2.

Proposal 2: For BM-Case2, reusing part of the input data from the previous inference instance for each new inference instance to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

## 2.2 UE side model update with NW assistance

For UE-side models, updates can be assisted by the NW. The need for NW assistance in updating UE-side models arises in the following scenarios:

- Model Initial Training on the NW Side: The initial training of the model is performed on the NW side and then distributed to the UE. Given that the NW has comprehensive knowledge of the model’s structure and parameters, it is better positioned to update the model based on measurement reports or prediction outcomes reported by the UE.

- Limited UE Capabilities: The UE may lack sufficient computational power, memory, or battery life to perform model updates. In such cases, the UE might rely on the NW to assist with model updates. However, this approach could potentially lead to some privacy-related parameters being disclosed to the NW, which is unavoidable in these situations.

NW assists model updating that ensures a comprehensive updating mechanism that leverages external resources for enhanced efficiency and accuracy. In the AI-based beam management case, this approach requires the UE to report  model difference.  The model difference can be one of the following:  difference between the required value and the monitored value of model performance metrics, difference between expected results and actual predicted results (model output) of the current model to the NW.  By observing the difference in model performance metrics, the NW can determine how well the model is performing against the optimal/required level and may initiate a model update before its performance degrades to an unacceptable level. The expected results represent actual optimal beam based on UE measurements, which can be obtained through traditional beam sweeping, as illustrated in Figure 1.

Figure 1. Traditional beam sweeping

Proposal 3: For the UE-side models, UE can report the following types of model difference to the NW when requesting assistance in model updates:  difference between the required value and the monitored value of model performance metrics, or difference between the predicted results and the traditional beam measurement-based results. .

# Summary

In this contribution, we have the following proposal for RAN2 to agree:

Proposal 1: Support for defining event(s) to trigger reporting for monitoring,

- when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1.

- when the multiple prediction results for the same future time instance vary significantly in BM-Case2.

Proposal 2: For BM-Case2, reusing part of the input data from the previous inference instance for each new inference instance to reduce measurement overhead based on the RS resource set configured by NW in model monitoring.

Proposal 3: For the UE-side models, UE can report the following types of model difference to the NW when requesting assistance in model updates:  difference between the required value and the monitored value of model performance metrics, or difference between the predicted results and the traditional beam measurement-based results.

# References

- Chair notes RAN1#117 eom