3GPP TSG RAN WG2 Meeting #130			R2-2504052

St. Julians, Malta, May 19th – 23rd 2025

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on paging for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new WID for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

At RAN2#129 [3], the following agreements were made related to Paging.

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

Agreements The A-IoT paging message can include a number of msg1 resources From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Also the following Post email discussion was agreed [4]

[POST129][035][AIoT] Paging (Qualcomm)

The summary from the email discussion included the following proposals.

Proposal 1:	Discuss and agree on the device behavior if it gets a new service request while one procedure is still ongoing (i.e. not completed or failed yet): (a) ignore all new requests, or (b) terminate the ongoing procedure and respond to the latest request. (a/b/ffs = 17/7/5)

Proposal 2:	Discuss and decide how a device determines the procedure is ‘ongoing’ or not.

Proposal 3:	Discuss and decide whether the device needs to distinguish and behave differently between when the same service request is received again from the same reader vs a different reader (yes/no/ffs = 14/12/2).

Proposal 4:	Discuss and decide whether RAN2 needs to consider the case where the device has already successfully responded to the same service request but now received the same request from a different reader e.g. for location use case (yes/no/unclear = 9/16/3).

Proposal 5:	Discuss and decide whether Reader ID information should be provided to the device (yes/no/FFS = 11/17/1).

Proposal 6:	Wait for SA2/RAN3 progress before further discussing on how the reader calculates transaction ID.

Proposal 7:	Discuss whether to send an LS: To SA2 asking whether correlation ID is expected to be same or different when the same service is requested from different readers, and to RAN3 asking whether coordination between readers is expected.

Proposal 8:	Postpone discussion on exact size of transaction ID until further progress on other issues captured in previous proposals.

Proposal 9:	Discuss whether and how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.)

At the RAN2#129bis meeting [4], the following agreements were made.

Behavior for Parallel service requests

Agreements

FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release.

Agreements on CFRA

Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message.

Agreements on paging ID length

- A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

- The number of bits required for paging ID length field should be as small as possible.  This would require the number of different Paging ID lengths to be small.

- Send an LS to SA2 to tak this into account for their design.

In this contribution, we discuss our view on some RAN2 related aspects related to subsequent paging, a new R2D trigger signal for D2R transmission and views on a number of the proposals from the email discussion.

# Discussions on paging for Ambient IoT

In legacy cellular systems, for the Network to reach the UE normally paging is used, for which a UE-ID is needed. The UE-ID is not only used for identification, but also to group the UEs in different wake-up groups for the system to page the UE without having to wake-up all UEs and for the UEs to save power, not having to listen for paging continuously.

In the case of Ambient IoT, the traffic assumptions have been defined as Device Terminated (DT), and Device Originated (DO). In this release, DO traffic only considers DO-DTT, Device Originated, Device Terminated Trigger, which means that the network wakes up the device but for expected or required uplink traffic. This could e.g., be sensor data that should be reported.

## Subsequent paging/parallel service requests

One issue that has been identified in the study item [3GPP TR 38.769 V2.0.0 (2024-12)] [2], is when the reader sends multiple subsequent paging messages associated with the same service request [ chapter 6.3.3]. (This is not about resending msg 2.

The duplicated response from a device for the same service request should be avoided, this to reduce the power consumption at both device and the reader as well as reduction of spectral usage. To address this, the A-IoT paging message can include information to avoid this duplicated response from the device to a reader. (Multiple reader, see separate paper).

The overall procedure between the A-IoT device and reader is according to Figure 1 where overall AS procedures between A-IoT device and reader, [2] is presented.

Figure 1, Overall AS procedures between A-IoT device and reader (Figure 6.3.1-1 in [2])

At the last meeting there is still an open FFS on the behavior in case of a new service request while one procedure is still ongoing. The discussions are amongst other ongoing in joint paper [6].

In this paper we discuss and list different alternatives on the type of information that can be included in the Paging response message to reduce and/or minimize the impact of duplicated responses from the AIoT device.

Downlink Ack:

As one option or alternative, the subsequent paging message includes either those devices that have responded, or alternatively those devices that have not responded.

This would not be as part of any RACH or STEP B procedure but included in the subsequent A-IoT paging message. For the case where the device -ID is included in the subsequent A-IoT paging message, the corresponding devices can simply ignore to respond again. The behavior can be indicated either that service, transaction or reader id is repeated, or a flag is set indicating that those A-IoT devices whether the device id is included can refrain from responding. (Whether the device id is and AS-Id or a NAS level Id can be discussed, and fi the reader may have access to a NAS level ID).

For the other case, the reader has prior information on what devices are to be paged and can then include the ID of those devices that have not responded to the initial A-IoT paging message. For the initial paging message, only group or type ID is included, to limit the message size.

Proposal 1: The subsequent paging message for the same service includes information, on which devices that has either responded or which have not responded.

## R2D trigger signal for D2R synchronization

Due to the simple design of the Ambient IoT devices, the quality and reliability of the clock accuracy may vary, and it may be difficult to maintain sync due to clock inaccuracy or clock drift.

During the study phase timing and clock information has been discussed and it has been captured in [TR 38.769], the usage of clock acquisition signals. An R2D timing acquisition signal (R-TAS), immediately preceding the transmission of PRDCH, is included at least for timing acquisition and indicating the start of R2D transmission, and a D2R timing acquisition signal (D-TAS), preceding each PDRCH, is included at least for timing acquisition, indicating the start of the D2R transmission.

The pilot signals R-TAS and D-TAS is used by the receiver in both the device and Reader. However, the timing of the device transmitter is based on an internal device clock which may use the R-TAS and in particular the CAP (Clock-acquisition part) of the R-TAS. For device type 1, the most constrained Ambient IoT device, it may not be able to keep a clock running with an accuracy assumed for good slot alignment. Furthermore, when the device is assigned a specific slot to transmit in, it needs to calculate when in time this will occur and set a timer accordingly.

At RAN2#129 it was discussed and agreed that resources for msg1 needs to be indicated by the reader, but still open on how this should be done. Two alternatives where presented.

From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later:

- Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

- Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

We think that an additional paging (Way-2) message gives limited benefit and may confuse the A-IoT device whether to act on the first paging message or wait for a subsequent message. Furthermore we don’t think we need a new message to include the msg 1 resources, but merely needed to include a timing indication for when the resources. The msg 1 resources should be conveyed in the first Paging message.

To handle this timing issue for the D2R transmission, the Reader could transmit a specific slot indication, to indicate for the A-IoT device, the start of the slot to be used for the D2R transmission.

This would in particular useful for transmission of message 1 for slotted Aloha, where the device needs to wait for a random slot to transmit in. see Figure 2.

Also for the case for triggering the transmission of message 3 it would be beneficial to introduce a slot or start of resource indication for that message 3. This would be even more useful if there are multiple devices included in the same RAR, message 2, and they have individual slots or resources assigned, then the message 2 would include a relation to what slot or resource number the device should use.

This number or id is included in the slot indication, in order for the device to know which slot or resource that is the intended slot or resource for the device to respond in.

Figure 2, Slot indication

Proposal 2: The msg 1 resources are provided in the initial paging message, and a new slot trigger is introduced only for timing purpose indicating the start of the uplink resources.

## Transaction ID

[6] has following proposals regarding the transaction ID:

Proposal 2: The R2D trigger message includes the transaction ID either explicitly as MAC payload or preferably, implicitly in physical layer signalling (e.g. mask the CRC with transaction ID or include it implicitly in the R2D preamble if feasible – details up to RAN1 – send an LS to RAN1 asking them to implement this) Proposal 3: In case of inventory procedure for CBRA: - The device assumes that the inventory procedure is ongoing once MSG1 is transmitted and the procedure is considered complete upon receiving a subsequent R2D trigger with the same transaction ID as the one included in the paging message that triggered CBRA - Upon receiving the subsequent R2D trigger message after transmitting MSG1: - the inventory procedure is considered to be completed successfully if msg3 was transmitted and no NACK was received. In this case the device will not respond to the same transaction ID (i.e. no re-access) - The inventory procedure is considered to be completed unsuccessfully for all other cases (e.g. MSG1 failure, MSG2 failure, MSG3 failure). In this case the device will respond to paging for any transaction ID (including the same transaction ID – i.e. re-access)

However, there could be a scenario where a device should be able to respond to any of the readers e.g. for the purpose of inventory. In other words, all readers are involved in inventory procedure and any of the readers is able to pick up the response from the device. In this case, presence of transaction ID will restrict the device to respond to a particular reader only. We think that depending on network configuration, the presence of transaction ID should be optional in different messages.

Proposal 3: Presence of transaction ID is optional.

# Summary

In this contribution, we have discussed our views on aspects related to subsequent paging, a new R2D trigger signal for D2R transmission and views on a number of the proposals from the email discussion

Proposal 1: The subsequent paging message for the same service includes information, on which devices that has either responded or which have not responded

Proposal 2: The msg 1 resources are provided in the initial paging message, and a new slot trigger is introduced only for timing purpose indicating the start of the uplink resources

Proposal 3: Presence of transaction ID is optional.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- 3GPP TR 38.769 V2.0.0 (2024-12)]

- Minutes RAN2#129 meeting, Athens

- [POST129][035][AIoT] Paging (Qualcomm)

- Minutes RAN2#129bis, Wuhan.

- R2-250xxxx: Details of option A for overall A-IoT MAC procedure, ZTE et al