3GPP TSG-RAN WG2 Meeting #130  	                            R2-2504057

St.Julians, Malta, May 19th – 23rd, 2025							                (Revision of R2-2502493)

Agenda Item:	8.8.2

Source: 	Sony

Title:	SMTC impacts due to NTN downlink coverage enhancements

Document for:	Discussion

# Introduction

In RAN2#129bis meeting, RAN2 discussed SMTC impacts in NTN downlink coverage enhancement and the corresponding agreements are as follows.

Agreements;

1.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

2.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

3.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this contribution, we mainly discuss the SMTC impacts due to beam hopping/extended SSB periodicity and the way forward.

# Discussion

With only the serving cell’s reference location and a threshold to that reference location, is not enough for UE to perform the proper the SMTC measurement selection. Furthermore, for the RRC_CONNECTED UEs, the propagation delay difference is not accurate enough either for network to configure the proper SMTCs for the UEs. Therefore, we propose to enhance the SMTC measurement adjustment, for both UEs in RRC_CONNECTED and RRC_IDLE/INACTIVE.

The signalling details for additional assistance information e.g. additional reference location information from network, finer UE location information from UE if available and requested should be further discussed. The additional reference location information can be for example the neighbour cells’ location information, and together with the serving cell’s location information, UE can get a rough estimation of the distance to the neighbour cell and then can decide whether it needs to monitor the SMTC corresponding to that particular neighbour cell.

On the other hand, if the UE would like to share its finer location information to the network, network can dynamically instruct the UEs the SMTCs to be monitored.

Proposal 1: Neighbour cell’s location information is one of the assistance information for UE to perform SMTC configuration selection.

Proposal 2: If UE agrees to share its finer location information, the UE’s location information can be shared with network and network can configure the appropriate SMTC configuration for the UE in RRC_CONNECTED.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Neighbour cell’s location information is one of the assistance information for UE to perform SMTC configuration selection.

Proposal 2: If UE agrees to share its finer location information, the UE’s location information can be shared with network and network can configure the appropriate SMTC configuration for the UE in RRC_CONNECTED.