3GPP TSG RAN WG2 Meeting #130			R2-2504062

St. Julians, Malta, May 19th – 23rd    2025

Agenda Item: 	8.2.3

Source:		Sony

Title:			Considerations on RACH msg 2 configuration

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new WID for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

At RAN2#129 [3], the following agreements were made related to.

Agreements

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

Agreements

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

At RAN2#129bis meeting [4], the following agreements were made related to Random Access procedure,

Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

In this contribution, we discuss our view on msg 2 monitoring configuration.

# Discussions on message 2 monitoring configuration

In legacy solutions for Random access procedure, when the UE has transmitted its initial random access message (msg1), the UE waits for the Random Access response, RAR (msg2). This waiting period is defined by the use of ra-Response window [38.321].

For Ambient IoT general definition of paging and random access procedure has been described in [2].

The concluding results are collected in TR 38.769 V2.0.0. The overall AS procedures for AIoT is formulated as following, see also Figure 1:

- Step A: A-IoT paging. Based on the service request, including “inventory-only”, “inventory and command”, and “command-only” the reader sends the A-IoT paging message indicating device(s) that need to respond.

- Step B: D2R data (device ID) transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure.

- Step C1: Possible R2D data transmission (e.g., for sending the command).

- Step C2: Possible D2R data transmission (e.g., the corresponding response to command).

Figure 1 – Illustration of procedures for an Ambient IoT compact protocol stack

A-IoT random access procedure in step B further includes the following steps:

- A-IoT Msg1: It is a PDRCH signal from the AIoT Device to the Reader). After the AIoT device identifies its access occasion, it transmits the AIoT Msg1 which includes 16-bit random ID generated by the A-IoT device to the reader.

- A-IoT Msg2: It is a response in a PRDCH from the Reader to the AIoT Device, indicating it has successfully received Msg1, i.e., responding with the Msg1 received device random ID.

- AIoT Msg3: It is a PDRCH signal from AIoT Device to Reader) in response to Msg2 and includes e.g. Device ID and other data.

A signalling diagram including Msg1 to Msg3 is illustrated in Figure 2.

Figure 2 - Signaling diagram of basic random access procedure, including Msg 1 to Msg 3 related to step B

In the current normative work of AIoT, details of AIoT paging (step A) and random-access procedure (step B) are under discussion but not concluded. In this document, we focus on aspect related to Msg2 and propose a scheme to enable configuration of Msg2 transmission and associated Msg2 monitoring occasion from AIoT device perspective. During the previous meeting, the following agreement was made on Msg2.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

- Depending on whether Msg2 is transmitted in response to Msg 1 from one or multiple devices, the total length of Msg2 as well as the duration for monitoring Msg2 may vary.

For a device to be able to monitor for Msg2 in a sufficient manner, i.e., with low monitoring cost in term of power consumption and delay, the device needs to have information related to a starting time reference and a monitoring time duration to be able to monitor the channel for potential Msg2 reception. The Msg2 monitoring window includes both the starting time e.g., TMsg2_start and TMsg2_end/time duration for Msg2 monitoring from device perspective [2].

Additionally, the device also needs to know whether Msg2 are transmitted in response to Msg 1 of one device or multiple devices as it may influence the monitoring time duration and its start time. Selecting the same msg2 monitoring window for both options may lead to extra delay for msg2 transmission corresponds to an A-IoT msg1 received from one device. It also may result in extra power consumption for these AIoT devices as msg2 may become longer if msg2 transmission corresponds to msg1 from multiple devices, see Figure 3.

In the previous RAN1 meeting aspects related to monitoring starting time and monitoring window were discussed and the following alternatives were listed. For a device to determine the reference time for the starting time for monitoring Msg2 in response to multiple Msg1 transmissions the following options have been identified in 3GPP [2]:

•	Option 1: End of time domain resource where the device transmitted the Msg1

•	Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s)

•	Option 3: End of the R2D transmission triggering random access

•	Option 4: End of a R2D transmission carrying Msg2

Note:	The reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

To determine the time duration for monitoring Msg2 the following options have been identified:

•	Option 1: Predefined,

•	Option 2: Indicated in the R2D transmission triggering random access.

Note: Option 1 and Option 2 may not be mutually exclusive.

There is, however, no association of the Msg2 start and monitoring time to the content it carries, i.e., whether it is associated to one device or multiple devices. As discussed earlier, in order to have a cost-efficient monitoring, it is necessary for the devices to have early knowledge about the upcoming Msg2. We therefore propose to include early information about Msg2 content already in either the paging message or in the newly introduced R2D message following the paging message. We also propose to select the reference time and monitoring time in accordance with the content of Msg2. Through these, we can enable a more cost-efficient monitoring window in terms of power consumption and delay.

Proposal 1 – RAN2 to include early information about Msg2 content, i.e. its transmission to Msg1 is associated to one or multiple devices, in either the paging message or in the newly introduced R2D message following the paging message.

Proposal 2 – To support selecting of Msg2 monitoring window in association to Msg2 content to allow for a more cost-efficient monitoring window in terms of power consumption and delay.

Figure 3, Timing diagram illustrating options A1 and A2, selecting the same Msg2 monitoring window for both options A1 and A2 may lead to extra delay or power consumption. A1 and A2 represent Msg2 in response to one device or multiple devices, respectively

# Summary

In this contribution, we have discussed our views on aspects related to msg 2 monitoring configuration.

Proposal 1 – RAN2 to include early information about Msg2 content, i.e. its transmission to Msg1 is associated to one or multiple devices, in either the paging message or in the newly introduced R2D message following the paging message.

Proposal 2 – To support selecting of Msg2 monitoring window in association to Msg2 content to allow for a more cost-efficient monitoring window in terms of power consumption and delay.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR.38.769 V 2.0.0

- Minutes RAN2#129, Athens

- Minutes RAN2#129bis, Wuhan