3GPP TSG-RAN WG2 Meeting #130	R2-2504093

Malta, Malta, 19th – 23rd May, 2025

Agenda item:	8.17

Source:	Samsung

Title:	On RAN2 aspects of IoT NTN TDD

WID/SID:	IoT_NTN_TDD

Document for:	Discussion and Decision

# Introduction

In RAN#106 the WID for IoT NTN TDD was agreed [1]:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In this contribution, we provide some initial discussion on IoT NTN TDD.

# Discussion

- The key aspect of the work item is to adapt NB-IoT NTN to a special TDD band, which can be seen in Figure 1. The UL and DL frames are 8.28 ms, the simplex frame is 20.32 ms and with guard bands the total comes in to 90 ms.

Figure 1.

- The challenging aspects of this specific frame is that the periodicity of the TDD bands has a periodicity that is not aligned with 3GPP periodicities and frame structures, and that each UL or DL frame being 8.28 ms makes allocating the subframes difficult. Another challenging aspect has to do with the fact that legacy services prevent all of the UL and DL frames to be available for NB-IoT. This means that very few subframes are available, potentially interrupting or delaying procedures. Due to the existing services, it was agreed in the WID that only a single UL and DL frame with 8 subframes each, are available every 90 ms.

## GNSS measurement gaps

- In the last meeting, RAN1 had the following agreements:

Agreement

For GNSS measurement gap, RAN1 to further discuss at least the following options:

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap

- Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe

- Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD

- GNSS measurement gaps were partly specified by RAN1, but mostly implemented in RAN2. This feature which on the surface seems simple has a lot of conditions and special cases and it took a considerable effort from RAN2 to specify it. NB-IoT NTN TDD use case, where at most 8 DL and UL subframes are available for data transmissions every 90 ms, will not allow for higher data rates and it is thus unlikely that a UE in such a use case will be in connected mode for long enough where the GNSS is no longer valid. With this in mind, we propose that RAN2 can make a decision to down-prioritize any enhancements on GNSS gap. If needed, RAN2 can indicate this to RAN1.

Proposal 1: RAN2 agrees to down-prioritize enhancements on GNSS measurement gap for IoT NTN TDD.

## Timers

- In RAN2#129, the following agreement was taken:

5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system.

- And in RAN2#129bis, the following agreement:

3.	Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes

- The contribution for which the proposal originated from discussed whether the current timer ranges are sufficient as IoT NTN TDD pattern only maximum allows for 8 ms DL/UL every 80 ms. However, while in theory the transmission durations may be increased due to the low amount of resources, we do not think that it would be a reasonable system design to revisit the timer values because it may require a lot of work in RAN2 and CT1, with likely next no to reasonable practical reasons for it. If the repetitions are only transmitted during the 8 ms out of 90 ms duty cycle, it may in theory be possible to assume that transmission times are increased by a factor 90/8=11.25 on top of the maximum 2048 repetitions, but there are no reasonable deployments where such long transmission times would be used. In other words, the deployment would have to ensure that the number of repetitions would be dimensioned to take into account the reduced resources in an IoT NTN TDD network. Therefore we propose:

Proposal 2: RAN2 agrees current value ranges of NB-IoT timers (in PDCCH periods and in absolute time) are sufficient for practical deployments for IoT NTN TDD.

## On extended k-Mac

In the last meeting the following was agreed:

6.	For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

It was essentially agreed that the extended k-Mac is introduced for IoT NTN TDD. The main reason for this is that if the extension would be introduced for all features, then there would be issues with backwards compatibility since this would impact the random access procedures. This would also imply that a Release 19 IoT NTN TDD UE would need to mandatorily support extended k-Mac, else the issue is the same as for legacy devices. Therefore we propose:

Proposal 3: Extended k-Mac is mandatory for UEs supporting IoT NTN TDD band(s).

This we believe that be specified by having a conditionally mandatory capability without signalling, where the condition is support of the IoT NTN TDD band.

Proposal 4: Extended k-Mac is indicated to be supported as a mandatory feature of IoT NTN TDD in 36.306. FFS how the IoT NTN TDD feature is captured in 36.306.

The extended k-Mac can be introduced by introducing a new value and not configuring the legacy value k-MAC. A text proposal can be seen in Appendix 5.2.

Proposal 5: The extended k-Mac is introduced by introducing a new value and not configuring the legacy k-Mac.

Proposal 6: Take the RRC implementation example in Appendix 5.1 as a baseline.

# Conclusion

In this contribution, we provided some proposals on IoT NTN TDD:

Proposal 1: RAN2 agrees to down-prioritize enhancements on GNSS measurement gap for IoT NTN TDD.

Proposal 2: RAN2 agrees current value ranges of NB-IoT timers (in PDCCH periods and in absolute time) are sufficient for practical deployments for IoT NTN TDD.

Proposal 3: Extended k-Mac is mandatory for UEs supporting IoT NTN TDD band(s).

Proposal 4: Extended k-Mac is indicated to be supported as a mandatory feature of IoT NTN TDD in 36.306. FFS how the IoT NTN TDD feature is captured in 36.306.

Proposal 5: The extended k-Mac is introduced by introducing a new value and not configuring the legacy k-Mac.

Proposal 6: Take the RRC implementation example in Appendix 5.1 as a baseline.

# Reference

RP-243293, Revised WID for introduction of IoT NTN TDD mode, Iridium Satellite LLC, RAN#106, Madrid, Spain, December 2024.

# Appendix

## 36.331 TP

------------------------ Example based on 36.331 V18.5.0 ------------------------

–			SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell. SystemInformationBlockType31 is only signalled for an NTN cell.

SystemInformationBlockType31 information element

-- ASN1START

SystemInformationBlockType31-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-r17 ::=	SEQUENCE {

ephemerisInfo-r17			CHOICE {

stateVectors				EphemerisStateVectors-r17,

orbitalParameters			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r17			SEQUENCE {

nta-Common-r17					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r17				INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r17	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

ul-SyncValidityDuration-r17		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900},

epochTime-r17					SEQUENCE {

startSFN-r17					INTEGER (0..1023),

startSubFrame-r17				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Offset-r17					INTEGER (0..1023),

k-Mac-r17						INTEGER (1..512)				OPTIONAL,	-- Need OP

...

}

ServingSatelliteInfo-v1820 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18					OPTIONAL,	-- Need OR

referenceLocation-r18			CHOICE {

fixedReferenceLocation-r18			ReferenceLocation-r18,

movingReferenceLocation-r18			ReferenceLocation-r18

}															OPTIONAL,	-- Need OR

distanceThresh-r18				INTEGER(0..65535)			OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType31 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED. Each step represents 50m.

epochTime Epoch time of the satellite ephemeris data and common TA parameters, see TS 36.213 [23]. This field also indicates the epoch time for the reference location of earth moving cells if present. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. For serving cell, the startSFN indicates the current SFN or the next upcoming SFN after the frame where the message indicating the epochTime is received. If the field is absent, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB31(-NB) is transmitted. E-UTRAN always includes epochTime when SIB31(-NB) is provided through dedicated signalling. In case of handover or conditional handover, this field is based on the timing of the target cell, i.e. the startSFN and startSubFrame number indicated in this field refers to the SFN and sub-frame of the target cell, and UE considers the target cell epoch time (indicated by the startSFN and startSubFrame in this field) to be the frame nearest to the frame where RRCConnectionReconfiguration message is received. In case of handover or conditional handover, the reference point for epoch time of the target NTN payload ephemeris and Common TA parameters is the uplink time synchronization reference point of the target cell.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the value in extK-Mac in SIB31-NB if configured, else the UE uses the (default) value of 0.

k-Offset Scheduling offset used in the timing relationships in NTN, see TS 36.213 [23]. Unit in ms.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

orbitalParameters Instantaneous values of the satellite orbital parameters. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

referenceLocation Reference location of the NTN (quasi-)earth fixed cell or earth moving cell, used in location-based measurement initiation in RRC_IDLE (as specified in TS 36.304 [4]) and RRC_CONNECTED if distanceThresh is also configured. If configured by an earth moving cell, the broadcast reference location corresponds to the epoch time and is also used in the evaluation of Event D2 and CondEvent D2, and the UE derives the real-time reference location based on the serving satellite ephemeris, see TS 36.304 [4].

stateVectors Instantaneous values of the satellite state vectors. The signalled values are valid at least for the duration as defined by ul-SyncValidityDuration and epochTime.

ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated.

<OMITTED>

–			SystemInformationBlockType33

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

SystemInformationBlockType33 field descriptions

epochTime Epoch time of the neighbour satellite ephemeris data and common TA parameters, see TS 36.213 [23]. The reference point for epoch time of the neighbour satellite ephemeris and Common TA parameters is the uplink time synchronization reference point of the serving cell when this field is provided in an NTN cell and the eNB when this field is provided in a TN cell. epochTime is the starting time of a DL subframe indicated by startSFN and startSubframe. If this field is absent in an NTN cell, the UE uses epoch time of the serving cell, otherwise the field is based on the timing of the serving cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the serving cell. The startSFN indicates the SFN nearest to the frame where the message indicating the epochTime is received. If this field is absent in a TN cell, the epoch time is the starting time of the DL subframe corresponding to the end of the SI window during which the SI message carrying SIB33(-NB) is transmitted.

k-Mac Scheduling offset used when downlink and uplink frame timing are not aligned at the eNB, see TS 36.213 [23]. Unit in ms. If the field if absent, the UE uses the value in extK-Mac for the associated satellite info element if configured, else the UE uses (default) value of 0.

neighValidityDuration Validity duration of the neighbour satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. If this field is absent in an NTN cell, the UE uses validity duration from the serving cell assistance information. If this field is absent in a TN cell, how the UE sets validity duration is left to UE implementation.

nta-Common Network-controlled common TA, see TS 36.213 [23]. Unit of μs. Step of 32.55208 ×10-3 μs. Actual value = field value * 32.55208 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDrift Drift rate of the common TA, see TS 36.213 [23]. Unit of μs/s. Step of 0.2 ×10-3 μs/s. Actual value = field value * 0.2 ×10-3. If the field is absent, the UE uses the (default) value of 0.

nta-CommonDriftVariation Drift rate variation of the common TA, see TS 36.213 [23]. Unit of μs/s2. Step of 0.2 ×10-4 μs/s2. Actual value = field value * 0.2 ×10-4. If the field is absent, the UE uses the (default) value of 0.

t-ServiceStartNeigh Indicates the earliest time when the area covered by the current serving cell is going to be covered by the neighbour cell(s) served by the satellite indicated by satelliteId, see 5.5.3.1, 5.5.8 and 36.304 [4]. This field is only present for the NTN quasi-Earth fixed neighbour cell(s).

<OMITTED>

–				SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information. SystemInformationBlockType31-NB is only signalled in a NTN cell.

SystemInformationBlockType31-NB information element

-- ASN1START

SystemInformationBlockType31-NB-r17 ::= SEQUENCE {

servingSatelliteInfo-r17		ServingSatelliteInfo-r17,

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[	servingSatelliteInfo-v1820	ServingSatelliteInfo-v1820		OPTIONAL	-- Need OR

]],

[[	servingSatelliteInfo-v19xy	ServingSatelliteInfo-NB-v19xy	OPTIONAL	-- Need OR

]]

}

ServingSatelliteInfo-NB-v19xy ::=	SEQUENCE {

extK-Mac-r19					INTEGER (513..FFS)				OPTIONAL	-- Need OP

}

-- ASN1STOP

<OMITTED>

–	SystemInformationBlockType33-NB

The IE SystemInformationBlockType33-NB contains satellite assistance information for neighbour cells.

SystemInformationBlockType33-NB information element

-- ASN1START

SystemInformationBlockType33-NB-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18		NeighSatelliteInfoList-r18		OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING					OPTIONAL,

...,

[[  neighSatelliteInfoList-v19xy	NeighSatelliteInfoList-NB-v19xy OPTIONAL    -- Need OP

]]

}

NeighSatelliteInfoList-NB-v19xy ::=		SEQUENCE {

extK-Mac-r19							INTEGER {513..FFS}		OPTIONAL	-- Need OP

}

-- ASN1STOP

------------------------ Example based on 36.331 V18.5.0 ------------------------