3GPP TSG RAN WG2 #130	R2-2504098

St Julian’s, Malta, May 19 - 23, 2025

Agenda Item:	8.9.3 
Source:	Toyota ITC
Title:	Discussion on Diversity Slotted ALOHA Randomization

Document for:	Discussion and Decision

# Introduction

In the RAN #103 meeting, the work item for Rel-19 Non-Terrestrial Networks (NTN) targeting Internet of Things (IoT) Phase 3 was approved [1], with one of key objectives being the enhancement of uplink. The purpose of this contribution is to motivate a standardized randomization algorithm for Diversity Slotted SLOHA.

# Discussion

The main principle of Diversity Slotted ALOHA (DSA) is to transmit multiple copies of the same message across various randomly selected sub-frames within a transmission window. This approach increases the likelihood that at least one copy will be received without interference, thereby enhancing the chances of successful message delivery [2].

Currently, RAN2’s proposals have primarily emphasized the advantages of specifying pointers to assist receiver design. However, the benefits of employing a standardized randomization algorithm from the User Equipment (UE) perspective have not been adequately addressed. The term “random and uniform” is frequently used in 3GPP protocol specifications, yet it often lacks concrete requirements or defined test cases, as randomness is inherently challenging to test. Consequently, the performance of some radio protocols is largely dependent on the UE implementation.

A significant issue arises when different chipsets are utilized for the same application. For instance, in vehicles where connectivity is critical for road safety—such as during warnings and alarms—using different chipsets in the same vehicle model can lead to varying behaviours and performance levels. This inconsistency can result in disparities in the performance of the vehicle's security features, which is highly undesirable.

To address this, it is proposed to standardize a randomization algorithm to ensure predictable, consistent, and testable UE behaviour based on the algorithm proposals presented thus far. Notably, Msg3 contains pseudo-random content, and the message content of the NAS container varies, as there is no justification for repeatedly sending identical message content.

Observation 1: A standardized randomization algorithm ensures predictable, consistent, and testable UE behavior.

An example of pseudo-random content is the S-Temporary Mobile Subscriber Identity (S-TMSI) [3] which uniquely defines the subscriber unit in a tracking area and is included, e.g., in early data transmission message [4] as shown below. It should be noted that S-TMSI is a temporary identity, and it is periodically reallocated, e.g. during registration updates [5].

RRCEarlyDataRequest message

-- ASN1START

RRCEarlyDataRequest-r15 ::=		SEQUENCE {

criticalExtensions				CHOICE {

rrcEarlyDataRequest-r15			RRCEarlyDataRequest-r15-IEs,

criticalExtensionsFuture		CHOICE {

rrcEarlyDataRequest-5GC-r16		RRCEarlyDataRequest-5GC-r16-IEs,

criticalExtensionsFuture-r16	SEQUENCE {}

}

}

}

RRCEarlyDataRequest-r15-IEs ::=	SEQUENCE {

s-TMSI-r15						S-TMSI,

establishmentCause-r15			ENUMERATED {mo-Data, delayTolerantAccess},

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				RRCEarlyDataRequest-v1590-IEs			OPTIONAL

}

In reference [6], it is demonstrated how the S-TMSI and NAS container content can be extracted from the encoded octets in the transport block and utilized as a seed for a randomization algorithm. The details of this randomization algorithm are provided in the Annex as an excerpt from reference [6].

While it can be argued that the proposed randomization method may not fully match the robustness of a local random number generator within UE, the lack of any disclosed randomization algorithm from UE implementations leaves this argument unsubstantiated. Consequently, this contribution concludes with the following proposal and encourages a decision regarding the proposed approach.

Proposal 1: Standardize a randomization algorithm for Diversity Slotted ALOHA.

# Conclusion

The observations and proposals of this contribution are summarized as follows:

Observation 1: A standardized randomization algorithm ensures predictable, consistent and testable UE behavior.

Proposal 1: Standardize a randomization algorithm for Diversity Slotted ALOHA

# Reference

- RP-240776, "Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3", March 18-21, 2024.

- R2-2405202, "Discussion on EDT enhancements", May 20th - 24th, 2024.

- 3GPP TS 23.003 V18.5.0, "Numbering, addressing and identification".

- 3GPP TS 36.331 V18.1.0, "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

- 3GPP TS 24.501 V18.7.0, “Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3”

- R2- 2501265, “Implicit pointer for locating CB-Msg3 DSA replicas”, Feb 17 – 21, 2025.

# Annex:

Example of algorithm for generating replica positions

An example of an algorithm that could be used for generating the replica positions is provided in the following. It is assumed for simplicity of discussion that the set of time-frequency resources that can be used by the UE to place its replicas are described as an ordered list of num_resources elements. The algorithm implements a linear congruential generator (LCG) with modulus chosen as a Mersenne prime and increment set to 0.

INPUTS:

num_replicas: 		integer, number of DSA Msg-3 replicas that the UE shall send

num_resources: 		integer, number of available time-frequency resources

seed: 			24-bit unsigned integer. this can be obtained as a 24-bit linear hash function

taking as input the content of the CB-Msg3.

OUTPUT:

positions: 		array of num_replicas integers, each between 1 and num_resources,				corresponding to a time-frequency occasion in the ordered list of EDT available

resource elements

ALGORITHM:

const m = 2^31 - 1           // modulus of the LCG
const A = 7^5                  // multiplier of the LCG
int x = seed                     	// initialize the LCG with the seed
positions = []             	// initialize an empty int array for positions

FOR k = 1 TO num_replicas DO

REPEAT
          x = (A * x) MOD m                     		// generate the next pseudo-random number
          position = (x MOD num_resources) + 1  	// map to [1, num_resources]
      UNTIL position NOT IN positions
   positions.APPEND(position)                		// add the valid position to the array

END FOR

RETURN sorted positions