3GPP TSG-RAN WG2 Meeting #130	R2-2504100

Malta, 19 – 23 May 2025

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Discussion on remaining issues on Asymmetric DL sTRP/UL mTRP

Document for:  Discussion and Decision

1	Introduction

For Rel-19 MIMO, objectives relevant to asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts. Progress on the MAC CE definition approach for this topic has been made in RAN1 and RAN2 meetings. In the last meeting, RAN2 agreements were as follows:

Agreements

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE. RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA. We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording. From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE. For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: - • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; - • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario. For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

In this contribution, we present detailed analysis on RAN2 work on the remaining issues on asymmetric DL sTRP/UL mTRP based on RAN1 and RAN2 agreements so far, and provide our considerations.

2	Discussion

2.1	MAC CE for TCI State activation/deactivation

For the asymmetric DL sTRP/UL mTRP deployment scenario, RAN1 agreed to reuse the Rel-18 unified TCI framework:

Agreement - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The R18 legacy Enhanced TCI state activation/deactivation MAC CE format is shown in Figure 1.

According to TS 38321[1], the Rel-18 activation/deactivation MAC CE can support the {two DL TCI states + two UL TCI states} configuration, which covers the combination of {one DL TCI state + up to two UL TCI states} in RAN1 agreement. Therefore, the legacy MAC CE format can be reused in the asymmetric DL sTRP/UL mTRP scenario. Obviously, there will be no impact on the joint TCI mode. For the separate TCI mode, According to TS 38321[1], if Fi,j field is set to 0, it indicates the j-th DL TCI state for codepoint i is absent, since there is no DL TCI State for the UL-only TRP, the corresponding Fi,j field should set to be 0, and the UE should ignore this Fi,j field.

Proposal 1: Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

Proposal 2: Add a note in TS 38.321 “When the TCI activation/deactivation MAC CE for separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j field for the TCI state of the UL-only TRP is set to 0 and the UE ignores this Fi,j field.”

Figure 1. Rel-18 Enhanced TCI state activation/deactivation MAC CE format

2.2	PL offset Update

Based on the agreement, RAN2 should introduce in RRC a PL offset for each joint and UL TCI state and a new MAC CE with eLCID for updating the PL offset associated with TCI states after RRC configuration.

On the co-existence of RRC and MAC CE approaches, in the last RAN2 meeting, the following agreement is achieved

From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE.

RRC supports delta signalling, in which the UE is required to keep fields that are absent. For example, if a number of TCI-UL-State were previously configured in BWP-UplinkDedicated for some UL BWP, and the network performs a reconfiguration of some DL parameters, or of some other UL BWP, the UE keeps the existing TCI-UL-State configured in this UL BWP.

From our point of view, if delta signalling is supported for PL offset update, the following problems will occur due to the co-existence of RRC and MAC CE approaches.

As shown in Figure 2, when the PL offset is included in TCI-UL-State, after the reconfiguration, it would be problematic if UE uses the path loss offset modified by MAC CE because the target gNB may keep the original RRC configured PL offset which is indicated from the source gNB via UE context, and this will lead to different PL offsets kept by UE and target gNB, which will further impact the PRACH power control and power headroom report.

Observation1: During handover procedure, in case delta signalling is used, the UE may store the latest PL offset value received in MAC CE, while the target gNB is only aware of RRC configured value by the source gNB.

To solve the problem, one solution could be that, the network is mandatory to avoid delta signalling for PL offset so that the UE shall always use the value configured by RRC from target gNB. However, this solution would restrict NW implementation and cause additional RRC overhead. Another solution which is more aligned with RAN2 previous agreement is that the target gNB is aware of the latest value. More specifically, the MAC entity should indicate the PL offset to the RRC layers so that the target base station and UE can store the same PL offset. However, this solution may require additional signalling between DU and CU to align the latest value between MAC and RRC. Additionally, the existing description of the meaning of “need code” in TS 38.331 only refers to the configuration provided by the RRC message, not the MAC CE.

- TS 38331[2] - "Otherwise, the field is absent, Need R": The field is released if absent when this part of the condition applies. This handles UE behaviour in case the field is configured via another part of the condition and this part of the condition applies (which means that network when transmitting downlink RRC message or peer UE transmitting PC5 RRC message can assume UE releases the field if this part of the condition is valid). - "Otherwise, the field is absent, Need M": The UE retains the field if it was already configured when this part of the condition applies. This means the network when transmitting downlink RRC message or the peer UE when transmitting PC5 RRC message cannot release the field, but UE retains the previously configured value.

If the delta signalling is supported for PL offset update, the description of the “current” pathlossOffset configuration stored at the UE should refer to the latest configuration provided by RRC message or MAC CE.

Observation2: When the pathlossOffset is absent in RRC reconfiguration, the “current” PL offset configuration stored at the UE only refers to the last pathlossOffset configured by RRC, not MAC CE.

Proposal 3a: Select one from the following two options for PL offset update for handover:

- Option1: Delta signalling is not supported for PL offset.

- Option2:

Delta signalling is supported for PL offset.

Before RRC reconfiguration, the latest PL offset should be indicated from DU to CU for handover.

To clarify in TS 38.331 that when the pathlossOffset is absent in RRC configuration, the “current” pathlossOffset configuration stored at the UE refers to the latest configuration provided by RRC message or MAC CE.

Proposal 3b: If Option 2 is adopted, a LS to RAN3 is needed to ask for if any potential impacts.

Figure 2. Different PL offsets between UE and Target gNB under handover scenario

# 3	Conclusion

From the analysis on RAN2 impacts for MIMO, we made the following proposals:

Proposal 1: Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

Proposal 2: Add a note in TS 38.321 “When the TCI activation/deactivation MAC CE for separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j field for the TCI state of the UL-only TRP is set to 0 and the UE ignores this Fi,j field.”

Observation1: During handover procedure, in case delta signalling is used, the UE may store the latest PL offset value received in MAC CE, while the target gNB is only aware of RRC configured value by the source gNB.

Observation2: When the pathlossOffset is absent in RRC reconfiguration, the “current” PL offset configuration stored at the UE only refers to the last pathlossOffset configured by RRC, not MAC CE.

Proposal 3a: Select one from the following two options for PL offset update for handover:

- Option1: Delta signalling is not supported for PL offset.

- Option2:

Delta signalling is supported for PL offset.

Before RRC reconfiguration, the latest PL offset should be indicated from DU to CU for handover.

To clarify in TS 38.331 that when the pathlossOffset is absent in RRC configuration, the “current” pathlossOffset configuration stored at the UE refers to the latest configuration provided by RRC message or MAC CE.

Proposal 3b: If Option 2 is adopted, a LS to RAN3 is needed to ask for if any potential impacts.

# 4	Reference

- TS 3GPP 38.321 V18.5.0

- TS 3GPP 38.331 V18.5.1