3GPP TSG-RAN WG2 Meeting #130	R2-2504104

St Julian's, Malta, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0554 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the periodic AD of NR integrity service alert |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During R18, the feature of RAT-dependent integrity has been introduced. In this feature, the network could send assistance data to the UE for two RAT-dependent positioning methods, namely DL-TDOA and DL-AoD, either by LPP or by PosSIB, as follows: For another feature of PRU in R18, we have also supported periodic measurement result delivery. So, the mechanisms of periodic AD for RAT-dependent positioning methods are already in place in the spec today. Since NR-IntegrityServiceAlert is very similar to its GNSS counterpart GNSS-Integrity-ServiceAlert and GNSS-integrity-ServiceAlert supports periodic assistance data delivery, NR-IntegrityServiceAlert should also support periodic AD delivery |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add in the periodic assistance request the periodic delivery of NR-IntegrityServiceAlert Inter-Operability analysis: Impacted functionality: RAT-dependent integrity Impacted 5G architecture options: NR SA, NR-DC, NG-EN-DC Inter-operability: If the UE is implemented according to this CR while the NW is not, or If the NW is implemented according to this CR while the UE is not, the UE cannot support periodic assistance data delivery of RAT-dependent integrity service alert, which is essential for integrity without which the UE may not be able to well perform RAT-dependent integrity and results in service failure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may not be able to timely receive assistance for integrity support of service alert for RAT-dependent positioning methods, which may result in service failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2a, 6.4.3, 6.5.10.1, 6.5.10.2, 6.5.11.1, 6.5.11.2, 6.5.11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================================CHANGE BEGINS==================================

5.2.2a	Periodic Assistance Data Delivery procedure

The Periodic Assistance Data Delivery procedure allows the server to provide unsolicited periodic assistance data to the target and is shown in Figure 5.2.2a-1.

NOTE 1:	In this version of the specification, periodic assistance data delivery is supported for HA GNSS (e.g., RTK), NR DL-AoD and NR DL-TDOA positioning only.

Figure 5.2.2a-1: LPP Periodic Assistance data delivery procedure

1.	The server sends a ProvideAssistanceData message to the target using some available transactionID T1 and indicates the end of this transaction. The message contains a periodicSessionID S (different to any other periodicSessionID currently in use between the server and target) in the IE CommonIEsProvideAssistanceData. The message includes positioning method specific assistance data control parameters (e.g., in IE A-GNSS-ProvideAssistanceData) identifying the type of periodic assistance data being delivered together with periodicity conditions for sending it and a duration for ending the assistance data delivery (e.g., in IE GNSS-PeriodicAssistData). The ProvideAssistanceData message may also include non-periodic assistance data (but not any periodic assistance data).

2.	When the first periodic message is available, the server sends an unsolicited ProvideAssistanceData message to the target containing the periodicSessionID S and the periodic assistance data announced in step 1. The message uses some available transactionID T2 that may be different to T1.

NOTE 2:	The positioning method specific control parameters (e.g., IE GNSS-PeriodicAssistData) are not included in the data transaction.

3.	The server may continue to send further ProvideAssistanceData messages to the target containing the periodic assistance data announced in step 2 when each additional periodicity condition occurs.

NOTE3:	The target device expects a ProvideAssistanceData messages at the in Step 2 announced interval(s). If some or all of the assistance data is not available at each periodic interval, an error indication is provided in the positioning method specific IE (e.g., IE A-GNSS-Error).

4.	If the target requires the session to end, the target sends an Abort message to the server for transaction T2 that may optionally include an abortCause. Remaining steps are then omitted.

5.	If the server requires the session to end, the server sends an Abort message to the target for transaction T2 that may optionally include an abortCause. Remaining steps are then omitted.

6.	When the duration or other conditions for ending the periodic assistance data transfer occur, the last ProvideAssistanceData message transferred indicates the end of transaction T2.

======================================NEXT CHANGE==================================

6.4.3	Common NR Positioning Information Elements

–	NR-PeriodicAssistData

The IE NR-PeriodicAssistData is used by the location server to provide control parameters for a periodic assistance data delivery session (e.g., interval and duration) to the target device for RAT-dependent positioning methods-.

NOTE:	Omission of a particular assistance data type field in IE NR-PeriodicAssistData means that the location server does not provide this assistance data type in a data transaction of a periodic assistance data delivery session, as described in clauses 5.2.1a and 5.2.2a. Inclusion of no assistance data type fields in IE NR-PeriodicAssistData means that a periodic assistance data delivery session is terminated.

-- ASN1START

NR-PeriodicAssistData-r18 ::= SEQUENCE {

nr-PeriodicPRU-DL-Info-r18		NR-PeriodicControlParam-r18		OPTIONAL,	-- Cond dl-TDOA

...,

nr-PeriodicIntegrityServiceAlert-r18	NR-PeriodicControlParam-r18	OPTIONAL	-- Need ON

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| dl-TDOA | The field is optionally present if the field is included under NR-DL-TDOA-ProvideAssistanceData, need ON. Otherwise, it is absent. |

=====================================NEXT CHANGE===================================

–	NR-PeriodicAssistDataReq

The IE NR-PeriodicAssistDataReq is used by the target device to request periodic assistance data delivery from a location server.

-- ASN1START

NR-PeriodicAssistDataReq-r18 ::= SEQUENCE {

nr-PeriodicPRU-DL-InfoReq-r18	NR-PeriodicControlParam-r18	OPTIONAL, -- Cond pPRU

...,

nr-PeriodicIntegrityServiceAlertReq-r18	NR-PeriodicControlParam-r18	OPTIONAL -- Cond Integrity

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| pPRU | The field is mandatory present if the target device requests periodic nr-PRU-DL-Info; otherwise it is not present. |
| Integrity | The field is mandaroty present if the target device requests periodic NR-IntegrityServiceAlert; Otherwise, it is not present. |

====================================NEXT CHANGE====================================

6.5.10.1	NR DL-TDOA Assistance Data

–	NR-DL-TDOA-ProvideAssistanceData

The IE NR-DL-TDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-TDOA. It may also be used to provide NR DL-TDOA positioning specific error reason.

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...,

[[

nr-On-Demand-DL-PRS-Configurations-r17

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17																				OPTIONAL,	-- Need ON

assistanceDataValidityArea-r17	AreaID-CellList-r17					OPTIONAL	-- Need ON

]],

[[

nr-PeriodicAssistData-r18		NR-PeriodicAssistData-r18		OPTIONAL	-- Cond CtrTrans

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need ON, for UE based NR DL-TDOA; otherwise it is not present. |
| CtrTrans | The field is mandatory present in the control transaction of a periodic assistance data delivery session as described in clauses 5.2.1a and 5.2.2a, for UE based NR DL-TDOA. Otherwise it is not present. |

NR-DL-TDOA-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-TDOA-ProvideAssistanceData message.

nr-PositionCalculationAssistance This field provides position calculation assistance data for UE-based mode.

nr-DL-TDOA-Error This field provides DL-TDOA error reasons.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE 1: Void NOTE 2: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-DL-TDOA-ProvideAssistanceData message.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-TDOA-ProvideAssistanceData is valid.

nr-PeriodicAssistData This field specifies the control parameters for a periodic assistance data delivery session (e.g., interval and duration)-.

==============================NEXT CHANGE==========================================

6.5.10.2	NR DL-TDOA Assistance Data Request

–	NR-DL-TDOA-RequestAssistanceData

The IE NR-DL-TDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-TDOA-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16							OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs	(0),

posCalc (1) } (SIZE (1..8)),

...,

[[

nr-PosCalcAssistanceRequest-r17	BIT STRING {	trpLoc		(0),

beamInfo	(1),

rtdInfo		(2),

losNlosInfo	(3),

trpTEG-Info	(4),

nr-IntegrityBounds-r18 (5),

pruInfo-r18	(6)

}	(SIZE (1..8))				OPTIONAL,

nr-on-demand-DL-PRS-Request-r17	NR-On-Demand-DL-PRS-Request-r17				OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17

ENUMERATED { eAoD, eAoA }					OPTIONAL,

pre-configured-AssistanceDataRequest-r17

ENUMERATED { true }							OPTIONAL

]],

[[

nr-PeriodicAssistDataReq-r18			NR-PeriodicAssistDataReq-r18	OPTIONAL, -- Cond PerADReq

nr-IntegrityAssistanceRequest-r18	BIT STRING {

serviceParameters		(0),

serviceAlert			(1),

riskParameters			(2),

integrityParaTRP-Loc	(3),

integrityParaBeamInfo	(4),

integrityParaRTD-Info	(5)

} (SIZE (1..8))				OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerADReq | This field is mandatory present if the target device requests periodic NR assistance data delivery. This field may only be included if 'pruInfo' bit in nr-PosCalcAssistanceRequest is set to '1'. |

NR-DL-TDOA-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

nr-PosCalcAssistanceRequest This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 5 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are requested or not; bit 5 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is requested or not; bit 5 together with the bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is requested or not; - bit 6 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is requested or not. This field may only be present if the 'posCalc' bit in nr-AdType is set to value '1'.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for DL-TDOA. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

nr-DL-PRS-ExpectedAoD-or-AoA-Request This field, if present, indicates that the IE NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData is requested. Enumerated value 'eAoD' indicates that expected AoD information is requested; value 'eAoA' indicates that expected AoA information is requested. This field may only be present if the 'dl-prs' bit in nr-AdType is set to value '1'.

pre-configured-AssistanceDataRequest This field, if present, indicates that the target device requests pre-configured assistance data with area validity.

nr-PeriodicAssistDataReq This field indicates the control parameters for a periodic assistance data delivery session (e.g., interval and duration)-.

nr-IntegrityAssistanceRequest This field indicates the Integrity Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is requested or not.

====================================NEXT CHANGE====================================

6.5.11.1	NR DL-AoD Assistance Data

–	NR-DL-AoD-ProvideAssistanceData

The IE NR-DL-AoD-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-AoD. It may also be used to provide NR DL-AoD positioning specific error reason.

-- ASN1START

NR-DL-AoD-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Cond UEB

nr-DL-AoD-Error-r16					NR-DL-AoD-Error-r16					OPTIONAL,	-- Need ON

...,

[[

nr-DL-PRS-BeamInfo-r17				NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Cond UEA

nr-On-Demand-DL-PRS-Configurations-r17

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

OPTIONAL,	-- Need ON

assistanceDataValidityArea-r17		AreaID-CellList-r17					OPTIONAL	-- Need ON

]],

nr-PeriodicAssistData-r18			NR-PeriodicAssistData-r18			OPTIONAL	-- Cond CtrTrans

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need ON, for UE based NR DL-AoD; otherwise it is not present. |
| UEA | The field is optionally present, need ON, for UE-assisted NR DL-AoD; otherwise it is not present. |
| CtrTrans | The field is mandatory present in the control transaction of a periodic assistance data delivery session as described in clauses 5.2.1a and 5.2.2a; Otherwise, it is not present. |

NR-DL-AoD-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-TDOA-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-AoD-ProvideAssistanceData message.

nr-PositionCalculationAssistance This field provides position calculation assistance data for UE-based mode.

nr-DL-AoD-Error This field provides DL-AoD error reasons.

nr-DL-PRS-BeamInfo This field provides spatial direction information of the DL-PRS Resources included in nr-DL-PRS-AssistanceData or indicated by nr-SelectedDL-PRS-IndexList.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE 1: Void. NOTE 2: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-TDOA-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-DL-AoD-ProvideAssistanceData message.

assistanceDataValidityArea This field specifies the network area for which this NR-DL-AoD-ProvideAssistanceData is valid.

nr-PeriodicAssistData- This field specifies the control parameters for a periodic assistance data delivery session (e.g., interval and duration).

=================================NEXT CHANGE=======================================

6.5.11.2	NR DL-AoD Assistance Data Request

–	NR-DL-AoD-RequestAssistanceData

The IE NR-DL-AoD-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-AoD-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs	(0),

posCalc (1) } (SIZE (1..8)),

...,

[[

nr-PosCalcAssistanceRequest-r17				BIT STRING {	trpLoc		(0),

beamInfo	(1),

rtdInfo		(2),

beamAntInfo	(3),

losNlosInfo	(4),

nr-IntegrityBounds-r18 (5)

}	(SIZE (1..8))	OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Request-r17	ENUMERATED { eAoD, eAoA }		OPTIONAL,

nr-DL-PRS-BeamInfoRequest-r17				ENUMERATED { requested }		OPTIONAL,

nr-on-demand-DL-PRS-Request-r17				NR-On-Demand-DL-PRS-Request-r17	OPTIONAL,

pre-configured-AssistanceDataRequest-r17	ENUMERATED { true }				OPTIONAL,

nr-IntegrityAssistanceRequest-r18			BIT STRING {

serviceParameters		(0),

serviceAlert			(1),

riskParameters			(2),

integrityParaTRP-Loc	(3),

integrityParaBeamInfo	(4),

integrityParaRTD-Info	(5),

integrityBeamAntInfo	(6)

} (SIZE (1..8))			OPTIONAL

]],

nr-PeriodicAssistDataReq-r18			NR-PeriodicAssistDataReq-r18	OPTIONAL -- Cond PerADReq

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerADReq | This field is mandatory present if the target device requests periodic NR assistance data delivery. |

NR-DL-AoD-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

nr-PosCalcAssistanceRequest This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not; - bit 5 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are requested or not; bit 5 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is requested or not; bit 5 together with bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is requested or not; bit 5 together with bit 3 indicates whether the field nr-integrityBeamPowerBounds in IE NR-TRP-BeamAntennaInfo is requested or not. This field may only be present if the 'posCalc' bit in nr-AdType is set to value '1'.

nr-DL-PRS-ExpectedAoD-or-AoA-Request This field, if present, indicates that the IE NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData is requested. Enumerated value 'eAoD' indicates that expected AoD information is requested; value 'eAoA' indicates that expected AoA information is requested. This field may only be present if the 'dl-prs' bit in nr-AdType is set to value '1'.

nr-DL-PRS-BeamInfoRequest This field, if present, indicates that the IE NR-DL-PRS-BeamInfo is requested.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for DL-AoD. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

pre-configured-AssistanceDataRequest This field, if present, indicates that the target device requests pre-configured assistance data with area validity.

nr-IntegrityAssistanceRequest This field indicates the Integrity Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 6 indicates whether the field nr-IntegrityParametersTRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is requested or not.

nr-PeriodicAssistDataReq This field indicates the control parameters for a periodic assistance data delivery session (e.g., interval and duration).

================================NEXT CHANGE========================================

6.5.11.6	NR DL-AoD Capability Information

–	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3),

nr-IntegrityBoundsSup-r18	(4)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { sameSet, differentSet, sameOrDifferentSet }

OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17

OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

multiLocationEstimateInSameMeasReport-r17	ENUMERATED { supported }				OPTIONAL

]],

[[

locationCoordinateTypes-r18				LocationCoordinateTypes						OPTIONAL,

nr-IntegrityAssistanceSupport-r18		BIT STRING {

serviceParametersSup		(0),

serviceAlertSup				(1),

riskParametersSup			(2),

integrityParaTRP-LocSup		(3),

integrityParaBeamInfoSup	(4),

integrityParaRTD-InfoSup	(5),

integrityBeamAntInfoSup		(6)

} (SIZE (1..8))					OPTIONAL,

periodicReportingIntervalMsSupport-r18	PeriodicReportingIntervalMsSupportPerMode-r18

OPTIONAL

]],

periodicAssistanceData-r18				BIT STRING { solicited (0),

unsolicited (1)} (SIZE (1..8))	OPTIONAL

}

-- ASN1STOP

| NR-DL-AoD-ProvideCapabilities field descriptions |
| --- |
| nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device. |
| periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation. |
| ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation. |
| nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the target service supports the range of integrity risk (IR) for which the integrity assistance data are valid; - bit 5 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are supported or not; bit 5 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is supported or not; bit 5 together with bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is supported or not; bit 5 together with bit 3 indicates whether the field nr-integrityBeamPowerBounds in IE NR-TRP-BeamAntennaInfo is supported or not. |
| nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2,dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2 . Otherwise, the UE does not include this field. |
| nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData. |
| dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info. Enumerated value indicates the supported resource set relationship for the target DL-PRS Resource and the associated subset. |  |
| nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData. |
| nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests. |
| nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. |
| scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests. |
| nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports. |
| multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report. |
| mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |
| posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |
| multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report. |
| locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for UE-based DL-AoD. TRUE indicates that a location coordinate type is supported and FALSE that it is not. |
| nr-IntegrityAssistanceSupport This field indicates the Integrity Assistance Data supported. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 6 indicates whether the field nr-IntegrityParametersTRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not. |
| periodicReportingIntervalMsSupport This field, if present, specified the supported minimum millisecond periodic reporting interval for location information per positioning mode – i.e., supports the subfield reportingIntervalMs of the IE PeriodicalReportingCriteriaExt in IE CommonIEsRequestLocationInformation – and the minimum millisecond report interval supported for the periodic reporting. |
| periodicAssistanceData This field identifies the periodic NR assistance data delivery procedures supported by the target device. This is represented by a bit string, with a "one" value at the bit position means the periodic NR assistance data delivery procedure is supported; a "zero" value means not supported. Bit 0 (solicited) represents the procedure according to clause 5.2.1a; bit (1) (unsolicited) represents the procedure according to clause 5.2.2a. |

====================================CHANGE ENDS==================================