3GPP TSG-RAN WG2 Meeting #130	R2-2504105

# St. Julian's, Malta, 19 - 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5360 | rev | - | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to delivery of SIB segments by dedicated signalling in RRC_CONNECTED |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R16, we have introduced on-demand SIB request in RRC_CONNECTED by dedicated and SIB delivery by RRC message RRCReconfiguration. The lists of SIB and posSIBs that are supported in the RRCReconfiguration message are as follows: For different releases, segmentation of SIB/posSIB is supported. For SIB, segmentation is supported in the RRC layer for SIB12, SIB17, SIB17bis, SIB23; for posSIB, segmentation is supported in the LPP layer for all the posSIBs. With the above two features, we need to consider how SIB/posSIB segments can be sent by dedicated RRC signalling in RRC_CONNECTED. First, for SIB, if the network wants to deliver the requested SIB in RRCReconfiguration, they can be sent without segmentation since the maximum PDCP size can be as large as 9000 byte Next, for posSIBs, similar to normal SIBs, multiple segments of the same posSIB can be delivered by different items of SIBs within the SI message. Then, within the LPP spec, the meta-data is defined, according to which the posSIB can be re-assembled. The reason why it cannot be done as normal SIB is because the posSIB are not generated by the gNB but by the LMF and the content of the posSIB is transparent to the gNB. Then problem with the current spec is that there is the restriction that two segments cannot be inlcuded in the same SI message. This is reasonable for SI message delivery by broadcast since it does not make sent to segment SIB if they are inlcuded in the same SI message. But this does not make sense if we want to support delivery of different posSIB segments by dedicated signaling. The reason is that the LMF is responsible for the segmentation of the posSIBs and the gNB just transparently forwards the posSIB (segments). One might argue that the posSIB can be sent by multiple instances of RRCReconfig message periodically like the current posSIB/SIB by broadcast. This might work for cases when the UE is in good coverage and in low mobility scenairo. But for high mobility scenairo where the posSIB is time-sensitve and bad coverage scenairo when the TB size is small, this is not possible. Also, delivery of multiple RRC reconfig message is quite in-efficient. Based on the discussion above, we think that it is necessary to remove the restriction in the RRC spec for dedicated signaling |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the restriction that different segments of the posSIBs in the same SI message if the SI message is delivered by the dedicated signaling. Impact analysis Impacted functionality: SI request in RRC_CONNECTED Impacted 5G architecture options: NR SA Inter-operability: If the network is implemented according to the CR and the UE is not, or vice versa, there is no inter-operability issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to send the posSIB segments by dedicated signaling when the UE is in high moblity or in bad coverage. Also delivery of differnet posSIB segments by diff RRCReconfig message is quite in-efficient. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 5.2	System information

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs and posSIBs where:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms (TS 38.212 [17], clause 7.1) and it includes parameters that are needed to acquire SIB1 from the cell. The first transmission of the MIB is scheduled in subframes as defined in TS 38.213 [13], clause 4.1 and repetitions are scheduled according to the period of SSB;

NOTE 1:	If the period of SSB is larger than 80 ms, the MIB is transmitted with the same periodicity as that of SSB.

-	the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 and posSIBs are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs or posSIBs having the same periodicity can be mapped to the same SI message. SIBs and posSIBs are mapped to different SI messages, i.e. an SI message contains either only SIBs or only posSIBs. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Each SI message is associated with an SI-window and the SI-windows of different SI messages do not overlap. That is, within one SI-window only the corresponding SI message is transmitted. An SI message may be repeated with the same content a number of times within the SI-window. Any SIB or posSIB except SIB1 can be configured to be cell specific or area specific, using an indication in SIB1. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area, which consists of one or several cells and is identified by systemInformationAreaID;

-	The mapping of SIBs to SI messages is configured in schedulingInfoList and schedulingInfoList2, while the mapping of posSIBs to SI messages is configured in posSchedulingInfoList and schedulingInfoList2.
Each SIB and each posSIB is mapped to a single SI message. posSIBs of the same posSibType carrying GNSS Generic Assistance Data for different GNSS/SBAS (identified by gnss-id/sbas-id, see TS 37.355 [49]) are mapped to different SI messages.
Each SIB is contained at most once in an SI message. Each poSIB is contained at most once in an SI message when delivered by broadcast signalling.
For SIBs and posSIBs with segments when delivered by broadcast signalling, the segments contained in SI messages are transmitted according to the SI message periodicity, with one segment of a particular sibType/posSibType in each SI message;

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information, paging, or upon request from the UE.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, the network releases and adds the concerned SCell. For PSCell, the required SI can only be changed with Reconfiguration with Sync.

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

END OF CHANGES